BRKBOX    by     Tim Burmeister

V1.1 fixed snow when using IBM color card

detects monochrome monitor automatically and changes display to all white

removed time out error indicator - replaced with transmit indicator

can use on second serial port

SYNTAX

BRKBOX       -for com1 will display graphic screen if you have a color monitor

BRKBOX=      -for com1 no graphic screen (good for batch files)

BRKBOX 2     -for com2 will display graphic screen if you have a color monitor

BRKBOX=2     -for com2 no graphic screen (good for batch files)

(if you have a monodisplay the graphics intro is not displayed)

ERROR BEEPS

STEADY TONE  error in the parameters you gave it

PULSED TONE  program is already installed

NOTES

Uses interupt 1C

uses less than 1k memory

writes screen only during vertical retrace

If the com port has not been initialized you wont see any display

NO DISPLAY ????

Try putting your modem in ALB or if you dont have com2 port type brkbox 2

and you will see the display (although it wont mean anything)

This program will display in the upper right hand side of display

the active (SPACE, CONTROL ON, POSITIVE VOLTAGE) RS232 signals. The DTE

generated signals are displayed in red, the DCE in green, also uart errors

are shown in white. ***** NOTE if you have a mono display everything is white

DISPLAY DEFINTIONS

TXD	TRANSMIT DATA

FRM    FRAMING ERROR

PAR	PARITY ERROR

OVR	OVERRUN ERROR

CD	Recieve Line Signal Detected --CARRIER DETECTED

RNG	RING INDICATOR

DSR	DATASET READY

CTS	CLEAR TO SEND

DTR	DATA TERMINAL READY

RTS	REQUEST TO SEND

This program installs itself in memory and the display can be toggled off and

on by depressing the ALT AND LEFT HAND SHIFT KEYS simultaeneously.

The display is turned on initialy (if you dont see anything put your modem

into analog or local loopback). Since this routine is active only about 18

times per second it will not catch all error conditions that may occur

during a normal communications session.

IT HAS BEEN KNOWN TO CAUSE INTERFERENCE WITH COM PROGRAMS AT 9600 BAUD

**********************************************************************

If the display is active while accessing a HALF-HEIGHT floppy drive

it causes data errors. If so toggle the display off while accessing

the floppy.If anyone has any info about this I would appreciate hearing from you.

I would like to hear your comments and suggesstions

Tim Burmeister

#s)

(if y