___________________________________________________

T H E

M O D E M

D O C T O R

Shareware Version 2.1S

Hank Volpe (c) 1989

108 Broadmoor Dr

Tonawanda NY 14150

___________________________________________________

Section 1. Introduction -

-------------------------

The Modem Doctor is a professional diagnostic tool designed to work with IBM-

PC's and compatibles. There are two versions of the Modem Doctor, the

Shareware Version and the Registered Version.

The Registered Version of the Modem Doctor contains complete diagnostics for

8250 and compatible UARTS, RS-232 data lines, and Carrier tests. Some of the

many diagnostic features of the Registered Version are ;

**  Support for COM 1 through COM 4

**  Baud Rates from 300 to 19200 bps

**  Support for Courier HST/Dual Standard/V.32 Modems  [+]

**  Auto log-in of COMM Ports to DOS for use by other programs

**  Manual log-in of COMM ports for Non-Standard Hardware

**  On-screen Display of all 8250 and Compatible UART registers

**  On-screen Display of Modem S-Registers

**  Full Diagnostic tests of UART and RS-232 Channels including;

-- ALL UART REGISTERS

-- ALL RS-232 Handshaking Signals

-- 8259 Interrupt Controller Mask Registers

-- Digital Loopback UART Tests

-- Analog Loopback Data Tests

**  Full Diagnostic Checks of "AT" Instruction Set Modems

-- Carrier Tests on both Originate and Answer Frequencies

-- Carrier Test interface for "Dumb" Modems or Data Channels

-- Modem Microprocessor Command Channel Diagnostics

-- Support for High Speed Modems

**  Interactive Mode with 4K Communications Buffer for;

-- TTY mode Interactive Communications over Serial

Port or Modem while on-line

-- Custom Designed Tests...Help Menu Included

-- On-Line "Hot-key" Viewing of 8250 UART Registers

The Shareware Version of the Modem Doctor is designed for people who would

like to try a piece of software before buying it. This "Look-see" version has

some limits over the Registered Version, however it is does provide the

following full-functioned Diagnostics;

**  Support for COM 1 and COM 2

**  Baud Rates from 300 to 2400 bps

**  Auto log-in of COMM Ports 1 and 2

**  On-screen Display of all 8250 and Compatible UART registers

**  On-screen Display of Modem S-Registers

**  Diagnostic tests of UART and RS-232 Channels including;

-- ALL UART REGISTERS

-- ALL RS-232 Handshaking Signals

-- 8259 Interrupt Controller Mask Registers

**  Full Diagnostic Checks of "AT" Instruction Set Modems

-- Carrier Tests on both Originate and Answer Frequencies

-- Carrier Test interface for "Dumb" Modems or Data Channels

-- Modem Microprocessor Command Channel Diagnostics

**  Interactive Mode with 4K Communications Buffer for;

-- TTY mode Interactive Communications over Serial

Port or Modem while on-line

-- Custom Designed Tests...Help Menu Included

-- On-Line "Hot-key" Viewing of 8250 UART Registers

The Modem Doctor is copyrighted by Hank Volpe. The Shareware Version of this

program may be distributed on Bulletin Board Systems or in Public Domain

Libraries as long as a Fee is not charged for its duplication or acquisition.

Under no conditions can anyone accept payment for the Modem Doctor but Hank

Volpe or agents authorized for resale by Hank Volpe.

The Modem Doctor is distributed on as "As-is" basis, with no warranties for

usefulness or suitability expressed or implied. Users of the Shareware

version are granted a limited license to use this program. It is expected

that if you use it for more than 30 days, that you will register the Modem

Doctor.

Registration adds many powerful features that make the Modem Doctor an

excellent RS-232/UART/Modem Diagnostic Tool. Comparable programs cost well

over $100, while the Modem Doctor Registration Fee is just $19.95 for a

single copy.

Group Discount prices for software distributors and Businesses are available.

For Single-user registration, send a check or Money order for $19.95 to

Hank Volpe

108 Broadmoor Drive

Tonawanda NY 14150

You will receive the latest version of the Modem Doctor, plus several other

useful Modem Utilities. Registered Users also are entitled for updates to

newer versions (when available) for a small fee to cover shipping and

handling.

Section 2. Starting MDR

------------------------

The Modem Doctor has been designed to work with PC-Type Serial Ports and "AT"

Instruction Set (so called Hayes Compatible[++]) Modems. The Modem Doctor

requires at least 1 Serial Port to Operate.

MDR is written to run under MS or PC DOS 2.2 or higher, requires a

MDA or CGA adaptor and at least 80K of program memory. When MDR is

started, it will use the last mode that the display adaptor was in

(unless it was in EGA or VGA mode or a graphics mode). MDA and CGA are

the two execution modes regardless of what kind of adaptor is in use.

Code has been included to work with hardware-incompatible display

adaptors as long as they are at least IBM-PC Bios compatible. This is

operator selectable at runtime.

To start MDR, type MDR from the dos prompt. If you wish to use the

Bios compatible mode instead of direct display writes, start the

program using the Bios switch ;   MDR /B

Section 3. First Time Setup

------------------------------

MDR checks to see what hardware is available and reports its results

to you. Displayed on the bottom line is the serial number unique to

each copy (Registered Version..Shareware Version says "Shareware").

Press any key to continue.

�[  Ports  ]�ͻ

� Use COMM 1  �

� Use COMM 2  �

� Use COMM 3  �  * Registered Version Only

� Use COMM 4  �  * Registered Version Only

�             �

�������������ͼ

Next, you are instructed to pick from a list of valid comm ports. The

only ports that are listed are valid ports that MDR found when it

first started. The Registered Version allows you to add ports that are not

"standard" ports, (check the section on Login). Use the arrow keys to find

your selection, and then press return to select.

�[  Baud  ]ͻ

�   300 BPS �

�  1200 BPS �

�  2400 BPS �

�  4800 BPS �   * Registered Version Only

�  9600 BPS �   * Registered Version Only

� 19200 BPS �   * Registered Version Only

�����������ͼ

A window now opens showing you valid comm port speeds. Select a speed

from this menu. Please note, don't select a speed faster than your

modem can handle. The reason for faster speeds is so communication

links can also be tested. Use the arrow keys and press return to make

your selection.

�[  Select an Option  ]�������������͸

� Test Modem / Port 1 to 300 BPS     �

� Test the UART Hardware Only        �

� Dont set up modem, return to Menu  �

��������������������������������������

At this point you are in the options menu. If you select Test Modem, a

series of commands and tests will be performed to make sure the modem

is on line and running. If you select Test UART Hardware, only the

8250 UART will be tested. The modem will not be setup. This selection

is useful only if you wish to test an RS-232 line or a "dumb" modem.

Selection 3 aborts to the main menu

A. Test Modem/ UART

---------------------

Preliminary Diagnostic Checks of Hardware

-----------------------------------------

Testing Modem Microprocessor

----------------------------

Modem Microprocessor Command Bus Active

Modem reports a 9600 bps Modem installed

Modem Microprocessor Accepts Setup Commands

Modem Microprocessor ON-LINE

Modem - UART Status appears correct

Press any key to continue

Above is displayed then normal test results with a modem.  First step

tests the UART to Modem hardware. If any connection errors are

discovered, they are reported to the screen. Next the modem command

bus is tested and if its compatible, the modem ID information is

queried from the Modem Processor. Finally, the modem is setup for

diagnostics. Note: These setup strings do not overwrite any non-

volatile storage. If all is ok, all the UART registers and the First

16 Modem S registers are displayed. This display is not shown the next

time you log in a comm port unless you press the space bar

If the modem does not respond to certain parts of the tests, error

messages are reported.

Testing Modem Microprocessor

----------------------------

Modem Microprocessor Command Mode Not Detected

Software Reset Initiated

This message is displayed if the modem did not respond the first time

for a query to its status. This could happen for any number of

reasons, including that the modem was still on-line and not in the

command mode. A soft-reset is performed, which essentially tries to

wake the processor up.

No Response...resetting Modem Microprocessor ...attempt #1

No Response...resetting Modem Microprocessor ...attempt #2

No Response...resetting Modem Microprocessor ...attempt #3

After 3 soft-resets, the Modem Doctor assumes that the modem is jammed

and needs a hardware reset. A hardware reset is the program equivalent

of turning the modem on and off again. This is the only way to reset

some internal modems that get "jammed" when the computer is first

turned on. Clearing a "jam" usually takes no more than 2 attempts.

Trying a Hardware reset of the Modem Microprocessor

The Hardware reset is performed, and again the modem microprocessor is

strobed for its status.

Software Reset Initiated

After a hardware reset, a soft reset is performed again, attempting to

wake the processor up. If this fails, the following message is

displayed;

Modem Microprocessor Did Not Accept Setup Commands

Trying a Hardware reset of the Modem Microprocessor

After one more attempt at waking the modem up, the Modem Doctor

assumes that the modem is not compatible  and based on the status

given it by the UART, it will report if the device is off or not

connected.

WARNING...This unit does not have a Compatible Modem attached

WARNING...The Modem Device appears to be off or not connected

The registers are now displayed for diagnostic tests, and a conclusion

based on the status of the registers is displayed. There are many

reasons why the modem/UART could fail. See the error code section for

details.

If the UART is fine, but the modem is not compatible, the program will

tell you that the UART hardware tests ok and display this in a window

to the right of the register display.

B. UART Tests

---------------

The UART test is performed every time either the Modem or the UART test

is selected. Essentially, the UART test programs all of the registers

of the UART and looks to see that the correct values have been placed

in them. After that, it looks to see that an RS-232 loop is active. If

not, it displays an error message saying that the loop is either

disconnected or a register in the UART is defective.

Each register is checked for proper values...if anyone of them fail, a

window will open and display a possible cause for the problem.

Preliminary Diagnostic Checks of Hardware

-----------------------------------------

Testing UART Hardware Only

--------------------------

Modem - UART Status appears correct

The first time through these tests, the registers will display for

diagnostics. Unlike the modem tests, only the UART registers are

displayed.

C. Return to main menu

----------------------

Selecting this returns you to the main menu, this allows you to change

an entry made in error without having to wait for the diagnostic

checks to fail.

In any event, after any of these tests have been run, you will then

enter the Modem Doctor's Main Menu.

Section 4. The Main Menu

----------------------------

�������������������������� The Modem Doctor ���������������������������Ŀ

�Log-in   Regs      Carrier   Loopback  Manual    Setup     End Program �

�������������������������������������������������������������������������

All of the above selections are toggled using the arrow keys and

selected using the return key. Pressing the down arrow displays a

brief message about what each function does.

Log-in COMM Ports

------------------

The ROM-BIOS only looks for COMM ports 1 and 2. Use this

section to manually log-in a COMM port that cannot be

detected by DOS. Entries are also good after you quit

(NOTE..This feature in Registered Version Only!)

UART Register Diagnostics

-------------------------

This test section can be used to check the contents

of an 8250 UART and the 8259 Interrupt Controller mask

register. White = Active, Gray = Not Active

Carrier Mode Tests

------------------

This tests runs a Smartmodem in an test of the Carrier

Originate and Answer frequencies. A Smartmodem or a modem

with Manual Carrier selection is required

Loopback/ UART Tests

------------------

This test is designed to check out a RS-232 COMM port

for proper operation. No Modem is required for this test

however this test is not for an Internal Modem, although

it can be run as a test of the command channel.

(NOTE..This feature in Registered Version Only!)

Manual Interactive Tests

-------------------------

This test is designed to check out the command channels

and user interface to a Smartmodem. You can also use it

with an RS-232 loop or to send your own commands.

Setup  Diagnostics

-------------------------

Use this test section to return to the setup mode to

change a Port assignment or a Baud Rate to a new value.

Only Logged-in Comm ports can be accessed.

Exit Program

-------------

Quit the program, all vectors and interrupt tables

are restored.

4.1 Login In Comm ports

-----------------------

(NOTE..This feature in Registered Version Only!)

When you select this menu, you are offered these choices

�[  Comm Port Login  ]����������ͻ

� Let me Auto Detect COMM Ports  �

� You Manually Enter COMM Ports  �

�      Return To Main Menu       �

��������������������������������ͼ

Option 1 repeats what the Modem Doctor does when you first begin to

run the program.

Option 2 allows you to enter a "non" standard Port address and assign

one of the two Interrupt lines to use with the UART.

Option 3 returns to the main menu.

Port addresses for PC's are standardized in order for Communication

programs to properly work. The industry standards are

COMM 1  3F8        COMM 2   2F8        COMM 3  3E8      COMM 4  2E8

IRQ4                IRQ3               IRQ4             IRQ3

The Port addresses for Serial Ports is held in a reserved area of PC

Memory. The "base port" of each of these UARTS is saved in a distinct

area. When you first start the Modem Doctor, the program checks these

locations to see if there are any Comm ports already logged in by Dos.

If not, a search is conducted for the PC's default Comm 1 and Comm 2

port values and these are logged in. Next, a search is performed for

the other industry standard comm ports. Some PC's already log these

in, however IBM-PC's and those that mimic exactly their ROM-BIOS

routines do not log in a COMM 3 or a COMM 4. The Modem doctor will,

and as a side benefit, these stay logged in until the next warm or

cold boot is performed on your computer. Why would dos not log in a

port? Well, some multi-tasking programs overwrite this area to prevent

"swapping" of communication programs and it is possible that they

might not be properly restored on exit.

Option 2 is the only real option available. Please use it with

caution. The only real need for this option is for logging in a COMM

port that does not use any of the listed industry standards as its

base port address.

Consult your Serial Port or Modem's user manual to make sure you enter

the exact Port address and select the correct Interrupt line to use

with it. Otherwise the Modem Doctor will not recognize or be able to

test any device hooked to this port.

Protection is included in this routine to keep someone from

accidentally entering a port that is used by the system board, display

adaptors, or fixed disk adaptor. Entering any of these values, or an

error, will result in the display;

"WARNING...RESERVED PORT...RETRY ENTRY"

After running Option 1 or 2 , you must tell the Modem Doctor which

COMM port you wish to use so that testing can begin

4.2 Regs

-------------

The register display section can be an invaluable diagnostic tool.

What this does is "poll" all of the status registers of the UART, and

the Interrupt Controller to tell you exactly what is going on.  When

you use the setup mode, the Modem Doctor checks all of these registers

for correct information. If something wrong is discovered, a warning

is displayed on the screen telling you what the Modem Doctor believes

is wrong. Many different error conditions are checked, see the error

code section for a listing. Each register contains a Mnemonic for each

one of the data bits. Bits that are a logic low ( = 0) are displayed

in low intensity, those that are a logic high (= 1) are displayed in

high intensity. On a color monitor, this is easy to see. On a Mono

Monitor, make sure you have the brightness and contrast controls

properly adjusted to see the difference between the two status

conditions.

�[ Line Control Register ]���������͸

�  DLB STB STP EPS PEN STB WS1 WS0  �

�������������������������������������

This register controls many of the setup characteristics of your UART

and how it behaves to your PC. This register is responsible for

setting baud rate, setting break code transmission, parity, and the

length of the serial stream word. When you enter the setup mode,

DLB    Divisor Latch          Used to access the baud rate registers

Should be 0 for the Modem Doctor

STB    Set Break              If High, the modem transmits a break

signal, should be 0 normally.

STP    Stick Parity           Should be 0 normally

EPS    Even Parity select     High = Even Parity..should be 0 normally

PEN    Parity Enable          Enable Parity checks, should be 0 normally

STB    Stop Bits              Number of stop Bits, should be 0 normally

to Indicate 1 stop bit character only.

WS1    Word Length Select 1

WS0    Word Length Select 0   These two bits are used to determine the

Serial data word length. Both should be

High normally to indicate a length of

8 bits.

�[ Line Status Register ]����������͸

�      TSE THE BI  FE  PE  OE  DR   �

�������������������������������������

The line status register reports error conditions that might have

occurred between the PC and the UART.

TSE    TX Shift Register      A High indicates that all characters

Empty                  sent to the UART were transmitted.

Should be High Normally.

THE    TX Holding Register    A High indicates that the UART can now

Empty                  accept a new character from the PC.

Should be High Normally.

BI     Break Interrupt        A High indicates that a break signal

was received. Should be Low Normally

FE     Framing Error          A High indicates that a serial data

train did not have a proper stop bit.

Should be Low Normally.

PE     Parity Error           A High indicates that the received

serial data's parity did not match

what the UART was programmed to receive.

This should be Low Normally.

OE     Overrun Error          A High indicates that a character in

the receive buffer was not read by the

PC in time, and another character

overwrote and destroyed it. Should be

Low Normally.

DR     Data Ready             A High indicates that a character has

been received and is ready to be read by

the PC from the Receiver Buffer. Should

be Low normally for these test, but can

vary while data is coming in from the

Modem.

�[ Modem Control Register ]��������͸

�              LP  OT2 OT1 RTS DTR  �

�������������������������������������

The Modem Control register handles all of the interfacing between the

UART and the modem. Only 5 signals are needed to handle this

LP     LOOP                   A High indicates that the Loopback

digital diagnostic has been activated.

This feature only works with 8250 UARTS

or those that are 100% Compatible. This

is used in the Digital Loopback test.

OT2    OUT 2                  A High enables the UART to generate

interrupts. A Low disables interrupts

This should always be high for testing

purposes.

OT1    OUT 1                  On 100% compatible modems, a High on

this pin will perform a Hardware reset

of the modem. Normally this is a Low.

RTS    Request to Send        A Handshaking signal that is used by

some modems to enable data transfer

between the Modem and the UART. Should

be High for testing purposes.

DTR    Data Terminal Ready    A Handshaking signal that is used by

some modems to enable data transfer

between the Modem and the UART. Should

be High for testing purposes.

�[ Modem Status Register ]���������͸

�  RLS RI  DSR CTS DRD TER DDR DCS  �

�������������������������������������

The Modem Status Register returns the status of the control signals

sent by the modem to the UART.

RLS    Received Line Detect   A High indicates that a carrier has

been received by the modem. When the

Analog tests are run, this should be

High. If it is not, it is still possible

that the Modem is ok, however the

cable from the Modem to the UART (for

externals only) might not have a complete

circuit due to faulty wiring of the

RS-232 cable. This signal is very

important for some communications

programs to operate properly.

RI     Ring Indicator         A High indicates that the modem detected

a Ringing signal on the phone line.

DSR    Data Set Ready         This is part of the DTR/DSR Handshaking

pair that is used by some modems to

control the flow of data. Should

normally be a High.

CTS    Clear to Send          This is part of the RTS/CTS Handshaking

pair that is used by some modems to

control the flow of data. Should

normally be a High.

DRD    Delta Received

Line Signal Detect     A High indicates that the Received Line

Signal has changed state. Some

Communications programs use this also as

an indication that a Carrier was

detected by the modem.

TER    Trailing Edge

Indicator              A High indicates that the Ring Indicator

changed state. Some Communications

programs use this as an indication that

the phone as Rung.

DDR    Delta Data Set Ready   Indicates that the DSR signal has changed

state.

DCS    Delta Clear to Send    Indicates that the CTS signal has

changed state.

�[ Interrupt ID Register ]���������͸

�                      ID2 ID1 ITP  �

�������������������������������������

The Interrupt ID Register is used to tell the CPU what kind of

interrupt occurred.

ID2    Interrupt Bit 2

ID1    Interrupt Bit 1        These two signals are a binary code

which can be read by the CPU to tell

what kind of interrupt occurred. Both

Bits high indicates an error occurred or

a Break Interrupt was received.

Bit 2 high and Bit 1 low indicates that

data is ready to be read by the CPU.

Bit 1 High and Bit 2 low indicates that

a Ring signal or Carrier signal was

received.

For testing purposes, only the Data

ready mode is used.

ITP    Interrupt Pending      A Low indicates that an interrupt has

occurred. For testing purposes, this

should always be high if data was

successfully read by the Modem Doctor.

If it is not, then an interrupt did

occur but the CPU did not respond.

Suspect a problem with an interrupt

Controller Chip or the UART interrupt

generating capability.

�[ Interrupt Enable Register ]�����͸

�                  ESI ELI ETI ERI  �

�������������������������������������

The Interrupt Enable register is programmed to allow certain types of

interrupts to occur.

ESI    Enable Status          A High causes the UART to generate an

Interrupt              interrupt whenever RI or RSD signals in

the Modem Status register go high.

Should be low for testing.

ELI    Enable Line            A High causes the UART to generate an

Interrupt              interrupt whenever an error or break

interrupt is detected in the Line

Status Register. Should be low for

testing.

ETI    Enable Transmitter     A High causes the UART to generate an

Holding Register       interrupt whenever the THE signal of the

Empty Interrupt        Line Status register goes High. Should

be low for testing.

ERI    Enable Receive Data    A High causes the UART to generate an

Ready Interrupt        interrupt whenever Data is ready to

be read by the CPU. The DR line of the

Line Status register going to a high

condition generates this condition. This

signal should be High for testing

purposes, since it is the only interrupt

needed by the Modem Doctor.

�[ Interrupt Mask Register ]�������͸

�  IQ7 IQ6 IQ5 IQ4 IQ3 IQ2 IQ1 IQ0  �

�������������������������������������

The Interrupt Mask Register is contained on the 8259 Priority

Interrupt Controller on your Computer's Motherboard. Each one of these

Hardware Interrupts corresponds to an external device.

IQ7    Printer Interrupt

IQ6    Floppy Disk Interrupt

IQ5    Hard Drive interrupt

IQ4    COM1 Interrupt (also COM3)

IQ3    COM2 Interrupt (also COM4)

IQ2    reserved by IBM (Gateway to second 8259 on AT's)

IQ1    Keyboard Interrupt

IQ0    Timer Interrupt

Under Normal Conditions, IQ0,IQ1 and IQ6 should be lit, along with

IQ4 if testing COM1 (or COM3)  or IQ3 if testing COM2 (or COM4). In

addition, other lines could be lit depending on your computer. The only

line the Modem Doctor looks for is the IQ4 or IQ3 lines (depending on

which Serial Port you are testing).

4.3 Carrier Tests

-----------------

The Carrier tests are designed to test your modem under actual use

conditions (minus a telephone line).  There are two versions of this

test, one of which is automatically selected by the Modem Doctor

depending on the hardware it detects.

Smart Modem Test - If you selected the Test Modem/UART option from the

setup menu and a Modem with a compatible Microprocessor was detected,

the Modem Doctor will test the Originate and Answer frequencies of

your modem automatically. Before running this test, make sure you have

disconnected the modem from the telephone line, because the modem will

go off-hook and sometimes the dial-tone will interfere with testing.

You will be prompted to enter a "Loop Count". If you select a count of

1, you will be able to hear the connecting tones on your speaker. If

you select a count greater than 1, then the speaker is turned off.

After you make your selection, two windows will open on your screen.

One contains data that is being sent to the modem, the other Data that

has been received by the modem. All of the characters from Binary 0 to

Binary 255 are transmitted. Some of these will look strange because

they are characters of the Extended Character set. The Modem Doctor

takes a great deal of care in preserving the normal condition of your

modem before testing, and sends all the character strings necessary to

test without disturbing any non-volatile memory settings. If you have

an external modem, you will see the data being sent and acknowledged

before testing begins. If you have an internal, please be patient

while it sets up.

UART or Dumb Modem - This test works the same as the Smart Modem tests

with two main exceptions. First, Your modem must be able to generate a

carrier on its own on the answer and originate frequencies. Most

external "dumb" modems have front panel switches for this function.

Most are marked as AL (for Analog Loopback). If your modem can

generate this, answer yes and follow the prompts on the screen. All

testing is performed as above, except you setup the modem instead of

the Modem Doctor.

Errors - If A RLS (Received Line Detect) signal is not received before

testing begins, this indicates that either A Carrier is not being

generated (or properly received in self test) by the modem, or an RS-

232 Cable to an external modem is improperly wired. Although RS-232

loops will work fine without an RLS signal being wired, there is no

way for the modem Doctor to detect on a Dumb Modem if a carrier was

generated. On a Smart Modem, the Modem Doctor alternately looks for

the word "CONNECT" to be sent from the modem if there is no RLS signal

available. If you wish your all of your communications programs to

work properly, you should get a cable with this signal wired.

If an Error is detected during the Analog test, the error Counter is

incremented. There are two error counters; One is used for the

Originate Mode, and the other for the Answer Mode. If you receive an

error, it could be due to a problem with a UART or a Modem at a

certain Baud Rate. Try testing at other Baud rates to determine if the

errors repeat. Also Errors could occur on only the Originate or only

the Answer frequency. This is more prevalent at higher baud rates.

Finally, make sure that the modem was disconnected from the Phone line

before testing. The Dial-tone interferes with some modems. Finally,

there could be a problem with a cable or interrupts not properly being

generated or properly handled by the CPU. Run the Digital/Loopback

tests and if the problem repeats, you might have a bad UART or

motherboard problem.

4.4 Loopback Tests

-------------------

(NOTE..This feature in Registered Version Only!)

The Loopback tests were designed for two purposes. First, to test an

RS-232 loop including cables that are properly terminated with a

loopback plug, and second, to run a digital loopback test on 8250 and

100% Compatible UARTS.

Selecting Loopback and pressing return fetches this menu

�[  Loopback Tests  ]����������ͻ

� Analog Loopback using a PLUG  �

� Digital Loopback 8250 UARTS   �

� Loopback Plug Technical Data  �

�     Return To Main Menu       �

�������������������������������ͼ

Analog Loopback tests require that you attach a Loopback plug either

to the back of the Comm port connector on your computer, or at the end

of a RS-232 Cable. This test is designed for external Modems, however

you could run this on an internal modem as a test of the command

channel's ability to receive characters. Caution should be used on

internal modems, for some will fail this test. If this happens, don't

be concerned. However this test should NEVER fail with a Loopback

Plug.

Digital Loopback tests require that you have an 8250 or 100%

compatible UART in your system. These UART's have built-in diagnostics.

If this test does not run, it means that you do not have a compatible

UART installed. Some UARTS that are not 8250's have a problem entering this

mode properly. If the test does not start, do not be alarmed, because only

8250 and true compatibles will pass this test. However this test will NEVER

start and then fail unless you do have a hardware problem. You should never

pick up a character error using this test. If you do, you have a bad UART or

CPU Motherboard Problem.

Both Tests open up windows (like the Analog test) and send and receive

characters to the UART.

If you wish to know how to build a loopback plug, select the Loopback

Plug technical data section.

4.5 Manual

----------

As well written as the Modem Doctor is, there exists the possibility

that a Modem or a system might not be compatible with the diagnostics.

The Manual section allows you to send your own string commands to the

Modem, read back data, and in fact you could use this as a Mini-

communications program.

If you select this mode, a window opens with the banner Interactive

Mode. At the Bottom are listed the "Hot-Keys" you can use;

ALT-C   Clears the screen. This only clears the window of data, and

leaves all your prompts.

ALT-R   This displays the UART registers...handy for real-time

troubleshooting of RS-232 loops.

ALT-H   Help..this displays the basic AT Command set instructions that

you could use to control your modem. Note that this is only a

basic set. Some modems have many extensions to this, so for

advanced work, consult your owners manual.

ALT-X   Exits the interactive mode and returns to the main menu.

4.5 Setup Mode

--------------

This section was covered above. Use this to change the serial port

under test, the baud rate, and the type of test to be run.

4.6 Exit Program

----------------

This is the only way to exit the Modem Doctor. This is necessary

because the interrupt tables and controllers of your PC must be

returned to their original state in order to avoid problems after you

use the Modem Doctor.  Pressing Control-C or Control-Break during the

Modem doctor will cause one of two things to happen. First, it will

freeze the program deliberately (locking it up) forcing you to

warmboot your computer. Second, if it is at all possible to exit

without damage, a window will open telling you that you can control-

break abort if you wish..answer yes or no.

5.0 Error Messages and their Meanings

-------------------------------------

The Modem Doctor, if presented with an error, will make a diagnosis of

the problem. In most cases, this should be enough for you to figure

out what is wrong and go on from there to correct it.

5.1 Log In errors

-----------------

ALL Available COMM ports are in use...Press to Return

-COM1 through COM4 are logged in and no new ports can be added

COMx Not Found

-A COMM port (x) was not located at the port address specified.

This is normal during auto-selection, but not if a manual selection

was entered. If a manually entered address did not work, check your

Modem owners manual to make sure you entered it correctly.

WARNING...RESERVED PORT...RETRY ENTRY

- You entered a port address that is assigned to a critical

Motherboard or CPU address port. Your entry was an error..check

your modem owners manual to get the right port.

5.2 Carrier Errors

------------------

WARNING...CARRIER FAILURE...Status Registers Displayed above

followed by;

RLS (Received Line Signal Detect indicates that a carrier has

not been received by the UART Check modem or connector wiring

This indicates that a "dumb" modem did not signal the UART that it has

received a carrier (or is generating one).

Modem not responding to the Analog originate test

or;

Modem not responding to the Analog Answer test

A Smart Modem did not go into the self test mode. The Modem Doctor

looks for the word "CONNECT" to be returned by the modem. A possible

reason for a perfectly working modem to generate this is if the wrong

baud rate was selected for testing. Smart Modems return

identifications that can be read by the Modem Doctor. If you suspect

you selected the incorrect baud rate, go to setup and select 300 baud

and try again. If it doesn't work this time, you might have a problem

with your modem generating a self test.

Modem Transmit Error

- The Modem Doctor attempted to send a character to the Modem, but

the status of the Transmit Holding Register indicated that the character

was never sent. This will abort a test. Check cable wiring and that

somehow the modem did not get disconnected from the Serial port or

inadvertantly turned off.

Modem Queue Failure

- A character was sent to the modem, but nothing, not even the wrong

character was sent back to the Computer from the Modem. This test is driven

by interrupts, however if they fail to happen, a brute-force method is

tried one time and if that still fails to get a character, then this

error message is displayed. Check cable wiring and again make sure that

somehow the modem did not get disconnected from the Serial port or

inadvertantly turned off.

CARRIER NOT DETECTED

- Some Modems in the self-test mode do not raise a carrier (RLS) or send the

word CONNECT. Instead, the MB (Modem Busy) light blinks on the Modem. One

case of this is the USR Courier Dual Standard Modem. In this case, the

self test continues, but the words CARRIER NOT DETECED are displayed in

the message window instead of CARRIER ON

Modem Analog Test passed without any errors in Originate and Answer

Modes

- Normal results of the testing.

"You had a total of x Originate Errors  and  x  Answer Errors

- An error was detected on character transmission by the Modem

Doctor. Use the window display to isolate the error if you wish,

or run the test again, making sure that the phone line is

disconnected.

5.3 Loopback Errors

-------------------

UART not responding using Analog Loopback Plug mode

- No characters were received over the RS-232 Loop. Make sure you

are using a proper RS-232 cable and a proper Loopback plug

No Loopback Plug or General Failure of the Hardware

- None of the handshaking signals (RTS/CTS DTR/DSR) were detected

(or generated) by the UART. Either you have a bad cable, a bad

loopback plug or a faulty UART.

Loopback Transmit Error

- The Modem Doctor attempted to send a character to the UART, but

the status of the Transmit Holding Register indicated that the character

was never sent. This will abort a test. If you are performing the

Analog Loopback test, check cable wiring and that  somehow the modem did

not get disconnected from the Serial port or  inadvertantly turned off.

If you are running a digital loopback test, then this indicates that

the UART is failing to send characters. If you have an 8250 or compatible

UART, then this would indicate a problem. If your UART is more or less

compatible, then this message might get displayed

Loopback Queue Failure

- A character was sent to the modem, but nothing, not even the wrong

character was sent back to the Computer from the Looping source. This test

is driven by interrupts, however if they fail to happen, a brute-force

method is tried one time and if that still fails to get a character, then

this  error message is displayed. If you are performing the Analog Loopback

test, check cable wiring and again make sure that  somehow the Loopback

source (Plug or Modem) did not get disconnected from the Serial port or

inadvertantly turned off.

If you are running a digital loopback test, then this indicates that

the UART is failing to loop characters. If you have an 8250 or compatible

UART, then this would indicate a problem. If your UART is more or less

compatible, then this message might get displayed

Analog Loopback Test passed without any errors

- Normal result of test

You had a total of x Errors

- At least one error was detected...this is the normal result if a

UART does not respond to any test.

UART not responding to the Digital Loopback test

- No characters were looped internally through the 8250 UART. Suspect

a non-compatible UART or a failure of the 8250 UART.

UART Registers did not set for Loop back test

- The UART Modem Control Registers did not program properly. This

should indicate a hardware failure of the UART under all

circumstances (including use with a non-compatible UART).

UART Digital Test passed without any errors

- Normal result of test

5.4 Setup Errors

----------------

Modem Microprocessor Did Not Accept Setup Commands

- Commands were sent to setup the modem for testing, the Modem did

not acknowledge. Suspect a non-compatible modem or a possible

hardware failure.

Modem Status Register readings show that there is no status coming

back from the modem to the UART.  If this is an external modem,

check" that the cable is plugged in.

- The only condition that the UART will not have any status

is if an External Modem is off, not connected, or the UART

has a serious hardware problem.

Line Control register readings show that the word length selected is

incorrect. WS1 and WS0 should both be on indicating that 8 bit words

are to be used. The UART is defective or communications to the UART

are suspect.

Line Status Readings Show

show that the Tx Holding Register THE or the Tx Shift Register TSE is

not resetting properly. UART is suspect or connections to the UART

from the computer.

Modem Control register readings show that either DTR,RTS or OT2

signals are not set properly. DTR and RTS are essential for

communications. OT2 is essential for interrupts. UART is suspect or

connections to the UART from the computer

Interrupt ID register readings show that an interrupt is pending when

none should be. UART is suspect or connections to the UART from the

computer.

Interrupt Enable register readings show that ERI Enable Receiver ints

is not set properly. UART is suspect or connections to the UART from

the computer.

8259 Interrupt Controller has not responded to setting the IRQ4 line.

This problem is on the system board of your computer. Run your

computer diagnostics program to locate the error

8259 Interrupt Controller has not responded to setting the IRQ3 line.

This problem is on the system board of your computer. Run your

computer diagnostics program to locate the error

I cant find a COMM Port

- If Dos does not Log in Comm 1 or Comm 2 and if no port is

Auto-Detected, then the Modem Doctor Aborts for there is no

reason to run the program.

WARNING...This COMM PORT is MODEM STATUS REGISTER IS DEFECTIVE

- The Modem status register is one of the registers tested during

auto-login detection. If it doesn't respond properly, the UART

is probably defective and this message is displayed.

Section 6. Special Modes and Information

----------------------------------------

Currently, the only High-Speed Modem supported is the USR Courier HST and

Dual Standard /V.32 Modems. The Modem Doctor recognizes these modems upon

initial testing and sets up its test parameters a certain way to compensate

for their Carrier self test limitations.

Because of the trellis-encoded Async/modulation of the USR HST Mode, the

maximum Carrier test Baud rate is 2400 bps (see the USR Manual for this

info). To work around this, the Modem Doctor Programs these modems to use a

fixed DTE/DCE Rate with the &B1 Parameter, allowing communications to and

from the modem at speeds up to 19200 bps. When working in the V.32 mode

however, the Carrier test (and all loopback tests ) can be performed at 9600

since the modulation is symmetrical again. Also when the selected baud rate

is higher than 2400 bps, the MNP Protocol switch is set to &M0, otherwise if

there was an error, the Modem would try to correct it instead of passing the

error on to the Modem Doctor. None of these changes is saved to the non-

volatile ROM, so you should not experience a problem. However, the V.32 and

Dual Standard Modems might experience a problem if register S13 is not set to

1. This is the default from the factory and insures that the modem will be

reset when the DTR line is dropped. If you are experiencing a problem, use

the interactive mode and type S13=001. This change is not permanent.

If you are using a USR Dual Standard Modem, then the Modem Doctor will test

whatever mode is active via the B switch. If the current Modem setting is

B0, then the V.32 mode will be tested, if B1 is set, then the HST mode will

be tested. You can change this setting to whatever you wish by using the

Interactive mode, type ATZ, then type ATB0 &W or ATB1 &W. This has to be

written to the firmware in order to stay valid throughout all the testing,

but the ATZ command before a save will always recall your settings, insuring

you have only changed the one parameter. Please note, this is included for

advanced users and is one of the reasons for the interactive mode. It is not

necessary for you to do this to test your modem unless you wish to.

For more information, see the next section regarding "Quirks".

The Modem Doctor will not send any special codes to a High-Speed Modem unless

the baud rate selected is over 2400 bps. There are so many different

manufacturers extended command sets that its possible the codes for the

Courier could interfere with codes for other brands of Modems. I hope to

eventually include tests for other 9600 bps and higher modems, but for now

only the USR Couriers are supported. The reason I mention this is that as

registered users, you might try this program out on other modems, and if they

are not USR Courier compatible, they might not properly test at speeds above

2400. I have tested several other brands, and as long as they are running

what is called the "Compatible" instruction set, the Modem Doctor should run

fine.

Finally, users of 8250 compatible UARTS that do not support the Digital

(Chip) Loopback will get an error. This should be of no concern. However if

the test runs for a while, then fails, it could indicate a problem with your

UART. This is explained in the pull-down window in the program.

The Modem Doctor has been tested on about 25 Modems from Major Manufacturers

and on a variety of Clones and Compatibles (plus the "Real Thing"). It is

possible that a certain combination of hardware still might give problems,

however if you are a registered user, I will find a way to make it work.

What tests cannot Do

--------------------

Although I tried to be extremely thorough in my tests, there are somethings

that tests just cannot do or pinpoint. If you are having problems with

excessive line-noise, you might have a bad connection between your modem and

your telephone wiring in the house. Another possibility, although remote, is

that your modem does not offer the correct input impedance to the telephone

line. This can cause poor connections. The problem is with the electronic

design of the modem, and again, it is more prevalent in inexpensive modems.

Also, tests cannot check that the communications program you are using is set

up correctly. At times, installing a communications package can be very

tiresome and challenging for a novice user.

Tests also cannot check the wiring in your serial Cable. The Modem Doctor

will tell you if a handshaking pair is missing, but it cant tell you why. It

could be because the UART is bad, but more than likely, your RS-232 Cable

doesn't carry all of the handshaking signals to or from the external modem or

serial device. This can be important if you are using a high-speed modem.

Most High-speed modems will drop the CTS line when their communication

buffers are full. Previous Modems ignored the RTS/CTS signals and instead

used the DTR/DCE pair. What this means is that if you buy a new modem, it

might not work with the older-cable (yet at the same time the older modem

will work..which might lead you to believe that you have a defective modem).

Quirks with Some Modems

-----------------------

Carrier tests - Some Modems have two "AT" instruction sets, usually referred

to as Compatible and Enhanced. If your modem does not perform a carrier test

at speeds below 4800, then this is usually the case. Some modems provide a

dip-switch or a toggle switch to select either mode. Setting your modem to

the Standard Compatible Mode should get rid of your problems below 4800 bps.

At speeds above 4800 bps, the Modem Doctor is only compatible with USR V.32,

Dual Standards (V.32 and HST) and HST brand of modems. If you have one of

these modems and experience a problem below 4800 bps, the culprit is leaving

the MNP Protocol (&M4) selected instead of turning it off (&M0). You can

correct this by going to the Manual Mode, type ATZ, then AT &M0 &W. This

permanently writes the &M0 setting to the modem. If desired, you can at the

end of the test turn the &M4 switch back on the same way you turned it off.

Future versions of the Modem Doctor will include a configuration file, but

for now, I cannot write this setting to your Modem, because other Modems use

this &M0 switch to set Sync/Async applications. REMEMBER..this is only for

USR Modems below 4800 bps. Above 4800 bps (9600/19200) the Modem Doctor will

take care of this for you, since compatibility is not a problem with other

brands at these speeds.

Finally, if only one of the tests (Originate or Answer) runs and NO CARRIER

is displayed for the other, the problem lies with the Enhanced Command

settings of your modem. With the USR Modems, make sure again that you have

the &M4 switch set to &M0 for speeds below 4800 bps. Genuine Hayes 2400

Modems use &T1 through &T8 commands for their 2400 and also have other

special switches. Again for the sake of compatibility with the widest range

of modems at speeds below 4800, this cannot be corrected for in this version.

You can however use the Manual Mode and type AT &T1 to test your modem. In

fact, you can leave it in that setting, switch to the Analog Loopback test,

and perform your own custom carrier test.

Considering the vast numbers of "AT" compatible modems in the marketplace,

these two exceptions are minor and will not affect most users, and this

information is only included as a special help for those "tekkies" out there.

Section 7 Registration Information

-----------------------------------

The Modem Doctor is copyrighted by Hank Volpe. The Shareware Version of this

program may be distributed on Bulletin Board Systems or in Public Domain

Libraries as long as a Fee is not charged for its duplication or acquisition.

The Modem Doctor is distributed on as "As-is" basis, with no warranties for

usefulness or suitability expressed or implied. Users of the Shareware

version are granted a limited license to use this program. It is expected

that if you use it for more than 30 days, that you will register the Modem

Doctor.

Registration adds many powerful features that make the Modem Doctor an

excellent RS-232/UART/Modem Diagnostic Tool. Comparable programs cost well

over $100, while the Modem Doctor Registration Fee is just $19.95 for a

single copy.

Group Discount prices for software distributors and Businesses are available.

For Single-user registration, send a check or Money order for $19.95 to

Hank Volpe

108 Broadmoor Drive

Tonawanda NY 14150

You will receive the latest version of the Modem Doctor, plus several other

useful Modem Utilities. Registered Users also are entitled for updates to

newer versions (when available) for a small fee to cover shipping and

handling.

Notes -

[+]  Copyrighted by US Robotics INC

[++] Copyrighted by Hayes Inc

References to Smartmodem Copyrighted by Hayes.

References to USR/HST/Dual Standard Copyrighted by US Robotics.

References to MNP refer to An error free protocol Copyrighted by

Microcom Inc.

Disclaimer -

It is not the intent of the Author to recommend any brand of modem

over another nor to infringe on the Copyrights of the Manufacturers

by mentioning their products. A modem fits a need, and whatever one

works for you is the one you should have. As time goes on, more

modems will be added to the list of "special enhanced " tests. The

modems mentioned are done so only because of the popularity they

enjoy and for no other reason.