What a Modem Does

-----------------

The way calls were made with older telephone systems illustrates

how modems make connections. By talking the receiver off the

hook, then turning a crank, you got an operator to come on the

line. You told the operator the number you wanted, and waited

for the call to be placed for you. Until the person on the other

end picked up the receiver, you stayed connected to the

operator. The operator generally reported the progress of the

call, then gave the line to you to wait for the person on the

other end to answer. Your modem, like the operator, responds to

your commands to dial a number or answer a call, then gives the

line to your computer to begin data communications when the

connection is made.

Why You Need Communications Software

------------------------------------

Like telephones that store the numbers you frequently dial,

communications software automates your connections. This can

range from saving numbers in a "telephone book" so you don't

have to look them up and enter them each time handling the

complete process from dialing through file transfer.

You can use what is called "terminal emulation" software to

issue modem commands that set up the modem and dial a number,

but just as you make selections from within your word processor

or spreadsheet software to use a printer, the easiset most

effective way to use your modem is through communications

software. Once you've made the connections to your computer and

the telephone lines, you can almost forget the modem is there,

and access its features through the software. In the same way

you don't have to issue commands directly to the printer, you

can simply reach for the printout, knowing that the software has

taken care of everything. Your new modem will work with nearly

any communications software designed for personal computers.

Regardless of its level of sophistication, software is required

to provide the link between your modem and the computer's

keyboard.

If your software has a problem communicating with the modem,

you'll know when you start the program or when you run the

software's installation procedure. A message like "modem not

responding", means the software is not recognizing the modem. If

you've made the port selection with the software and still can't

make a connection, you'll need to use the software to manually

chnage some of the settings that control how the modem operates.

Most programs have an option for setting a modem

"initialization" or "command" string to set the modem up to talk

to the computer the way the software expects. The changes will

probably involve the way the modem and the computer use the

signals to talk to each other across the cable that connects

them. Most programs that do not work with the modem's factory

settings will operate properly if you change the way the modem

uses the signal on the DCD and DTR line. For example, at the

command line or window in which you adjust these settings,

simply type AT&C1&D2, or add &D2&C1 to the string. If the string

contains &K3, &F, or &W remove them. (These amper-sand-letter-

number combinations are modem AT commands.) The software will

use the AT commands you put in this string each time it sets up

the modem.

For software the requires additional modem setup, refer the the

software manual for specific descriptions of the settings that

frequently need to be adjusted are result code display (messages

the modem send in response to commands), and local character

echo (the way your commands are displayed on the screen). These

and other options are easily specified with an initialization

string, or with the AT Command Set described in the following

pages. If your software requires more settings to be changed,

check your software's user's guide, then refer to the sections

that follow for the particular command option. With the setup

string in place, you will be able to use your software to place

and answer calls to go on-line with another computer without

issuing the commands to setup the modem each time you start the

program.

How to "Talk" to the Modem

--------------------------

If you find it necessary to communicate directly to the modem to

set it up or to initiate calls, you are probably using terminal

emulation software. This means that the only function the

software provides is linking your keyboard input directly to the

modem. This would be like connecting your printer directly to

your keyboard; what you type would appear on the screen, and be

sent out to the printer.

Unless you have selected the option that some programs call

"local mode" or "go local", communications software doe not

operate in this way. For certain uses, like determining whether

there is a problem with your modem, the software, the line, or

the other modem, this is a helpful way of talking to the modem.

The next sections explain in detail how to setup your modem and

initiate and answer calls using AT Commands once your software

has linked your keyboard to the modem.

Issuing AT Commands

-------------------

The Hayes Standard AT Command Set for intelligent modems is the

language understood by modems used with most personal computers.

AT commands can be used to set up the modem to suit the way your

softawre and/or computer wants it to be configured. They can be

used for dialing or answering. However AT coomands are not used

to perform communications activities like sending a file. Once

on-line, you'll be talking with another computer. The command to

break the connection is probably the only instruction you would

issue after going on-line.

AT Command Format

-----------------

Modem commands begin with an AT prefix that gets the modem's

attention (also tells the modem the speed and character format

of the data coming from the computer). The AT prefix is followed

by the command(s). The modem executes the command line when you

press the key on your keyboard that you normally use to send

commands. The command descriptions here show this key as <CR>,

although it may be labeled on your keyboard as carriage return,

Enter, or Return Key. For example, the command line ATV0<CR>

gets the modem's attention and uses the V0 command option to

tell the modem to display the responses to commands (result

codes) as numbers rather than as words (the V1 option is the

factory setting). The <CR> send the command to the modem.

Many commands are used to select between two or more options.

Numeric parameters indicate the form of the command you want to

be in effect. If you issue a command without specifying a

numeric parameter, the modem assumes the 0 command option. For

example, Q0 tell the modem to respond to commands with result

codes. Issuing Q1 tells the modem not to respond to result codes

(some software programs call this "quiet mode").

A command line can contain as many as 40 characters (the AT,

spaces, and the <CR> are not counted).  The command prefix can be

entered in either all upper case or all lower case letters, but

not in a combination (e.g., AT or at, but not At or aT).

If you need to change something in the line before entering the

<CR>, use the backspace key.  To abort the command line, hold down

the key your computer recognizes as the control key (ctrl) and

press "x."  The modem will return OK without executing the

command(s) on the line.

Moving Between Command and On-line States

-----------------------------------------

Before beginning communications or after "hanging up," the modem

is in command state, ready to accept configuration commands or to

go on-line when you instruct it to dial the number of a remote

modem.  You mayalso place the modem in command state from the

on-line state without breaking the connection.

Escape Sequence--+++

--------------------

Not actually a command, the escape sequence tells the modem to

"escape" or leave the on-line state and enter the command state.

The sequence consists of a single character issued three times in

succession.  A one second "guard time" interval prevents the modem

from mistaking a ramdom occurrence of of the same three characters

as the escape sequence.  The character used in the sequence and

the duration of the guard time can be changed by writing values to

the modem's S-Registers (special memory locations described later

in this guide).

To issue the escape sequence simply hold down the shift key and

press the plus key three times (+++).  Be sure to pause at least a

second before and after issuing the three characters.  If

operating properly, the modem will return the OK result code as an

indication it is ready to accept commands.

On-Line Command--0

------------------

This command returns the modem from command state to on-line state

when a connection has been "escaped" but not broken.  If you have

placed the modem in command state and are still connected to the

remote system, issue ATO<CR> to fo back on-line.  What you type

wil then be transmitted to the modem attached to computer on the

other end of the line.  If the connection is broken, to go on-line

again you must instruct the modem to re-dial the telephone number.

Getting On-Line

---------------

In the same way voice telephone calls are made, one of the modems

must originate the call, and the other must be set up to answer

the incoming call.  Either modem may originate or answer the call.

The following examples illustrate how to use the modem to place

and answer calls from your computer.

Originating a Call

------------------

To originate a call with another system, issue the D command to

"dial" the number.  This command instructs the modem to go off

hook, claim the telephone line, and originate a call.  When you

issue dialing commands to the modem, the data is communicated to

the modem, but does not pass across the telephone line.

The following dial modifiers specify the number and method the

modem uses to dial.

-----------------------------------------------------------------

T    Specifies Tone method for subsequent dialing; follows the

D command and precedes telephone number; can be issued as a

command separate from dialing string.

P    Specifies Pulse method for subsequent dialings; follows the D

command and precedes telephone number; can be issued as a

command separate from dialing string. (factory setting).

,    Pause 2 seconds before continuing to process the characters

in the dialing string.

;    Return to command state after dialing and maintain

connection.  This modifier is useful in situations like

electronic banking where the modem needs to stay in

command state to receive other connection instructions.

0-9  Numbers and characters for the modem to dial.

*,#

-----------------------------------------------------------------

A typical dial command such as as ATD,T5552368<CR> gets the

modem's attention (AT), instructs it to go off hook to originate a

call (D), pause 2 seconds (,), and use the tone method (T) to dial

the telephone number (5552368).  The <CR> sends the commands to

the modem.

Answering a Call

----------------

Incoming calls to your modem can be answered in either of two

ways.  You can issue a command to the modem when you hear the

telephone ring.  You can also set up the modem to answer

automatically when it detects a number of rings.

Answering Manually - A

----------------------

To manually answer a call from a remote system, issue the A

command.  ATA<CR> instructs the modem to go off hook, take control

of the line, issue an answer tone, and connect with the incoming

call.  This command is used when the phone is ringing and you are

expecting a modem call (rather than a voice call).

Answering Automatically - S0=n

------------------------------

One of the modem's S-Registers provides the capability for

automatic answer.

The S0=n command ("0" is the number of the register or memory

location; "n" is the number of rings to wait before answering)

lets you specify the numbers of rings.  Setting S0 to a number

other than its factory setting of 0 configures the modem for

auto-answer.  The modem will then operate the same way as a

telephone answering machine, picking up the phone after it detects

the specified number of rings.

Answering a Call

----------------

Inconing calls to your modem can be answered in either of two

ways.  You can issue a command to the modem when you hear the

telephone ring.  You can also set up the modem to answer

automatically when it detects a number of rings.

Answering Manually - A

----------------------

To manually answer a call from a remote system, issue the A

command.  ATA<CR> instructs the modem to go off hook, take control

of the line, issue an answer tone, and connect with the incoming

call.  This command is used when the phone is ringing and you are

expecting a modem call (rather that a voice call).

Answering Automatically - S0=n

------------------------------

One of the modem's S-Registers provides the capability for

automatic answer.

The S0=n command ("0" is the number of the register or memory

location; "n" is the number of rings to wait before answering)

lets you specify the number of rings.  Setting S0 to a number

other than its factory setting of 0 configures the modem for

auto-answer.  The modem will then operate the same way as a

telephone answering machine, picking up the phone after it detects

the specified number of rings.

For example, ATS0=3<CR> sets up the modem to go off hook and issue

answer tone (no ATA<CR> is necessary) when the modem detects the

third ring.

Communicating While On-Line

---------------------------

When you are connected (on-line), data is passed across the line

between the two computers.  Either party can type information to

the other.  Characters received by the modem on the other end of

the connection will be displayed to the screen.  The user on the

other end can also type characters to your screen.  Sentences and

words will mix on the screen, however, if you both type at the

same time.

If you cannot see what you are typing, you will need to change the

local character echo setting (see Configuration Options later in

this guide).

Terminating a Call

------------------

Whenever you are ready to end communications, you must issue the

escape sequence (+++) to place the modem in the command state.

Then use the H command to hang up: ATH<CR>.  This command puts the

modem on hook and releases control of the line, so it can be used

by a telephone.

Unexpected interruptions to communicate occur with modem

connections just as with telephone calls.  Line noise, equipment

malfunction, and user error can all contribute to disruptions in

communication.  The modem indicates a loss of connection with the

NO CARRIER result code.  To re-establish the connection, simply

re-dial the number.

To interrupt call placement during the handshaking process, you

can press any key before the two modems connect.

Monitoring Operation and Call Progress

--------------------------------------

When you send a command or issue the escape sequence, the modem

responds with a "result code."  If it understands the

instructions, the modem will return the OK result code; if it does

not understand, the modem will return the ERROR result code.  The

modem also provides call progress messages to let you know how

your connection attempt is going:

-------------------------------------------------------------------

Result Code                     Explanation

-------------------------------------------------------------------

0  OK                           Command reconized and accepted.

1  Connect                      Connection established at 0-300 if

X1, X2, X3, or X4 is selected(no

speed indicated if X0 is

selected).

2  Ring                         Incoming Call

3  No Carrier                   Carrier signal lost or not

detected.

4  Error                        Command not recognized; command

line exceeds 40 characters or in

sn improper format; modem checksum

incorrect.

5  Connect 1200                 Connection established at 1200bps

if X1, X2, X3, or X4 is selected.

6  No Dial Tone                 Dial tone not detected when

expected.

7  Busy                         Number dialed is use.

-------------------------------------------------------------------

The X command allows you to set up the modem for a variety of call

progress monitoring options and corresponding result codes.

Regardless of the option selected, the modem will report the OK,

Connect, Ring, No Carrier, and Error result codes.  Other call

progress monitoring feature combinations, like reporting 1200bps

connections, detection of busy signal, and no dial tone can be

selected with the various command oprions.

-------------------------------------------------------------------

Result Code...          X0      X1      X2      X3      X4

-------------------------------------------------------------------

OK                      Y       Y       Y       Y       Y

CONNECT                 Y       Y       Y       Y       Y

RING                    Y       Y       Y       Y       Y

NO CARRIER              Y       Y       Y       Y       Y

BUSY                                            Y       Y

NO DIAL TONE                    Y                       Y

ERROR                   Y       Y       Y       Y       Y

CONNECT 1200                    Y       Y       Y       Y

-------------------------------------------------------------------

If you want the modem to wait for a dial tone before dialing, you

should select either the X2 or X4 command options.  Either of

these configures the modem to wait until it detects a dial tone

before proceeding; if it does not, it returns the NO DIAL TONE

result code.  The same is true for configuring the modem's

response to a busy signal; if X1 or X4 is selected, the modem

recognizes a busy signal from the other end and returns the BUSY

result code.  The X4 option provides full call progress

monitoring.  However, X0 is the factory setting because many

software programs expect 1200bps modems to be configured initially

with the basic call progress capabilities and corresponding result

codes.

Managing Modem Configuration

----------------------------

When your modem is first installed and powered up, it is

configured with the settings selected at the factory.  The options

specified for each command and the value is each of the modem's

S-Registers determine how the modem operates.  The modem's factory

settings suit a wide range of user applications, while taking

advantage of the modem's full capabilities.

Creating a Special Configuration

--------------------------------

If your software or the system with which you are attempting to

connect required that you set up the modem to operate other than

with factory settings for commands and S-Registers, you can create

a special configuration, or profile, by issuing AT commands.  The

configuration you create will be used until you issue commands to

change the settings, or reset the modem (either with the Z command

or by unplugging/replugging it).

Resetting the Modem

--------------------

If the modem ever seems to be set in a way that makes it not

communicate as it should, or if you simply want to restore the

factory settings, you can reset it with the Z command.  ATZ<CR>

performs a "soft reset," replacing values currently in S-Registers

and AT command options to factory settings.  The modem's factory

configuration is also recalled when the modem is unplugged and

plugged back in, or if power is lost to the wall plug.  If you are

experiencing any difficulty with the modem, you may wish to issue

the Z command before unplugging the modem or issuing the test

commands.

Adjusting Modem Speed

---------------------

Modem speed is measured in bits per second(bps).  For example, if

Hayes Personal Modem 1200 is operating at its highest rate, data

will be transmitted at 1200bps.  This speed is not set by an AT

command; it is the speed at which the software sends the AT prefix

(within the range supported by the modem) that determines the

speed at which the modem attempts to make the next connection.

Some programs offer an option for transmission sprrd, often called

"baud rate," to specify the speed of the connection.  The software

sends the AT prefix at the desired speed.

In addition, the transmission rate of the connection will be

determined by the highest common speed supported by the two

connecting modems.  For example, if the modem at the other end is

capable of a maximum speed of 300bps, the connection and data

transmission will be at 300bps, even if your modem can communicate

at 1200bps.

Choosing a Communication Standard

---------------------------------

Communication standards are protocols that determine how the

modems perform their connection handshake and the format and speed

used to transmit data.  For example, connections at 300bps use the

domestic 103 standard; connections at 1200bps use the domestic

212A standard.  Because standards are closely linked to speed, the

modem selects the appropriate standard as it adjusts to the speed.

Testing the Modem

-----------------

In addition to the diagnostics the modem automatically performs

each time it is powered up, the I command can be used to gain

information about the modem.

I0              Modem reports Hayes product code.

I1              Modem calculates and reports ROM Checksum (3-digit

number).

The results of these tests are primarily used by Hayes Customer

Service and certain software programs to determine the

manufacturer/model of the modem you are using, and its feature

set.

Refer to the descriptions in the Personal Modem 1200 Installation

Guide for troubleshooting suggestions.  If you are able to issue

commands to the modem and it returns an OK result code, the modem

is probably operating correctly.  If the modem does not seem to be

responding, try resetting the modem (see above).

Setting Local Character Echo

----------------------------

Your computer and software combination may require that you adjust

the character echo feature of your modem to provide display of

characters as you type commands, or to prevent double characters.

Two options are available:

E0              Modem does not return (echo) characters from the

keyboards to the screen when in command state.

E1              Modem returns (echoes) characters typed from the

keyboard to the screen when in command state

(factory setting).

This command is effective only if you are entering instructions

from a command line.  The settings for this command do not affect

the display of characters while you are on-line with another

system.  The software must provide on-line character echo, if

desired.

Adjusting Result Code Display

-----------------------------

The two commands below configure the modem's use of result codes.

Q0              Modem responds to commands with result codes

(factory setting).

Q1              Modem does not respond to commands with result

codes.

V0              Modem reports result codes as numbers.

V1              Modem reports result codes as words (factory

setting).

Some software requires that result codes not be displayed, or that

they be numbered rather than as words.

Configuring Modem Signals

-------------------------

Some computers or software expects  settings for signalling

between modems and between the modem and the computer to be

configured in ways other than that set at the factory.  Any

special modem-to-computer signalling requirements should be

described by your software's user's guide or your computer's

owner's manual.

The C command controls the modem's response to an incoming carrier

signal over the line from the remote modem.

C0              Modem ignores an incoming carrier signal; the

transmitter carrier is always off.

C1              Modem turns the transmit carrier signal on and off

to reflect the presence of and incoming carrier

signal.  When the modem originates a call, answers

a call, or is connected to another system, the

signal is on, and is off when the modem is not in

one of these situations (factory setting).

The &C command determines the way the modem uses the carrier

signal when controlling the Data Carrier Detect (DCD) line (pin 2

on modem-to-computer cable).

&C0             Modem presumes the presence of the carrier signal

and maintains the DCD signal on at all times.

&C1             Modem turns the DCD signal on when it detects a

carrier signal from the modem on the other end of

the telephone line, and off when it does not

(factory setting).

The &D command selects the modem's response to signal sent from

the computer on the Data Terminal Ready(DTR) signal (pin 1 on the

modem-to-computer cable).

&D0             Modem presumes presence of DTR signal and ignores

the actual status of the signal (factory setting).

&D1             Modem monitors DTR signal.  When communications

software is used to cause an on-to-off transition

of DTR signal, the modem enters command state and

ignores AT commands.  When the software causes an

off-to-on transition of DTR, the modem will again

respond to AT commands and can be returned to the

on-line state (if the connection has not been

broken) with the AT0<CR>.

&D2             Modem monitors DTR signal.  When communications

software is used to cause an on-to-off transition

of DTR, the modem will again respond to AT

commands.

Writing to the Modem's S-Registers

----------------------------------

S-Registers provide special access to setting that determine modem

operation.  The factory-set values stored in these registers are

recalled and used to control the modem each time it is powered up

or reset.

Aside from using S0 to store the number of rings for automatic

answer (described earlier), you will have little need to change an

S-Register value.  However, if your use of the modem requires that

one or more of these be changed, you can do so by using the Sr

command:

Sr?             Modem reads and responds with the value of

register "r" (r=number of register; "?" requests

the value).

Sr=n            Modem sets the value of register "r" to "n"

(n=value within the range of the register).

Writing to the modem's S-Registers is not recommended unless your

use of the modem requires that one or more of these settings be

changed.

For your refrence, the modem's S-Registers, their function, ranges

in units, and factory settings are provided in the chart below.

-------------------------------------------------------------------

S-Register       Number &          Units     Range        Factory

Function                                 Setting

-------------------------------------------------------------------

S0      Ring to answer on       Rings      0-255         0

S1      Rings received          Rings      0-255         0

S2      Escape sequence         ASCII      0-127        43

character

S3      Carriage return         ASCII      0-127        13

character

S4      Linefeed character      ASCII      0-127        10

S5      Backspace character     ASCII     fixed at       8

S6      Wait time for dial     seconds    fixed at       4

tone

S7      Wait time for carrier  seconds    fixed at      45

S8      Duration of comma      seconds    fixed at       2

S9      Carrier detect         1/10 sec     1-255        6

response time

S10     Delay between loss     1/10 sec     1-255        7

of carrier signal

and modem hang up

S11     Duration of DTMF        msec      fixed at      95

tones for dialing

S12     Escape sequence        1/50 sec    20-255       50

guard time

S25     Delay between loss     10 mec       1-255        5

of DTR signal and

modem hangup

S30     Delay time from no      10 sec      0-255       60

modem activity until

auto hangup

-------------------------------------------------------------------

Cable Connections

-----------------

The table below describes the pin number on the modem's cable, the

direction the signal pass, and the name of the signal used by the

modem.

-------------------------------------------------------------------

Pin     Direction                       Signal

-------------------------------------------------------------------

1       From Computer                   DTR - data terminal ready

2       To computer                     DCD - data carrier detect

3       From computer                   TxD - transmit data

4       Not applicable                  ground - signal ground

5       To computer                     RxD - receive data

6       To computer                     DSR/CTS - data set ready/

clear to send

7       To computer                     RI - ring indicator

8       Not applicable                  ground - signal ground

-------------------------------------------------------------------

The housing of the connector provides a shield that reduces noise

and interference.

The actual pin number on which a signal is carried will vary

depending on the adapter used.

The table below cross-refrences the pin numbers used for signals

on the 8-pin DIN with those for the adapters provided with your

modem.

-------------------------------------------------------------------

Signal            DIN-8           DB-9            DB-25

-------------------------------------------------------------------

DTR                 1               4              20

DCD                 2               1               8

TxD                 3               3               2

ground              4               5               7

RxD                 5               2               3

DSR/CTS             6               8               5

RI                  7               9              22

signal ground       8               5               7

shield           shield          shield             1

NOTE:  If you construct an adapter for use with the modem, consult

your computer's owner's manual for the signals supported and the

appropriate pin numbering.

#################################################################################################################