PORTVIEW information

--------------------

Disclaimer: The author assumes no responsibility whatever for, and makes no

warrantee whatsoever with regard to this software. This program is supplied

AS IS, and is NOT guaranteed to operate on any particular system. Use it at

your OWN RISK.

This software is NOT Public Domain. It is being distributed free of charge

to Radio Amateurs and Computer Hackers for NON-COMMERCIAL use ONLY.

Background: Communication Interfaces are complex little critters to debug.

I developed this program to help analyze what the heck is coming over the

RS-232 line between computers and modems. (modems include Packet TNC's,

and Amtor Terminal Units.

Portview is meant to be used as an analysis tool to test communications

interfaces. It supports most common ASCII baudrates and protocols. Both

7 and 8 bit word sizes are supported.

The program's flexibility allows it to be used as an advanced split-screen

terminal, an electronic mailbox system, or protocol simulator. Though it

does have limitations, the program is designed to provide most needed

simulation functions.

Available Functions include:

1. Save incoming binary data to file, separate A and B port files.

2. Send a file to selected port.

3. Send a binary string of characters to selected port.

4. Execute up to 8 commands on match of up to 8 input strings.

5. Play tone strings (music) on match of input string.

6. Run DOS commands.

7. Run remotely to analyze a system.

8. Split-Screen Terminal.

9. Display Character lists in Hex and Decimal.

10. Transfer binary files between computers

11. There are many more.

Running the Program:

When PV first comes up, it displays the "Author's Ego" then a 2 window screen.

The Top window is for COM1, and the lower window COM2. If you do not have a

second serial port, the lower window (Port B) will not be used. (In that case

Only outgoing text will show in it)

PV will run under DV (Desqview). Give it 200K of space, Direct screen writes

= Y, MUST run in both foreground and background, keyboard difficulty = 4,and

any BBS system must NOT be active. The WA7MBL Packet BBS RS-232 Interrupt

handlers are OK, but the BBS itself is NOT! PV will restore everything as it

was when it exits. This means that interrupt handlers will not have to be

reloaded! PV uses interrupt vectors 0bh, 0ch, 1bh, 1ch, 23h, and 24h while

loaded.

Display Modes:

1. Analyzer mode. This is the default. All characters are displayed, including

spaces, binary zero, and  hex ffh. Makes it easy to count spaces and nulls.

In this mode linefeed and carriage return are only printed characters. The

window "wraps" to the top instead of scrolling. The cursor is a blinking "+"

plus-sign. Alt-S switches Screen modes.

2. Text mode. This is most useful for standard terminal usage, as linefeed,

carriage return, and tabs and spaces produce the standard result. In this

mode the windows "scroll" when text reaches the bottom of the window, and

the only translated character is hex ffh. Nulls are ignored.

3. Extended Hex mode. This mode displays the character followed by its

hexadecimal value. This is an override mode, and is only active while

the Alt-E toggle is active (shown by a greek Sigma character on the status

line.

There are 3 operating modes:

1. Monitor. This mode is meant to "tap into" a RS-232 line using a specially

built "4 ended" cable. One window shows flow from A to B, and the other shows

flow from B to A. This allows monitoring of flow without distrubing handshake

signals or line voltages. The program is on a separate computer, monitoring

the flow between a computer and modem or another computer. Pin 3 of each

port is the INPUT. Connect up cables so that pin 3 hooks to pin 2 of the

computer you are analyzing on port A, and to pin 3 of that computer's port

on Port B of our Analyzer computer.

2. Terminal. User terminal mode when you need to send data.

This mode operates like a standard terminal. You can use this mode to talk to

Packet TNC's or Amtor/RTTY terminal units. You can connect a TNC to both COM1

and COM2 and see text incoming from both at the same time. If the selected

port is "A", text being sent shows up in a different color in the "B" window.

This split-screen operation is handy for interactive Packet operation. Use

the F6 key to change the port you want to transmit through.

3. Xover mode. Crossover mode was installed as a protocol translator. You

can go in one port at 19200 baud, and out the other at 75 baud, for instance.

Also, this mode allows direct connection of 2 modems "back to back". I often

use it as a Gateway between systems. You can connect just about any

dissimilar systems with it.

Macros:

Macros are strings you can assign to a regular number or letter key.

They can contain any value from 0 to ffh hex, and may be up to 32k bytes

in length. A data "packet" can be entered which can then be sent to a host

system or Point-of-sale terminal for testing by just one key sequence. Most

systems require the packets to be continuous, i.e. if time elapses between

characters (like when you try to type it) the receiving system times out.

Macros solve this problem as they are sent as a group.

A checksum, either an LRC (BCC 8 bit Xor) or CRC (CRC-CCITT 16 bit) may

be inserted into the macro with a single keystroke. The F5 key allows a

beginning offset to be entered so that the leading (n) characters will not

be included in the checksum calculation. Typically, the SOH character to

begin a packet is not included in the checksum.

In a typical test situation, connect the host computer to Port A of your

test computer (running PV), the TNC or modem to Port B of the test computer.

(a full-handshaking NULL MODEM adapter is required for the computer to

computer port connection)

Select the proper baudrate stopbits etc, and enter Xover mode (F5 Key).

In this mode you can watch the data flow between the computer and the

modem. Toggle F5 to enter Terminal Mode. You can select either A or B

Port to send data packets (macros you have made) to either the computer

or the modem. In this way you can quickly see if the resulting responses

are correct. To further simulate a system, use the "RESPOND Mode".

Respond Mode:

The Shift-F8 function creates a response command string. A series

of up to 8 command responses for up to 8 different match "elements" can

be set up. Enter binary strings via the Alt-F8 function. These strings

can be used for comparison, can be sent as a macro, or may contain a

filename for the "sendfile" and "savefile" functions.

Enter strings via the Alt-F8 function.

Activate the Matcher using Alt-1 Alt-2 Alt-3 or Alt-4. If no match string

has been entered, or if a macro string is NULL, activation of the Matcher

for that match element will not occur.

Properly constructed Sound Macros can produce sound output from the

speaker when a match of the input string occurs. Sound Macros must be

in the following form:

1000,100  0,20 330,20 256,40  ...etc.

|  | | | | |

|  | | | | `-- Duration for wait in hundredths of seconds

|  | | | `-- Zero frequency to just wait quietly

|  | | `-- Space delimiter between groups (any number of spaces)

|  | `-- Duration in hundredths of seconds

|  `-- Comma delimiter

`-- Frequency in Hz

Sound output can be used to determine that a particular string has been

received when the screen is across the room, or out of sight.

(Can also be used to play tunes...just for fun!)

Saving Data:

Default "capture" files opened by the F3 and F4 keys add new data to the

end of the file, retaining previous data.

Use Alt-F3 and Alt-F4 to open Named save files. These files must be given

unique filenames. They will not allow over-writing of an existing file.

When named files are saved in Respond mode the existing file is renamed

to TMPn.$$$ (where "n" is a number) In this way files are kept safe from

overwrite, yet the same filename can be used to store a number of incoming

files. Only the last stored file will be named with the name specified in

the Respond command string.

Sending Files:

Use F7 to send a file to the selected port. Its data will show in the

opposite window as it is flowing into the transmit buffer.

To abort a file send use Alt-F7. This closes the file, and clears the xmit

buffer. On very long files "BUFFER FULL" may intermittently flash in the

error area of the Status Line. This means that the program has found no

room for any more data, and is waiting until the buffer has room before

putting more data into it.

Binary File Transfers:

Program files can be sent between computers using PORTVIEW by setting up

the Response commands properly. Use match strings that are very unlikely to

occur in any file such as "bin-file-xfer-begin" and "bin-file-xfer-end".

The strings may be as complex as you like...but, you must remember it! I

create macro strings in the transmit system for each one, so that all I

have to do is send the begin macro, the file send, and then the end macro.

That way there are no typographic errors (which would ruin the file).

Set  up the receive system using two compare elements. The  first

should  be  set  up with "7" (close files) and  "5"  (named  file

open/close for port A). This sequence assures proper open of  the

file.  The second compare element should be set up with only  the

"5"  (named  file open/close) command. This second  command  will

be used to close the file. Set up key strings for open and close,

and  enter  the  filename string. The filename  MUST  be  in  the

current  default  directory.  It cannot contain  a  path  string,

however  it may contain a drive specifier. i.e.  A:MYFILE.DOC  or

MYFILE.DOC  are ok, but A:\STUFF\MYFILE.DOC or A:\MYFILE.DOC  are

not. The backslash is not allowed.

For  example,  use  the Shift-F8 key to enter  the  Respond  Mode

loader. press "1" to choose the first compare element. Next,  press

Ctrl-A. This makes Ctrl-A the key to the "openfile" string.  Now,

press "7" the  "close  all files"  command.  Since  there  is  no

parameter with this command, press Enter. Assuming transfer  will

be  from Port-A, press "5". This opens a named file. Press F.  This

key  will  represent the filename string. To end, press  the  Esc

key. Press any key to return to terminal mode.

The second step is to use Alt-F8 to enter the Macro String Loader

function. Press F and enter the filename you want to use. In like

manner  use  Alt-F8  again, with Ctrl-A as  the  key.  Enter  the

openfile command string. I use open-transfer-file. It should be a

string that would never be contained in normal text flow.

To complete the programming, use Alt-F8, this time with Ctrl-B as

the key. Enter the close-file string. I use  close-transfer-file.

It  is vital that this string be one that is NOT in the file  you

are transferring. The string can be as complex or long as you like.

Activate the Alt-1 and Alt-2 compare elements.

On  the  transmit  system, set up macro  strings  containing  the

openfile and closefile strings. These must be exactly the same as

the ones used in the receive system.

To  use the transfer now that it is programmed, simply  send  the

openfile  macro from the transmit end. Next, send the file  using

the F7 key. When transfer is complete, send the close-file macro.

Do  not hit any other key before the close-file macro is sent  by

mistake! If you do, that character will end up in the file.

Errors:

For all file operations, the error message "Device Error" will display

if a DOS Error occurs, such as when a floppy drive door is not closed.

When executing DOS commands via Alt-F6  device errors will cause the

familiar "Abort,Retry,Ignore?" message. It is "safe" to choose any of

the 3 options, and will not cause problems in the program.

The error message "Binary Trunc Err" usually means an attempt has been

made to save a named file (using the Response Mode) using a pathname.

Paths are not allowed for this kind of file-save operation. The file

will be saved anyway, but the match string will be appended to it.

Color:

This program makes use of color. Receive and Xmit data differ in color

making it easy to identify one from the other.

The default color scheme is RED=Receive, MAGENTA=Xmit. (Ugly!)

Characters that have been translated by the program, such as spaces, tabs,

ffh hex, and others are displayed at a lower brightness level.

The color scheme may be changed with Alt-C. This changes Both Receive and

Xmit colors. To change only the Xmit color use Alt-X. Any combination can

be attained by using both keys.

Pause:

Pause the display with the Ins  key. Press Ins the second time to restart

data flow. Flow is quick, so you have to be ready to hit the Ins key FAST.

Misc Info:

Buffer sizes (each port): Xmit = 4k, Receive = 8k

Handshaking inflow:  RTS & DTR are dropped when buffer is full.

outflow: CTS & DSR must be high for file or macro send.

Data from the keyboard is always sent, handshaking is ignored. If a file is

being sent, and a keyboard character is typed, handshaking is turned off

and the data flows until the next block is put in the transmit queue by

the file function.

XON/XOFF protocol is NOT supported!

If you have any comments, reports of bugs, or suggestions you think might

enhance the program, I would be happy to receive a message about it.

-Del  K7BUC

2514 E. Roma

Phoenix, Arizona 85016