How long distance charges are calculated

The following information deals specifically with calling from Ontario and

Quebec (Bell Canada). However, it may be possible to obtain similar

information from the tariffs of other companies.

The Bell tariffs specify how much a long distance call costs. First, all

calls from a given exchange will have the same set of rates. That is,

calls to Hamilton will cost the same from Etobicoke as they do from

Scarborough since both Etobicoke and Scarborough are considered to be

part of the Toronto (Canada) exchange.

The "rate distance" is used to determine the distance from one general

exchange to another, and the rate billed will be according to the rate

distance. To calculate a rate distance, you need to find out what the

"V-H" co-ordinates are for the place you call from and the place you call

to.

Toronto  has a V of 4981 and an H of 2488.

Hamilton has a V of 5097 and an H of 2504.

The differences between V and H are taken. In the Toronto-Hamilton case,

the difference of V is 4981 - 5097 = -116 and H is 2488 - 2504 = -16.

Ignore minus signs to get the differences: DV = 116 and DH = 16.

Divide DV and DH by 3, rounded to nearest integer. Thus, DV becomes

116/3 = 38 2/3, round to 39. DH becomes 16/3 = 5 1/3 round to 5. Square

the new DV and DH to get 39*39=1521 and 5*5=25. Add the squares to get

1521+25=1546. If this result is less than 1778 (and it is), multiply by a

special multiplying factor (which is 0.9 here) to get 1546*0.9=1391.4,

then take square root: sqr(1391.4) = 37 + fraction. Round *up* to 38

(don't round if the square root comes out exactly) and that's the rate

distance.

Now you can have the situation if that sum of squares above

goes to 1778 or higher. What happens is that you divide DV and DH by

three a second time (going back to *), and repeating as many times as

necessary. Keep a count of how many times 3 is divided into DV and DH.

Once the 1778 barrier is finally crossed, the multiplying factor will be

0.1 * (9**N) where N is the number of times 3 is divided (always have N

at least 1, for the first division).

A minimum rate distance is assigned according to "N" (the number of

divisions of 3). For N=1, there is no minimum (no problem with our

Hamilton calculation, as N=1 there). For N=2, though, the rate

distance must be at least 41 (use 41 if the calculated rate distance

doesn't get that high). For N=3, the minimum is 121. In fact, when N

goes above 1, the minimum rate distance is 40 * (3 ** (N-2)) + 1.

And that's the way it's done... in Ontario and Quebec, Canada, the full

details are in CRTC tariff 6716, that is, the main Bell tariff which

should be available through Bell's Business Office or the Canadian

Radio-Television and Telecommunications Commission. To get the V and H

co-ordinates for whatever centres are involved, you need to dig up CRTC

tariff 7063 (Ontario, Quebec, eastern NWT, St Pierre & Miquelon), or

AT&T Communication Tariff FCC #10 (for everywhere else). FCC #10 is

presumably available through the U.S. Federal Communications Commission,

or through AT&T.

In the Toronto-Hamilton example, the 38 rate miles (assuming no calculation

errors) represents a base rate of 31 cents/minute (as of September 1989)

The discount for the given time is then applied, but all long distance

calls have a minimum charge of 37 cents each. These rates and rate

calculations are subject to change, however.