DIALING CODES     (Telepress, 12 May 1990)

General contents of this file:

1. Area Codes, North America - a description of the numbering plan

2. Reserved exchanges within each area code

3. Other Information - refers to GUIDECOD.DOC file

4. Area Codes for North America

5. Country Codes for International Dialing

-----------

1. Area Codes, North America

Bell Communications Research (Bellcore) is the organisation responsible

for the maintenance of the area codes and the dialing setup for

North America since the breakup of AT&T several years back. This

group sets standards for dialing, telephone number format, etc.

Area codes (or Numbering Plan Areas (NPA)) in North America are presently

of the form NZX where N represents a number from 2 to 8, Z is 0 or 1 and

X is any number. This distinguishes area codes from central office (exchange)

codes which are usually of the form NNX (ie. the difference between an

exchange and an area code is usually in the second digit, whether or not

it is a 0 or 1).

Area codes ending in -11 are not used, as they are normally used for special

services. The special purposes generally reserved are:

411 directory assistance

611 repair

811 business office

911 emergency

The other N11 codes are sometimes used within specific localities for

special purposes (711 is used for mobile service in British Columbia,

in the form 0+711)). In some cases, services are accessed through the

long distance network (ie. 1+411 or 1+611).

Area codes ending in -10 have been TWX numbers (teletype service).

TWX area codes include 510, 610, 710, 810 and 910. Normal telephone

numbers do not (yet) use area codes ending in -10, though these codes

will be reassigned to regular telephone service as TWX is eliminated.

Upcoming examples of this are the area code splits of 213 (Los Angeles

adds area code 310) and 415 (San Francisco adds area code 510).

Area codes ending in -00 are usually reserved for special services

like 700, 800 or 900. In fact, these codes are not considered to be

area codes as such, but Service Access Codes (SACs).

(2) Dialing Codes...

Theoretical capacity of the telephone system:

The original design of telephone numbers was:

(NZX) NNX XXXX

In theory, this gives:

N   Z   X    N   N   X    X    X    X    X

8 * 2 * 10 * 8 * 8 * 10 * 10 * 10 * 10 * 10 = 1.024 billion numbers

However, as some area codes and exchanges are reserved for special

purposes (such as 411, 555 exchange, etc), the total possible number

of telephone numbers will be somewhat less.

As the exchange codes in some area codes were used up, some central offices

started using the NXX format, where the middle digit can then be a 0 or 1;

examples of these may be found in New York City or Los Angeles. The March

1990 conversion of the (416) area code around Toronto is another example,

as is a similar change to North Carolina's dialing around the same time.

The dialing is changed so that dialing 1+ a number always requires an area

code, and in the case of (416), will mean that 1+416+ will be needed for

long distance calls within (416). In New York City, however, all calls

within an area code are dialed with seven digits only (without dialing

1 first, let alone the area code).

Now, telephone numbers look like this:

(NZX) NXX XXXX

This gives a potential of:

N   Z   X    N   X    X    X    X    X    X

8 * 2 * 10 * 8 * 10 * 10 * 10 * 10 * 10 * 10 = 1.28 billion numbers

However, codes like 411 and 611 would not be assigned because they will

still be needed for services such as directory assistance and repair.

Nevertheless, some unused N11 codes like 211 may be found in some area

codes as active exchanges. It's also not a good idea to assign the home

area code (could cause confusion).

Going from NNX exchange codes to NXX only represents a 25% increase in the

total theoretical amount of telephone numbers, and not all area code regions

are expected to run out of exchanges. Also, area codes do not generally go

outside a state or provincial boundary (the only exceptions are in Canada,

where 902 serves both Nova Scotia and Prince Edward Island, area 819 which

covers the eastern Northwest Territories as well as part of Quebec, and

area 403 which covers Alberta, Yukon and the western Northwest Territories).

The ultimate goal is not only to use area codes for exchanges codes, but

to use exchanges codes for area codes also. This means that telephone

numbers will ultimately look like this:

(NXX) NXX XXXX

This gives a potential of:

N   X    X    N   X    X    X    X    X    X

8 * 10 * 10 * 8 * 10 * 10 * 10 * 10 * 10 * 10 = 6.4 billion numbers

With a five-fold increase in the number of possible area codes, there

should be plenty of room to grow for some time.

(3) Dialing Codes...

Basically, all calls within an area code will ultimately be dialed in

one of the following ways 1) dial seven digits, 2) dial 1 + home area

code + number, or 3) dial 1 + seven digit number within area code, then

wait for a few seconds to time out. One alternative not mentioned in

official documents (for touch tone phones) is to use 1 + seven digit

number in home area code + '#' with the # key terminating the dialing.

The initial set of new area codes will take the form NN0, or those numbers

ending in '0'. This means that areas codes that do not have exchanges

ending in '0' (or only a few NN0 exchanges that could be renumbered)

can still be able to tell the difference between an exchange and an

area code by looking at the first three digits.

The new NN0-type area codes will be assigned starting with these

first few codes (in order):

260, 480, 520, 590, 650, 220, 250, 490, 660, 680, 720, 730, 850, 940 ...

970 will be reserved for phone testing purposes, and is not slated to

become an area code.

All telephone systems are expected to change their systems over to allow

for the new style of area codes by 1 July 1995, or perhaps earlier. This

could mean that other area codes will be required to dial the area code

for long distance calls within area, even though the area code is not

running out of exchanges itself (eg. areas 519, 705).

Dialing Scheme

* 1 + will generally be used for direct-dialed long distance calls within

North America, especially calls outside the local area code.

* 0 + is used to dial operator-assisted or automated credit card calls within

North America. After 0 + (area code) + number are dialed, a prompt tone

(same tones as a dial tone, but for a very short duration) will be issued,

then one of the following actions will be taken:

1) wait for a few seconds, then an operator will come on line

2) dial '0' to get the operator immediately (for a collect, person to

person call, etc)

3) dial the telephone company credit card number for billing

It is unclear what will happen in the cases of automated collect calls,

as to what kinds of dialing would be standard in that case.

(4) Dialing Codes...

Dialing 0 and waiting will get the local area operator.

* 00 is used in the U.S. to get the operator for a default long distance

carrier. This is used as most long distance companies have their own

operators. A single 0 digit will call up the local operator (with the local

telephone company as opposed to the long distance company).

It's important to note that the U.S. is broken up into LATAs (Local Access

Transport Areas), and long distance calls are treated differently whether

they are made within a LATA, or if a call is placed to another LATA. Thus,

there could be an operator for within the LATA (intra-LATA calling) and an

operator for inter-LATA calling (from a long distance carrier).

* 01 is used for overseas calls. 01 + indicates an operator-assisted or

automatic credit card call, while 011 + indicates a direct-dialed overseas

call. 010 is reserved for some unspecified future use.

* 10XXX + is used in the U.S. to indicate which long distance carrier to use

in a situation known as "equal access". This allows a telephone subscriber

to select a long distance company for a particular call. For instance,

10288+ gets AT&T while 10222+ gets MCI and 10333+ US Sprint. After this

code, a 1 or 0 is dialed (to indicate direct dial or operator-assisted call)

then the number to be called.

The codes 10000 is not available for assignment

10001 - 10099 are reserved for restricted purposes

10100 - 10199 are reserved for international carriers

(note that because of possible dialing mistakes that may

confuse some codes with 01- overseas dialing codes,

10100-10119 are to be assigned after all 10120-10199 codes

are assigned.)

10200 - 10999 are assigned to standard long distance carriers

A list of the U.S. long distance carriers and their 10XXX+ codes is listed

in a separate document.

Canada doesn't have to worry about this code quite yet, depending on the

results of the CNCP/Rogers intention to provide alternate long distance

service.

* 11- or '*'- (tone dialing) is reserved for special calling services like

call waiting functions, etc. For instance, 1170 is used to disable the

call waiting, like *70 can be used on touch tone phones. The current

special calling codes on many local telephone systems are:

*57 or 1157 - call tracing request (some systems use this for call back)

*60 or 1160 - call blocking activated

*61 or 1161 - priority ring activated

*63 or 1163 - select call forwarding activated

*66 or 1166 - repeat dialing activated

*67 or 1167 - call number ID blocking (done for each call)

*69 or 1169 - call return activated

*70 or 1170 - disable call waiting

*71 or 1171 - 3 way calling according to usage

*72 or 1172 - enable call forwarding

*73 or 1173 - disable call forwarding

*74 or 1174 - modify speed calling directory entry (for 8 number service)

*75 or 1175 - modify speed calling directory entry (for 30 number service)

*76 or 1176 - call pickup

*79 or 1179 - ring again

*80 or 1180 - call blocking disabled

*81 or 1181 - priority ring disabled

*83 or 1183 - select call forwarding activated

*86 or 1186 - repeat dialing disabled

*89 or 1189 - call return disabled

(5) Dialing Codes...

2. Reserved exchanges

Each area code will have certain exchanges set aside for special purposes.

These exchanges are:

555 - directory assistance

950 - used to access other long distance services (U.S.)

958, 959 - reserved for phone tests

976 - special recordings, private services

844 and 936 have been reserved for time and weather services, but this

is not widely adhered to. In area code 416, these exchanges are for

Oakville and Tottenham respectively. These services will likely be

incorporated into 976 service, or into other exchanges.

Other Information

A file with the Tel Codes package, GUIDECOD.DOC, explains in more detail

the 700, 800 and 900 area codes, with lists of exchange/carrier assignments

and other details. This file should be included in Tel Codes from

release #7 henceforth.

(6) Dialing Codes...

4. Area Codes / North America

200 - [-- in some areas, for testing --] 201 - New Jersey (Newark, Jersey City)

202 - Washington DC                      203 - Connecticut

204 - Manitoba                           205 - Alabama

206 - Washington (Seattle)               207 - Maine

208 - Idaho                              209 - California (Fresno)

210 - [** not used yet **]               212 - New York City (Manhattan, Bronx)

213 - Los Angeles                        214 - Texas (Dallas)

215 - Pennsylvania (Philadelphia)        216 - Ohio (Cleveland, Akron)

217 - Illinois (Springfield)             218 - Minnesota (Duluth)

219 - Indiana (Fort Wayne, South Bend)

300 - [** not used yet **]               301 - Maryland

302 - Delaware                           303 - Colorado (Boulder, Denver)

304 - West Virginia                      305 - Florida (Ft Lauderdale)

306 - Saskatchewan                       307 - Wyoming

308 - Nebraska (western)                 309 - Illinois (Moline, Peoria)

310 - [California, eff. 1 Feb '92]       312 - Illinois (Chicago)

313 - Michigan (Detroit)                 314 - Missouri (St Louis)

315 - New York (Oswego, Syracuse)        316 - Kansas (Wichita)

317 - Indiana (Indianapolis)             318 - Louisiana (Shreveport)

319 - Iowa (Dubuque)

400 - ** not used **                     401 - Rhode Island

402 - Nebraska (Omaha)                   403 - Alberta, NWT, Yukon

404 - Georgia (Atlanta)                  405 - Oklahoma (Oklahoma City)

406 - Montana                            407 - Florida (Orlando)

408 - California (San Jose)              409 - Texas (around Houston)

410 - [-- TWX: Northeast US --]          412 - Pennsylvania (Pittsburgh)

413 - Massachusetts (Springfield)        414 - Wisconsin (Milwaukee)

415 - California (San Francisco)         416 - Ontario (Toronto)

417 - Missouri (southwest)               418 - Quebec (Quebec City)

419 - Ohio (Toledo)

500 - [** not used yet **]               501 - Arkansas

502 - Kentucky (Louisville)              503 - Oregon

504 - Louisiana (New Orleans)            505 - New Mexico

506 - New Brunswick                      507 - Minnesota (Rochester)

508 - Massachusetts, surrounding Boston  509 - Washington (Spokane)

510 - [San Fran. CA, eff. 7 Oct '91]     512 - Texas (Austin, San Antonio)

513 - Ohio (Cincinnati)                  514 - Quebec (Montreal)

515 - Iowa (Des Moines)                  516 - New York (Long Island)

517 - Michigan (Lansing, Saginaw)        518 - New York (Albany)

519 - Ontario (London)

(7) Dialing Codes...

4. Area Codes / North America (continued...)

600 - [** not in active service **]      601 - Mississippi

602 - Arizona                            603 - New Hampshire

604 - British Columbia                   605 - South Dakota

606 - Kentucky (Covington)               607 - New York (Binghamton)

608 - Wisconsin (Madison)                609 - New Jersey (Atlantic City)

610 - [-- TWX code for Canada --]        612 - Minnesota (Minneapolis)

613 - Ontario (Ottawa)                   614 - Ohio (Columbus)

615 - Tennessee (Nashville)              616 - Michigan (Grand Rapids)

617 - Massachusetts (Boston)             618 - Illinois (Centralia)

619 - California (San DIego)

700 - Special features within U.S.       701 - North Dakota

702 - Nevada                             703 - Virginia (Arlington)

704 - North Carolina (Charlotte)         705 - Ontario (North Bay)

706 - [ ** formerly NW Mexico ** ]       707 - California (Santa Rosa)

708 - Illinois (*new* as of 11 Nov 89)   709 - Newfoundland

710 - [-- TWX: Southern US --]           712 - Iowa (Council Bluffs)

713 - Texas (Houston)                    714 - California (southwest)

715 - Wisconsin (Eau Claire)             716 - New York (Buffalo)

717 - Pennsylvania (Pottsville)          718 - New York City (Brooklyn, Queens,

719 - Colorado (Pueblo)                            Staten Island)

800 - Toll-free '800' service            801 - Utah

802 - Vermont                            803 - South Carolina

804 - Virginia (Richmond)                805 - California (Bakersfield)

806 - Texas (Amarillo)                   807 - Ontario (Thunder Bay)

808 - Hawaii                             809 - Bahamas, Bermuda, Caribbean

810 - [-- TWX: Mexico --]                812 - Indiana (Bloomington)

813 - Florida (St Petersburg, Tampa)     814 - Pennsylvania (Altoona)

815 - Illinois (Freeport, Rockford)      816 - Missouri (Kansas City)

817 - Texas (Forth Worth)                818 - California (Pasadena)

819 - Quebec (Hull)

900 - Message/Mass Call/Premium service  901 - Tennessee (Memphis)

902 - Nova Scotia, P.E.I                 903 - [Texas (from 214) eff. fall '90]

904 - Florida (Daytona Bch, Jacksonvl)   905 - [ ** formerly Mexico City ** ]

906 - Michigan (Sault Ste Marie)         907 - Alaska

908 - [New Jersey, eff. 1991]            909 - [ ** not used yet ** ]

910 - [-- TWX: Western US --]            912 - Georgia (Savannah)

913 - Kansas (Topeka)                    914 - New York (New Rochelle)

915 - Texas (El Paso)                    916 - California (Sacramento)

917 - [New York City; eff. date unknown] 918 - Oklahoma (eastern)

919 - North Carolina (eastern)

(8) Dialing Codes...

Special Area Codes

600 - Some reports mention that this area code was once reserved for

special U.S. government purposes.

700 - Special Long Distance services (usually US only)

Often, a special (700) area number will identify the customer's

primary long distance carrier in the U.S.: dial 700 555 4141 to

get the name of the long distance carrier (in some areas of the U.S.)

700 may be used for other value-added services, according to the

region and long distance carriers involved.

800 - Toll Free calling (INWATS)

900 - special services (recordings, special promotions, vote tallying,

usually with toll charge applied)

former TWX (teletypewriter) area code assignments:

410 - Northeastern USA

510 - Eastern USA (510 will become a San Francisco CA code soon)

610 - Canada

710 - Southern USA (may be reserved for U.S. government services)

810 - Mexico

910 - Western USA

Area code 708 was in service November 1989 in the Chicago area. Basically,

area 312 was split up so that the city of Chicago proper retains 312, while

the area surrounding Chicago will get area code 708.

Area 903 was once used for northwest Mexico. It will be reassigned to

Texas, to split up the Dallas 214 area as of fall 1990.

Area code 908 has been slated for New Jersey, to split up the existing

201 area. This should take place by 1991.

California will get two area code splits over the next few years. Area

310 will split the Los Angeles 213 area again (it was split not too many

years back as 213/818, with Pasadena taking the 818 side). The split should

be complete 7th October 1991.

The San Francisco area will get its 415 area split into 415 and 510 which

should be completed by 1st February 1992.

New York City, which already has 212 and 718, is expected to get the

917 area as well. Details on when this area will be added are not known

at this time.

The only other standard area codes (i.e. those with 0 or 1 in the middle

digit) are  210, 410, 710, 810, 909, 910 (assuming that the obsolete

TWX teletype service using some of the N10-type codes is taken out of

service from the standard phone system).

706 and 905 were formerly patches on the phone system to allow access to

northwestern Mexico and Mexico City respectively; these are expected to be

reassigned at some point, also, as calls to Mexico now generally use

overseas dialing (country code 52).

It is possible that there could be assignments of area codes of the form

N11 (like 211, 311, or generally anything that avoids the standard service

codes 411, 611, 911 and perhaps 811) and of the form N00 (intended for

special services like 800 or 900 calls).

(9) Dialing Codes...

Thanks to Carl Moore on the Usenet service for extra information regarding

area code splits and when they occurred/will occur:

305/904 Florida, July 1965

703/804 Virginia, June 1973

714/619 California, November 1982

713/409 Texas, March 1983

213/818 California, January 1984

212/718 New York, September 1984

303/719 Colorado, 5 March 1988

305/407 Florida, 16 April 1988

617/508 Massachusetts, 16 July 1988

312/708 Illinois, November 1989

202 District of Columbia & vicinity, 1 October 1990

202 area code has been useable for DC proper AND all but the

outermost points in Maryland and Virginia suburbs, in areas 301

and 703 respectively.  202 is being withdrawn from the suburbs

and will then be useable only for DC proper.  This change does

not affect what you have for Maryland zipcodes starting with

206,207,208,209 or for Virginia zipcodes starting with 220,221,

222,223; I do not know how government offices using zipcodes

starting with 200,202,203,204,205 and located outside DC proper

and using Md. or Va. prefixes will be affected.  The Pentagon is

physically in Virginia, but its office phones are in area code

202 only and NOT in 703.

214/903 Texas, fall 1990

201/908 New Jersey, 1991

415/510 California, 7 October 1991 (full cutover 27 January 1992)

213/310 California, 1 February 1992 (full cutover 1 May 1992)

--------

When all the area codes run out, it is proposed to use NNX codes for area

codes (starting the mid- to late-90s). Codes ending in '0' would form a

new source of area codes (like 220, 850, 990, etc). This would mean that

all long distance calls beginning with 1+ will require area codes, unless

NN0 exchange codes used are avoided within an area code (ie. a method is

devised to distinguish the area codes from the exchanges). As long as no

exchange in an area code ends in '0', the 1 + 7 digit number dialing within

an area code could still be used.

(10) Dialing Codes...

5. Country Codes for International Dialing

International dialing uses country codes from 1 to 3 digits. These are

grouped (usually) according to geographic location. Not all countries can

dial direct to all other overseas codes, but these are the codes by which

a country's telephone network is recognised.

When listing a telephone number in international correspondence, one format

often used lists the country code followed by the area code and local number

digits required to complete the call. For instance, the number (400) 012.3456

in North America would be listed as "+ 1 400 012 3456". The overseas dialing

code would be used then the digits after the "+".

Codes marked with '%' mean that this is a country not dialable from

Canada as of March 1990.

INTERNATIONAL DIALING CODES

1   - North America

Canada, USA including Puerto Rico and the Virgin Islands,

Jamaica, Barbados, Anguilla, Antigua and Barbuda, Cayman Islands,

British Virgin Islands, Bermuda, Bahamas, Dominica, Dominican

Republic, Grenada, Montserrat, St. Christopher and Nevis,

St. Lucia, St. Vincent and the Grenadines (Bequia, Mustique,

Prune (Palm) Island, Union Island)

(Trinidad and Tobago [1])

Notes:

Mexico locations with Zone 1 style area codes (706, 903, 905) are generally

out of service by now. This was only a rigged-up system to allow dialing

to Mexico City and Northwest Mexico from U.S. or Canadian phones. These

are not official (were in use within the U.S. and Canada *only*) and are

now replaced with the overseas dialing system (country code 52).

[1] Trinidad and Tobago has been assigned code 296. The date whem this

country code will take effect is unknown.

(11) Dialing Codes...

Africa

20  - Egypt               231 - Liberia %           253 - Djibouti

210 - (reserved Morocco)  232 - Sierra Leone %      254 - Kenya

211 - (reserved Morocco)  233 - Ghana               255 - Tanzania [2]

212 - Morocco             234 - Nigeria             256 - Uganda

213 - Algeria             235 - Chad %              257 - Burundi

214 - (reserved Algeria)  236 - Central African Rep 258 - Mozambique

215 - (reserved Algeria)  237 - Cameroon            259 - Zanzibar [2] %

216 - Tunisia             238 - Cape Verde          260 - Zambia

217 - (reserved Tunisia)  239 - Sao Tome & Principe 261 - Madagascar

218 - Libya               240 - Equatorial Guinea % 262 - Reunion (France)

219 - (reserved Libya)    241 - Gabonese Republic   263 - Zimbabwe

220 - Gambia              242 - Congo %             264 - Namibia

221 - Senegal             243 - Zaire               265 - Malawi

222 - Mauritania %        244 - Angola              266 - Lesotho

223 - Mali                245 - Guinea-Bissau %     267 - Botswana

224 - Guinea %            246 - Diego-Garcia %      268 - Swaziland

225 - Cote d'Ivoire       247 - Ascension           269 - Comoros & Mayotte

226 - Burkina Faso        248 - Seychelles          27  - South Africa

227 - Niger               249 - Sudan               295 - San Marino [3] %

228 - Togolese Republic   250 - Rwandese Republic   296 - Trinidad/Tobago % [4]

229 - Benin               251 - Ethiopia            297 - Aruba [5]

230 - Mauritius           252 - Somalia %           298 - Faroe Islands [6]

299 - Greenland [7]

Unknown or unassigned:

280, 281, 282, 283, 284, 285, 286, 287, 288, 289

290, 291, 292, 293, 294.

Notes:

[2] Zanzibar is routed via Tanzania as + 255 54, though the

country code 259 is assigned for Zanzibar.

[3] San Marino will be split from Italy's country code (was 39).

[4] Trinidad and Tobago are new with country code 296, moving out of

North American area code 809 (ie. + 1 809). The date when this

country code takes effect is unknown.

[5] Aruba got its own country code when it formally became independent

of the Netherlands Antilles 1st January 1986.

[6] was under Norway

[7] Greenland (country code 299) is supposed to be in the part of the world

where country codes begin with 3 or 4 (Europe). However, since all those

codes are used up, 299 is as close to 3 or 4 as possible. Country codes

beginning with 5 are also all gone.

(12) Dialing Codes...

International Dialing Country Codes (continued)

Europe

30  - Greece              355 - Albania %           41 - Switzerland,

31  - Netherlands         356 - Malta                      Liechtenstein [10]

32  - Belgium             357 - Cyprus              42  - Czechoslovakia

33  - Andorra, France,    358 - Finland             43  - Austria

Monaco [8]        359 - Bulgaria            44  - United Kingdom

34  - Spain               36  - Hungary             45  - Denmark

350 - Gibraltar           37  - East Germany        46  - Sweden

351 - Portugal            38  - Yugoslavia          47  - Norway

352 - Luxembourg          39  - Italy, San Marino,  48  - Poland

353 - Eire (Irish Rep)            Vatican City [9]  49  - West Germany

354 - Iceland             40  - Romania %

All codes beginning with 3 or 4 have been assigned at this time.

Notes:

[8]  Andorra: + 33 628, Monaco: + 33 93

[9]  San Marino: + 39 541, but moving to country code 295 at some unknown date

Vatican City: + 39 66982

[10] Liechtenstein: + 41 75

South & Central America

500 - Falkland Islands %  51  - Peru                592 - Guyana

501 - Belize              52  - Mexico              593 - Ecuador

502 - Guatemala           53  - Cuba [11]           594 - Guiana (French)

503 - El Salvador         54  - Argentina           595 - Paraguay

504 - Honduras            55  - Brazil              596 - Martinique [13]

505 - Nicaragua           56  - Chile               597 - Suriname

506 - Costa Rica          57  - Colombia            598 - Uruguay

507 - Panama              58  - Venezuela           599 - Netherlands Antilles

508 - St Pierre&Miquelon  590 - Guadeloupe [12]             Antilles [14]

509 - Haiti               591 - Bolivia

All codes beginning with 5 have been assigned at this time.

Notes:

[11] Guantanamo Bay US Naval Base: + 53 99, a special hack dialable from USA

[12] includes French Antilles: St Barthelemy, St Martin, Guadeloupe

[13] should now be onlyu Martinique; was all of French Antilles

[14] includes Saint Maarten, Saba, Statia, Curacao, Bonaire

(13) Dialing Codes...

South Pacific

60  - Malaysia            673 - Brunei Darussalm    686 - Kiribati, Gilbert Is

61  - Australia           674 - Nauru %             687 - New Caldonia

62  - Indonesia           675 - Papua New Guinea    688 - Tuvalu, Ellice Is %

63  - Philippines         676 - Tonga                        (Saipan?)

64  - New Zealand         677 - Solomon Islands     689 - French Polynesia

65  - Singapore           678 - Vanuatu [16]        690 - Tokelan

66  - Thailand            679 - Fiji Islands        691 - F.S. of Micronesia

670 - Marianna Isl. [15]  680 - Palau %             692 - Marshall Islands

671 - Guam                681 - Wallis & Fortuna %

672 - Austr. Extrn. Ter.: 682 - Cook Islands

Christmas Island,   683 - Niue Island %

Cocos Islands,      684 - American Samoa

Norfolk Island      685 - Western Samoa

Unknown or unused: 693, 694, 695, 696, 697, 698, 699

Notes:

[15] that is, Northern Mariana Islands, or Saipan

[16] or New Hebrides

Soviet Union

7   - USSR

Note:

It may not be possible to dial that many places within the Soviet Union,

even where direct dialing facilities are available (for instance, calling

direct is perhaps restricted to Moscow and maybe a few other cities).

(14) Dialing Codes...

International Dialing Country Codes (continued)

East Asia (plus Inmarsat/Marisat)

81  - Japan               855 - Cambodia            878 - (reserved, national

82  - South Korea         856 - Laos                       mobile purposes)

84  - Vietnam %           86  - China               879 - (reserved, national

850 - North Korea         871 - Inmarsat (Atlantic)        mobile purposes)

852 - Hong Kong           872 - Inmarsat (Pacific)  880 - Bangladesh [17]

853 - Macao               873 - Inmarsat (Indian)   886 - Taiwan [17]

Unknown or unused: 800, 801, 802, 803, 804, 805, 806, 807, 808, 809,

830, 831, 832, 833, 834, 835, 836, 837, 838, 839,

851, 854, 857, 858, 859,

881, 882, 883, 884, 885, 887, 888, 889,

890, 891, 892, 893, 894, 895, 896, 897, 898, 899

Unknown or unused, but reserved for Maritime Mobile service:

870, 874, 875, 876, 877

Notes:

[17] By CCITT policy, no more country codes of the 880-889 series are

supposed to be assigned until all other country codes beginning

with 8 have been taken. 886 for Taiwan is an unofficial code used

for access to Taiwan. The "People's" Republic of China, on the

other hand, has assigned Taiwan access via country code 866

(presumably + 86 6, through China).

Middle East

90  - Turkey              962 - Jordan              971 - United Arab Emirates

91  - India               963 - Syria               972 - Israel

92  - Pakistan            964 - Iraq                973 - Bahrain

93  - Afghanistan %       965 - Kuwait              974 - Qatar

94  - Sri Lanka           966 - Saudi Arabia        976 - Mongolia %

95  - Myanmar (Burma)     967 - Yemen Arab Rep      977 - Nepal

960 - Maldives            968 - Oman                98  - Iran

961 - Lebanon             969 - Yemen Dem Rep %

Unused or Unknown: 970, 975, 978, 979,

990, 991, 992, 993, 994, 995, 996, 997, 998, 999

No country codes begin with '0'.

(continued...)

(15) Dialing Codes...

There would be 195 assigned country codes, with 8 codes reserved (210, 211,

214, 215, 217, 219, 878, 879). 79 more standard country codes can be assigned

(assuming that 2 digit codes aren't split up into 3 digit codes, etc), not

including the 5 unassigned/unknown mobile/marisat codes in the 87x series.

25 country codes are not dialable from Canada as of March 1990 (including

any country codes yet to be placed into service like Trinidad & Tobago).

The most recent to be in service from Canada are 233 Ghana, 253 Djibouti,

258 Mozambique, 264 Namibia, and 95 Myanmar (Burma). There are signs that

296 Trinidad and Tobago may be placed into service soon from Canada. From

a payphone, it would cost $16.30 for a 3-minute call to Burma from Canada.

Ghana is a relative bargain at $7.95 for 3 minutes. Don't even think of

trying to use coins for an Inmarsat call (likely $36 or more for the three

minutes).

Thanks go to those who discussed the country codes on Telecom Digest,

a service avaialable through Usenet and other places. In particular,

information from John Covert, Mike Grant, Dolf Grunbauer, John Levine,

Chris Johnston and undoubtedly others of this newsgroup has been helpful

in the updating of this list.