Tech Notes

This feature of Tel Codes provides a reference for some of the features

and standards of the telephone system.

Signalling and Hook Conditions

In dial-pulsed signalling, the phone line is taken on and off hook rapidly

to indicate which digit is being dialed. Two pulses (or times the line is

taken on hook) means a '2' was dialed. Ten pulses means a '0' was dialed.

The pulses are sent at 10 pulses per second (with a tolerance of 10% allowed

on either side). A time of 0.6 seconds minimum must be allowed between

digits. The time of the pulse (that is, how long the line should be taken

on-hook for a pulse) should be between 0.058 and 0.064 seconds (that is,

a 58% to 64% break).

Various Tones

Dial Tone:              350 and 440 Hz

Busy:                   480 and 620 Hz, pulsed at 1 per second

Circuit busy, or error: 480 and 620 Hz, pulsed at 2 per second

Ringing:                440 and 480 Hz for 2 seconds, then 4 seconds silence

for each ring; each ring generally takes 6 seconds

unless calling another country or using a PABX system

(These frequencies may not necessarily be used on certain exchanges, and

should be considered a North American standard only).

Touch Tone Frequencies

A common touch tone keypad will send out two frequencies when a key

is pressed. One frequency is taken from a "low group" and the other

frequency is taken from the "high group". These frequencies are:

<--------- High Group --------->

1209 Hz 1336 Hz 1477 Hz 1633 Hz

L           +-------+-------+-------+-------+

o   697 Hz  |   1   |   2   |   3   |   A   |

w           +-------+-------+-------+-------+

770 Hz  |   4   |   5   |   6   |   B   |

G           +-------+-------+-------+-------+

r   852 Hz  |   7   |   8   |   9   |   C   |

o           +-------+-------+-------+-------+

u   941 Hz  |   *   |   0   |   #   |   D   |

p           +-------+-------+-------+-------+

For example, when a 5 is pressed on a tone keypad, the frequencies 770 Hz

and 1336 Hz will be heard.

The tolerance for frequencies should be within 1.5% of the actual value.

A minimum of 0.05 seconds of tone is standard for each digit (though

some exchanges will pick up the touch tones consistently for 0.037 seconds

of tone). The time between each digit should be at least 0.045 seconds.

Trunk Frequencies

Here are frequencies used in the trunks between telephone exchanges and

toll centres... these were the tones used in blue boxes during the

telephone piracy years (and can now be detected, making blue boxing

all but impossible), but these are used by Bell when dialing from

exchange to exchange (or 'signalling'). You may sometimes hear these

just after dialing a call and before the ringing begins.

Two tones are selected from the following frequencies: 700, 900, 1100,

1300, 1500 and 1700. The codes are:

KP (start)   1100 and 1700

1            700  and 900   ('0' + '1' = '1')

2            700  and 1100  ('0' + '2' = '2')

3            900  and 1100  ('1' + '2' = '3')

4            700  and 1300  ('0' + '4' = '4')

5            900  and 1300  ('1' + '4' = '5')

6            1100 and 1300  ('2' + '4' = '6')

7            700  and 1500  ('0' + '7' = '7')

8            900  and 1500  ('1' + '7' = '8')

9            1100 and 1500  ('2' + '7' = '9')

0            1300 and 1500  ('4' + '7' = '0' ???)

ST (end)     1500 and 1700

plus, 700 & 1700, 900 & 1700, 1300 & 1700 are spare tone pairs which may

be used for various purposes.

Note that if we consider 700 as a '0' tone, 900 as a '1' tone, etc. to

1500 as a '7' tone, you can notice how these represent the digits dialed.

I think the 1700 Hz tone was referred to as a '10' tone as well.

2650 Hz is used as a disconnection tone on a trunk. During a long distance

call, it's possible to hear this tone momentarily when a call is completed.

To intercept blue boxes (ie. subscriber-generated multifrequency tones),

all Bell has to do is install detectors for these tones. If they come

from Bell's exchange equipment, fine. If they come from an ordinary

telephone, there's a problem.

These frequencies represent the 'R1' signalling scheme, and is public

knowledge. I also glanced at the 'R2' signalling scheme, which appears to

be hairier than 'R1' but with more features.

Now, there's a development called common channel interface signalling

which means the digits are sent through another channel, outside of the

lines that will be carrying conversations. Thus, no blue box other than

any sabotage at the telephone exchange centre will be able to affect the

line. Recently, New Jersey has offered services based on the CCS 7 system

which allows new features such as automatic number identification (ANI)

of those numbers calling your telephone. The feature will be generally

referred to as "Caller ID".

Recordings

Many recorded messages that indicate a failure to complete a call come

with a set of three tones. These "Special Information Tones" are set up

with varying frequencies and durations as follows:

Duration (ms)       Tone Frequencies (Hz)

------------------  -----------------------

reorder        274 / 380 / 380     913.8 / 1428.5 / 1776.7

not in service 380 / 274 / 380     985.2 / 1370.6 / 1776.7

no circuit     380 / 380 / 380     985.2 / 1428.5 / 1776.7

intercept      274 / 274 / 380     913.8 / 1370.6 / 1776.7

reorder *      274 / 380 / 380     985.2 / 1370.6 / 1776.7

no circuit *   380 / 380 / 380     913.8 / 1370.6 / 1776.7

other problem  380 / 274 / 380     913.8 / 1428.5 / 1776.7

future use     274 / 274 / 380     985.2 / 1428.5 / 1776.7

* - used in the U.S. when a recording regards an out-of-region connection.