Tel Codes - A TTY/TDD history, courtesy Telecom Digest (13 March 1990)

From: CER2520@ritvax.bitnet (Curtis E. Reid)

Subject: Brief TDD History

Date: 15 Feb 90 00:25:00 GMT

Sender: news@accuvax.nwu.edu

Organization: TELECOM Digest

Telephone Devices for Deaf Users: Milestones

Source: _1990 International Telephone Directory for TDD Users_ by

Telecommunications for the Deaf, Inc., 814 Thayer Avenue, Silver Spring, MD

20910, Copyright 1989.

1964:

An acoustic coupler modified for Baudot which permitted

teletypewriters to be connected to standard telephones, was invented

by Robert Weitbrecht, a deaf scientist.  Dr. James C. Marsters, a deaf

orthodontist, had sent Weitbrecht a Teletype Model 32ASR with a

request that a system be set up so that Marsters, in Pasadena,

California, could communicate with Weitbrecht in Redwood City.  Unlike

Weitbrecht, Marsters did not have a ham radio license, so they decided

to use the public phone system.

Weitbrecht's acoustic coupler was based on a half duplex operation in

which signals are transmitted one at a time.  Teletypewriters were

Baudot coded.  These machines were available because AT&T, Western

Union and other communications companies were in the process of

conversion to a new telegraphic code.

Weitbrecht's acoustic coupler was first publicly demonstrated at the

1964 biennial meeting of the Alexander Graham Bell Association for the

Deaf, Salt Lake City, Utah.

1968:

Approximately 25 teletypewriter stations were in operation for/by the deaf.

1969:

Tax-exempt status was granted to TDI [Telecommunications for the Deaf,

Inc.] by the U.S. Internal Revenue Service.

1971:

"Scanatype", a digital readout form of the TTY was introduced by ESSCO

Communications.

1979:

Telecommunication Device for the Deaf (TDD) was accepted as the

appellation for all TTY-like devices.

1985:

Krown Research introduced dual ASCII/Baudot modems to be used with

dumb terminals.  Audiobionics, Inc. introduced an enhanced version of

a synthesized voice "talking" portable TDD.  Ultratec, Inc. introduced

a low-cost basic TDD.  Other manufacturers discontinued production of

TDDs as competition for lower cost TDDs increased.  Nevada Association

for the Deaf hosted the Sixth TDI Convention.  Distribution of free

TDDs began in many more states such as Florida, Arizona, Nevada,

Wisconsin, Illinois and Massachusetts.

1987:

California opened first state-wide, 24 hour, 7 day a week relay

service operated by AT&T.

1988:

President Ronald Reagan signed the bill to expand federal

telecommunication services for speech and hearing-impaired people;

which was introduced by Representative Steve Gunderson (R-WI) and

Senator John McCain (R-AZ).

-=-=-=-=-=-=-=-=-=-=-=

From: CER2520@ritvax.bitnet (Curtis E. Reid)

Subject: Common TDD Terms

Date: 15 Feb 90 00:26:00 GMT

Sender: news@accuvax.nwu.edu

Organization: TELECOM Digest

Common TDD/TTY Terms

TTY  Teletypewriter

TDD  Telecommunication Device for the Deaf

GA   Go Ahead, your turn to type

SK   Stop Key, meaning end of conversation

CUZ  Because

HD   Hold, please

PLS  Please

OIC  Oh, I see

U    You

UR   Your

CD   Could

Q    Question mark (Also ?)

MTG  Meeting

R    Are

NBR  Number (Also NU or NO)

OPR  Operator

CUL  See you later

SHD  Should

WLD  Would

TMW  Tommorrow (Also TOM)

*  To be effective, punctuations are not used during TDD

conversations.

*  Please identify yourself while answering TDD calls as it is

rather difficult to recognize one's voice through printed

letters.

*  Tap the space bar several times for numbers that use both TDD

and Voice.

-=-=-=-=-=-=

From: CER2520@ritvax.bitnet (Curtis E. Reid)

Subject: TDD Specifications

Date: 15 Feb 90 00:25:00 GMT

Sender: news@accuvax.nwu.edu

Organization: TELECOM Digest

Specifications For TDD Data Transmit/Receive

All TDD manufacturers follow these specs but no tolerance level is given:

1.  Acoustic coupling (standard G- and K- type handset)

Input level: 65 dB SPL minimum / 110 db SPL maximum

-4        2

(0 dB SPL = 2 x 10   dyne/cm )

Output level: 95 dB SPL minimum / 100 dB SPL maximum

(acoustic pressure to produce a -9 dBm

signal at the telephone set line terminals

on an average loop)

2.  Direct modular connection

Connector:  standard 6-position modular jack

USOC No.:   RJ11C

Input level: -50 dBm minimum / 0 dBm maximum

Output level: -13 dBm minimum / -9 dBm maximum

3.  Modem

Modulation method:  Frequency Shift Keying

Mark frequency:  1400 Hz +-5 Hz

Space frequency:  1800 Hz +-5 Hz

Signal to noise ratio:  13 dB

Carrier to data delay:  180 ms

Carrier to hold time:  156 ms

4.  Protocol

Code:  Baudot

Mode:  Half duplex

Transmissions:  Asynchronous

Start bit:  1

Stop bit:   1

Data bits:  5

Parity:  None

Data rate:  45.5 baud (buffered)

Note: The above information is from Plantronics, Inc. (manufacturer of

the Vu-Phone) and is assumed to be consistent with other brands as the

Vu-Phone is compatible with all other TDDUs.