01

A     -  Select Auto Log Feature.

The  'A'  command selects the Auto Log  feature  which

provides  a  means  to make a hard copy  record  of  a

simulation run.   The 'AE' commands enables the  auto-

log feature and allows selection of the destination of

the log output.  'S' selects the display screen on the

numeric display screen,  'D' selects a disk file named

'HISTORY.DAT',  and  'P' selects the printer connected

to  'LPT1:'.   Auto-Log reocrds every command  isssued

and any danger warnings issued by the  reactor.   This

feature  can  be  disabled by the 'AD'  option  or  by

pressing 'A' and 'Return'.

02

B     -  Set Bias value.

The  'B' command allows the setting of the bias  value

used  by the automatic control system to maintain  the

heat exchanger temperature near the set point tempera-

ture.   This  value may range from 1 to 5.   1 is  the

most sensitive setting and should be used whenever the

reactor is operating close to its' limits.  This value

may only be changed when automatic control is on.  The

set  point  temperature is controlled by the 'C'  com-

mand.

03

C     -  Set Automatic Control Temperature.

This command sets then heat exchanger temperature that

will  be  maintained by the automatic control  control

system {F5}.   The value can be set anywhere from 350F

to  505F.   The automatic control program will  adjust

rod height to maintain the heat exchanger  temperature

within  the  Bias setting of the control  temperature.

This  temperature  will be automatically  adjusted  to

meet power demand when Automatic Set-point {F6} is on.

This value can only be changed when automatic  control

is on.

04

D     -  Make-up Feed Destination.

This  key allows selection of the destination for  the

make-up feed flow.  'P' causes make-up feed to be sent

to the primary loop, 'S' causes feed to be sent to the

secondary  loop,  and 'N' causes feed to be  recircul-

ated.   The  make-up feed pump should be operating for

flow  to go to the primary or  secondary  loops.   The

secondary  loop will drain if 'S' is selected and  the

make-up feed pump is off.

05

E     -  Emergency Coolant Pump Speed.

This command selects the emergency cooling pump speed.

The pump can either be off, speed 0, or on, speed 1 to

100%.   This  pump is used to provide rapid cooling to

the  reactor core.   The emergency cooling  supply  is

limited, but is adequate to lower the core temperature

to safe levels.

06

F     -  Freeze and Save Simulation Run.

This  command  stops the simulation run and saves  the

pertinent variables to a file named 'RXFRZ.DAT'.  When

the simulation is started at a later time,  the opera-

tor  is asked whether the saved simulation  should  be

executed.   A 'Y'es answer will load the variables and

restart  the  simulation at the point it  was  frozen.

Only one simulation run can be frozen at any time.

07

G     -  Current Production Graph

This command graphs the electrical output against  the

demand.   The  graph  shows  the most recent  10  hour

period.

08

09

M     -  Make-up Feed Pump Speed.

This  command  selects the speed of the  make-up  feed

pump.  The pump can be off, speed 0, or on, speed 1 to

100%.  This pump is used to provide additional coolant

to the primary and secondary cooling systems.   As the

reactor  heats up,  primary coolant must be drained to

maintain  the primary volume within  limits.   As  the

reactor  cools down,  coolant must be added to make up

for the drained coolant.   The secondary system  loses

coolant through gland sealing steam and minor leakage.

This pump should be on at all times, running at 50% or

less.   In  this recirculation mode,  make-up feed  is

instantly available.

10

P     -  Primary Coolant Pump Speed.

This  command  selects the pump speed of  the  primary

coolant  pump.   This  pump circulates  high  pressure

water through the reactor core and transfers heat from

the reactor to the heat exchanger.  This pump normally

operates at 90% or above.  Both the primary and secon-

dary coolant pumps must both be off to completely shut

down the reactor.

11

Q     -  Alarm and Warning Test

This command tests the alarm and warning indicators on

the  console.   Pressing this key a second time resets

the  alarm and warning indications and shows only  the

active alarms.

12

R     -  Rod Height.

This  command  selects the control  rod  height.   The

control   rods   provide  direct  control   over   the

reactivity of the core, and thereby control the core's

temperature.   The  higher the rods,  then the core is

more reactive,  and more heat is produced.  The opera-

ting  range of the rods is between 0,  on the  bottom,

and 100%,  completely withdrawn.   The two SCRAM  com-

mands {ESC  and  F1} cause the rods  to  be  instantly

dropped to the bottom.

13

S     -  Secondary Coolant Pump Speed.

This  command selects the speed of the secondary cool-

ant  pump.   This pump circulates the condensed  steam

from the cooling tower/condenser to the heat exchanger

where  it again produces steam.   This  pump  normally

operates  at  90% or above.   Both this pump  and  the

primary  coolant  pump must be off to completely  shut

the reactor down.

14

T     -  Terminate Simulation

This command terminates the simulation and  calculates

a  score based on the production rate,  damages incur-

red,  and production deficit.   The score is  adjusted

for  the length of the simulation run.   A  cumulative

score   is  maintained  in  the  file  'RXSTATUS.DAT'.

Erasing this will clear the cumulative score.

15

U     -  Disable Alarm

This  command switched the audible alarm system on and

off.

16

V     -  Current Version

This  command displays the current version  number  of

the  Reactor-100 main program.   Please refer to  this

number when reporting any problems with the program.

17

Z     -  Change Timer Setting

This command allows changes to the time interval  bet-

ween  screen updates.   The allowable intervals are  0

(no  delay),  or 3 to 10 seconds.   The normal  update

interval is once every 2.5 seconds.

18

'H'   -  Provides information.

Help  is available by using the help key or using  the

lightpen.   The  help menu can be used by entering the

number next to the subject of interest.   The lightpen

can be used to select Help on any item of interest  by

pointing  the lightpen at the subject and pressing the

switch.   The  lightpen can also be used on  the  Help

menus.

19

Tab   -  Bypass Sign-on Message

This command can be used during the program sign-on to

bypass  the delay.   This will also bypass the initial

Help screen.

99

#####################################################################################################################################################################################################################