Documentation for HDTEST V4.45          Copyright 1988, P. R. Fletcher          _______________________________        _______________________________

USER-SUPPORTED SOFTWARE                                  _______________________

Much of the software which is available for your computer seems to

have been produced and sold on the assumption that you are a crook, and

that you will take any available opportunity to cheat its creators and

vendors out of their rightful reward. Such software is copy-protected

within an inch of its (and your computer's) life, and is still priced

at a level which guarantees its producers a profit even if four or

five illegal copies are made for each one sold. User-supported

software, increasingly known as "Shareware" is different. You may have

got a copy of this package free from a friend or a bulletin board or

(for a nominal fee) from a users' group or software library. In any

case, you have in your possession a perfectly legal copy of the

program. You are further encouraged to make as many copies of the

package as you like and to distribute them to all your friends and

acquaintances. However, if, after trying the program out, you find                         ________

that it is useful and want to keep it and continue using it, you must                                                                           ____

register your copy by sending the registration fee ($35.00,) together

with a completed registration form, to the address on the form. If you

continue to use the program after an appropriate trial period (and the

"Shareware" system lets you determine what is appropriate), you are                                  ___

breaking both the terms of the license under which you have received

the software and the law. You may believe that the chances of the law

of Copyright being enforced against you are very low (and you may even

be right!), and the distributors of "user-supported" software do, in

fact, usually simply have to trust you to be honest and willing to pay

a reasonable price for a useful program. However, you should bear in

mind that unsupported "Shareware" software is likely either to cease

to exist altogether or to go commercial. The former eventuality

benefits nobody - the latter only the "middlemen". As an additional

inducement, Registered users will get a copy of the latest version of

HDTEST, which already (at the time of writing) includes a number of

significant enhancements over V4.45, and they will be entitled to

support and help with HDTEST-related problems (including telephone

support) for a year from the date of Registration. Additional features

in V4.71 of HDTEST included support for a still wider variety of

computer systems and storage media, full DOS 4.0 support, and the

removal of the ANSI.SYS requirement for the screen display to work

properly.

Documentation for HDTEST V4.45          Copyright 1988, P. R. Fletcher          _______________________________        _______________________________

INTRODUCTION                                       ____________

HDTEST was originally written to allow comprehensive Read/Write

testing of hard disks on IBM PCs, XTs, ATs, and "true" compatibles.

The V4.10 release relaxed many of the hardware restrictions imposed by

earlier (V2.xx and V3.xx) versions of the program and made it useful

for testing most MS-DOS-compatible storage devices currently in use on

IBM-compatible computers, including removable (floppy and hard) disks

and logically partitioned hard disks. This release (V4.45) includes

BIOS-level support for DOS 3.3 "extended devices" and more options for

the handling of totally unreadable sectors, and recognizes (but does

not support) large (> 32 Mb) devices under IBM DOS 4.0 and Compaq DOS

3.31. Although many programs, including IBM's diagnostics, allow

"surface scanning" and seek testing of hard disks, none that are

available to the average user test whether the whole disk can retain

the data that is or will be written to it. HDTEST's principal "claim

to fame" is the ability thoroughly to test the magnetic media in a

device which contains data while preserving the data (although data                                     _________________________

loss is unavoidable if an area which is in use has become unreadable).                                                                      ____

HDTEST tests the disk much more thoroughly than programs such as

DFixer, Mace, and Norton's DT. Mace and DT, like many public domain

"disk test" programs, simply read every cluster on the disk and report

on any errors encountered.  DFixer does better, writing 4 test

patterns to every free cluster and checking that they can be read back

correctly, but it makes no attempt to test clusters that are already

in use - the very ones which contain your precious data!

In addition to its testing function, HDTEST has been reported to be

useful as an automatic disk repair utility for both hard and floppy

disks which have developed (especially) intermittent or persistent

read errors, preventing or impeding their normal use. Although HDTEST

cannot read the totally unreadable, it tries much harder that DOS does

and will almost always, in the course of its operation, allow you to

recover the entire readable portion(s) of a file with one or more                             ________

unreadable clusters in it, optionally either remapping all of what it

reads from the disk or replacing bad sectors (the default) or clusters

with NULLs. HDTEST is not an alternative to backing up your floppies                                ___

and/or hard disk - it may sometimes be able to save your neck when you                                    _________

forget or your backup goes bad too! On hard disks which have developed

intermittent read errors, running HDTEST can sometimes totally cure                                                   ___

the problem, presumably because the "flaky" sectors get rewritten and

the minor misalignments and/or weakening of the magnetic "image" which

were causing the trouble are corrected when the data is rewritten.

In its normal ("slow") mode, HDTEST writes 20 different test patterns

to every cluster on the disk and checks that each pattern can be read             _____

back correctly, while preserving (and eventually restoring) the

contents of the clusters that contain data.  HDTEST can even (on many

devices) detect hard disk errors which are minor enough to be

correctable (and corrected) by the disk controller's own "error-

correcting code" (ECC).  These errors are normally totally invisible

to programs running under DOS and will not usually be picked up by

other disk test utilities.  They may, however, be early signs of

problems that will become significant later, and HDTEST consequently

marks these clusters, too, as bad. As long as HDTEST can read the data

from a flaky cluster correctly once (and it tries quite hard) it will

Page 2

Documentation for HDTEST V4.45          Copyright 1988, P. R. Fletcher          _______________________________        _______________________________

be able to save the file of which that cluster is a part intact, while                                                                   ______

marking the cluster so that DOS does not try to use it again. The

program was designed to minimize the likelihood of user error causing

data loss on a good (or slightly bad) disk, and to be easy to use - it

could not be made very fast, since multiple bit patterns have to be

written to and read from each cluster, and this takes a significant

time even on a fast hard disk. This version does, however, have a less

sensitive "fast" test mode, which writes only two test patterns to

each block and is recommended for use when testing/repairing floppy

disks and/or to correct intermittent problems or rule out gross

problems on a hard disk.

The program can be used to test/repair single and double-sided 8- and

9-sector minifloppies (if you have plenty of patience!), as well as                                             ______

AT-format 1.2Mb minifloppies, 8" standard floppies, and 3.5"

microfloppies (with the same proviso regarding a need for patience).

HDTEST's companion program, HDCHEK, simply examines the logical

structure of the system and target device and gives a short report on

what it finds. It does not write to the disk, and is provided                                 ___

principally to allow a quick and easy, but fairly reliable, check for

compatibility between HDTEST and a given software/hardware

configuration. As part of its report, HDCHEK notes at what level (DOS

interrupts only, INT 13, or direct entry to the BIOS) it was able to

access the target device, which has a bearing on HDTEST's ability to

detect certain types of errors. In general, if HDCHEK runs                                          __________

successfully on a system and target device, then HDTEST can be

expected to do so, although the latter program does require more

memory.

The HDTEST package is distributed as "User-supported Software". If you

intend to continue using the program, you are required to become a

Registered user. Send a completed copy of the Registration Form (which

is included in this documentation), together with a check or money

order for $35.00 (+ $5.00 for shipping outside the continent of North

America) either to me at the address which appears on the form or (for

European users) to the address displayed on the European version of

the program's copyright screen. Checks and money orders in

denominations other than U.S. dollars are welcome, but please add a

further $5.00 for bank charges and use the current exchange rate to

calclate the fee if you are paying with such a check or money order.

Please also add $2.50 (or equivalent) if you want the software on a

1.2 Mb HD floppy or a 3.5" minifloppy, rather than a 360 Kb DSDD

floppy, and specify which you want on the Reg. form. I will send you a

personalised copy of the latest, considerably enhanced, version of the

HDTEST package (currently V4.71), and register you for a year's

support. Note that the Registered version of the package will be

shipped from the United States (by Airmail, for non-U.S. users), even

if you use the European address, but European Registrations sent to

that address are forwarded by computer network, and will usually reach

me faster than if they were mailed to me in the U.S.A. Registration

entitles you to be notified of upgrades, bug fixes, newly discovered

incompatibilities, etc., and to ask for (and get) necessary help in

adapting the program to work on your hardware. I cannot guarantee to

make HDTEST work with every possible combination of hardware and

software, but I will promise to try to solve problems as they come up,

and I will return your registration fee if I cannot support your

Page 3

Documentation for HDTEST V4.45          Copyright 1988, P. R. Fletcher          _______________________________        _______________________________

system.  Support will be available TO REGISTERED USERS ONLY by mail,                                             ________________________

BIXmail, Compuserve EMail and telephone. Please also bear in mind that

my willingness and ability to continue to support and enhance these

programs will be directly dependent on users' willingness to pay the

registration fee and (to some extent) inversely dependent on the

number of questions I get that could be answered by reading the docu-

mentation(!).

The HD programs were written principally in C, compiled by the

Microsoft C compiler (Version 5.10) using its "small" memory model,

and linked with code from the distributed Microsoft C object libraries

using the Microsoft LINK program (Version 3.65).  Additional assembly

language routines were assembled by the Microsoft MASM Macro Assembler

(Version 5.10). Portions of the distributed program code are

consequently Copyright by Microsoft Corp., 1985 - 1988. All rights

reserved. These portions are used under the terms of a license from

Microsoft Corp.

The HD programs were written with care and have been tested on a

number of different systems.  I cannot, however, give any guarantee as

to their performance on a specific system, nor can I accept liability

for any actual or consequential loss resulting from their use. The

programs may only be used and/or copied in their original form.

Unauthorised modifications and/or the use of all or part of the

programs in other software, commercial or otherwise, are strictly

prohibited.

Page 4

Documentation for HDTEST V4.45          Copyright 1988, P. R. Fletcher          _______________________________        _______________________________

GETTING STARTED                                      _______________

This section gives step by step instructions for running HDCHEK and

HDTEST on the C: drive of a standard IBM PC, XT, or 100% compatible.

Users of ATs or AT-compatibles may safely follow them too, but should

really read the description of (and use) the /A command line flag.

Subsequent sections also describe some of the program's other features

and options in more detail, and should help to explain and let you

deal with any problems which you may encounter. Note that you should

never attempt to run HDTEST under a multi-tasking or multi-user          _____

operating system, nor activate any "pop-up" program (e.g. SideKick)

while it is running.

1) Use the DISKCOPY program to make a duplicate of the distribution

disk, put the original away in a safe place, and use the duplicate as

your working disk.

2) Make sure that you have current backups of all the files on drive

C:. This is particularly important when running the program for the

first time on a new system.

3) Run DOS's CHKDSK program on drive C: and correct any errors

reported by it before proceeding further.

4) Make the drive and/or directory which contains HDCHEK.EXE and

HDTEST.EXE your current drive and/or directory.

5) Insert a formatted, non-write-protected floppy disk in drive A.

HDTEST will write an ASCII log file called HDTEST.LOG to this disk -

any other files on it will be undisturbed.

6) Give the command: "HDCHEK C:". After a certain amount of disk

activity has taken place, and certainly within a minute or two, your

screen should look like one of the examples given in Appendix C. If

any errors are reported, DO NOT PROCEED until you have read the rest                                   ______________

of the documentation and/or are sure you know what is going on.

7) If HDCHEK reported no errors, give the command "HDTEST C: A:".             ____________________________

After some disk activity, the program will prompt for replies to three

or four questions - press <Return> in response to each prompt. The

program will report its progress to the screen and will record any

problems encountered in the log file on the floppy disk. Depending on

the size of your DOS partition and the access time of the disk, a

complete run of HDTEST (in its normal, "slow", mode) will take between

2 and 5 hours. Your intervention will only be required if a recovered                  _____

bad cluster is found in a "hidden", "system", or write-protected file,

when you will be asked to approve remapping; otherwise, bad clusters

will be automatically remapped. HDTEST can be aborted at any time by

pressing <Esc> and responding to the "Are you sure...." prompt by

pressing <Return>.

8) If HDTEST finishes testing without finding any bad clusters, it

will simply record that fact and the finishing time in the log file

and return to the DOS prompt. Otherwise, when read/write testing has

Page 5

Documentation for HDTEST V4.45          Copyright 1988, P. R. Fletcher          _______________________________        _______________________________

been completed, you will be prompted for permission to complete the

remapping process by updating the disk FATs and Directories.  Pressing

<Return> will allow this to take place, and you will then be returned

to the DOS prompt.

9) N.B. If HDTEST reported and remapped any bad clusters and you are             ____                                                  ___

using either the DOS (3.3 and above) FASTOPEN utility, or any other

resident directory- or file-caching software, you should reboot your

system before proceeding with other work on it.

10) If HDTEST reported and remapped some bad clusters, you should also

read the section on "Dealing with bad clusters" later in this

documentation.

Page 6

Documentation for HDTEST V4.45          Copyright 1988, P. R. Fletcher          _______________________________        _______________________________

HARDWARE & SOFTWARE REQUIREMENTS                             ________________________________

HDTEST and HDCHEK make a number of assumptions about the environment

in which they are operating. They will, generally speaking, abort with

an informative error message if they find one of these assumptions to

be invalid! The programs require MS/PC-DOS 2.x or above and (for

HDTEST) up to about 256 Kb of free memory (the exact amount required

depends on the size and other characteristics of the device being

tested) to run - the presence of more memory may (slightly) increase

HDTEST's speed when testing devices with large cluster sizes. The

device to be tested must be usable as a normal DOS device - commands

such as DIR, COPY, and CHKDSK should be valid for it and for files and

subdirectories on it. It need not, however, be bootable. Only the DOS

partition of a multi-operating-system disk will be tested, but all the

partitions of a disk that is divided into multiple DOS partitions can                                                             ___

be tested (one by one). This version of HDTEST fully supports DOS 3.3

extended partitions, and will test them as thoroughly as it does

normal DOS partitions. HDTEST's support for other disk partitioning

software is provided only to the extent that DOS support is provided

by the authors of the partitioning software.

If you wish to test a new hard disk which is to be configured to                         ____

support multiple operating systems, you should first configure it as a

single DOS partition, then test it in that configuration, and finally

reconfigure it to the form in which you are going to use it. Note          __

that, under these circumstances, HDTEST can only be used as a test -                                                      ____              ____

when you repartition the disk, any remapping and/or marking of bad

clusters which HDTEST may have done is lost, unless your partitioning

software also allows you to specify bad sectors to be locked out.

Ideally, for normal hard disks, the BIOS handler for interrupt 13

should be capable of handling (at least) normal disk reads and writes

(sub-functions 2 and 3) in a way that is compatible with that

documented for the XT disk adapter in IBM's technical documentation.

This should be no problem for most disk systems on IBM PCs and most

compatibles.  HDTEST may, however, be forced to use the (somewhat more

limiting) DOS device interface when used with non-bootable hard disks

and/or with hard disks which require special device drivers to be

installed, and will be forced to use the DOS device interface when

dealing with disks partitioned by means other that the DOS 3.3

utilities. Earlier V4.xx releases of HDCHEK and HDTEST would abort if

run on small (<= 10 Mb) hard disks which had been patched to use a

cluster size smaller than the DOS standard (and 16-bit FATs). This

deficiency has been corrected in the current version, but very small

(less than 1024 clusters) partitions may still cause problems if 16-

bit FATs are used (there is no good reason why such partitions should

use 16-bit FATs).

If run under DOS 3.0 or above on an IBM INT 13 compatible hard disk,

HDTEST attempts to locate and use the ROM BIOS INT 13 entry point,

rather than the normal INT 13 vector (which DOS 3.x intercepts and

interferes with). By default, unless the /H option is used to force a

different segment address, or the /A, /D or /I flag is set, the

program may assume that, if a ROM is present at absolute address                                   ___________________

C8000H, it contains the hard disk BIOS and has initialization code                                                 ___

starting at offset 3. This assumption is true for all hard disks which

Page 7

Documentation for HDTEST V4.45          Copyright 1988, P. R. Fletcher          _______________________________        _______________________________

"plug in" to the IBM PC and XT. It may not be true for all clones

which come with hard disks installed, although it is likely to be true

for most of them. The AT and most of its clones have no ROM at all at

this location, which is OK, too, even if you forget to use the /A flag

(but see para 5 in the "Limitations and Restrictions" section).

Problems (up to and including system halts) may occur if none of the

above flags is set and there is a ROM at C8000H but it does not                                       __                             ___

control the hard disk. They are almost guaranteed under these

circumstances if the initialization code does not start at offset 3 in

the ROM. If you know that your hard disk ROM is located somewhere

other than at the default segment address, you can use the /H flag to

tell the programs where to look for it. If you have other problems

running HDCHEK and HDTEST on non-standard systems, try running the

programs with the /I flag (for PCs and XTs) or the /A flag (for ATs),

or with the /D flag (for any hardware), and/or with the /C flag. All

these command line flags are discussed in more detail below.

Note that, since HDTEST remaps bad clusters to free space, there is

not much point in running it on a disk which has none (as shown by DIR

or CHKDSK), since the program will abort immediately if it finds a bad

cluster.  Unless you have a really flaky disk, however, you should not

need more than a couple of hundred Kb free to remap any reasonably

probable number of bad clusters.

ANSI.SYS or another ANSI console device driver should be installed on

those systems (including the IBM family) which do not support ANSI

screen I/O in Video BIOS - nothing terrible will happen if ANSI.SYS is

not installed, but the screen display will be very strange (!!).

HDTEST has been tested with the Public Domain NANSI driver, as well as

with FANSI-CONSOLE (tm).

Page 8

Documentation for HDTEST V4.45          Copyright 1988, P. R. Fletcher          _______________________________        _______________________________

LIMITATIONS AND RESTRICTIONS                               ____________________________

The current version of HDTEST has a few limitations, most of which

result from the way data is stored internally by the program. None of

them are "etched in stone", although some could only be avoided at the

cost of slowing down what is already far from a fast process. The

known limitations are as follows:

1) Maximum number of clusters per partition = 24,000 (the number is

approximate, because it depends on dynamic memory allocation). This

limit may be increased to 32,768 in a subsequent version of the

program.

2) Maximum cluster size = 65,528 bytes. I do not believe that a DOS

disk can ever have clusters larger than 64 Kb, but my limit is a few

less than that and may, since sector and cluster sizes tend to run in

powers of 2, effectively limit cluster size to 32 Kb - this should be

enough for most people!.

3) Because of certain hardware limitations of the chips used to

support direct memory access (DMA, the method used to transfer data

from disk to memory and vice versa) in the IBM PC and most clones and

compatibles, HDTEST may be unable to use available memory efficiently

when cluster sizes (and the buffers needed to hold them) are large.

This inefficiency can result in "Insufficient Memory" or "Memory

Allocation" errors, even on systems with more than the minimum 256 Kb

of free memory prescribed above. The only cure for this is to free up

(or buy!) more memory (remove RAMdisks, remove resident software,

etc.).

4) This version of HDTEST will run on ATs and most AT clones, but may

not, even with the /A flag set, be able to detect corrected ECC errors

when run on some of the clones - a warning message will be displayed

when the program initializes itself if corrected ECC errors are not

being detected.  The program will also not detect corrected ECC errors

when invoked with the /I flag (under DOS 3.0 or above) or the /D flag.

5) The current Shareware version of HDTEST will run under IBM DOS 4.0

and Compaq DOS 3.31 but cannot be used to test the large (> 32Mb) DOS

partitions which these DOS versions support. The Registered version of

the program fully supports these large partitions.

6) For reasons which are poorly understood (at least by the author!),

HDTEST runs very slowly on Epson Equity II systems, particularly when                      ____

used to test "plug-in" hard drives of the "Hardcard" or "Flashcard"

variety. For this hardware, the estimated run times given elsewhere in

the documentation need to be multiplied by about 2.5.                                       __________          ___

Page 9

Documentation for HDTEST V4.45          Copyright 1988, P. R. Fletcher          _______________________________        _______________________________

RUNNING THE PROGRAMS                                   ____________________

Always run HDCHEK on any new hardware/software configuration before

you attempt to run HDTEST (at least the first time). The latter

program has been written as conservatively as possible, but it is not

possible to predict the behavior of every conceivable combination of

hardware and operating system when dealing with a program which

bypasses most of the controls which normally protect the user from

destroying his world. HDCHEK is invoked with the command line:

HDCHEK device [/A] [/Cn] [/D | /I] [/Hnnnn]

"device" is the device to be tested (e.g. A:, B:, etc. - the colons

are permitted but not required). For versions of HDCHEK from V4.35,                            ___

the spaces between the device identifier and the flags (and between

different flags) are optional - the command lines "HDCHEK C:/C2/D" and

"HDCHEK C: /C2 /D" are equivalent.

The /A(T) flag is used to tell the HD program that it is being run on

an AT or AT-clone (which has its hard disk BIOS in the system ROM at

F0000H, rather than in a separate ROM at C8000H). This flag also

forces HDTEST to compensate for a known bug in the (IBM) AT's floppy

disk BIOS when used on 1.2 Mb floppy disks.

The /C(lone BIOS) flag is used in systems which do not support the IBM

BIOS's INT 11 "equipment determination" BIOS function (e.g. the Tandy

2000). The parameter ("n") is the number of floppy drives attached to

the system, which is normally determined automatically.

The /D(OS interrupts) flag forces HDCHEK to use the hardware-

independent DOS interrupt 25 for access to the target device. It is

usually not required, since the program is usually smart enough to          _______                                    _______

recognize devices which have to be accessed this way (e.g. logical

disk partitions, non-bootable hard disks, some removable-media hard

disks). If the /D flag is not used, the program will issue a warning                                    ___

message if it finds it has to use the DOS interrupts.

N.B. the definition of the /D flag changed at V4.00 - the new /I flag          _____________________________________________________________________

has the same effect as the old (pre-V4.00) /D flag. Only one of these          _____________________________________________________________________

flags may be present on the command line.          _________________________________________

The /I(nterrupt 13) flag has no effect under DOS 2.x.  Under DOS 3.0

or above, it forces HDCHEK to use the existing INT 13 vector (which it

always does under DOS 2.x), and prevents the program from trying to

determine the ROM BIOS entry point; the flag should be used if you

know that your system's fixed disk BIOS uses INT 13 but does not

follow IBM's "rules" or if the program crashes the system when invoked

without it.

The /H(ex BIOS address) flag is used to tell the HD programs that the

ROM BIOS of the device being tested is located at nnnn, which is

interpreted as a Hexadecimal segment address, rather than at C800,

which is the default. There must be no space between the H and the

number. /HF000 is similar to /A in its effects, but does not correct

for the bug in the IBM AT floppy disk BIOS mentioned in the above

description of the latter flag.

Page 10

Documentation for HDTEST V4.45          Copyright 1988, P. R. Fletcher          _______________________________        _______________________________

After a short delay, during most of which the specified device will be

active, a description of the device's logical structure will be disp-

layed.  After a further delay, and more disk activity, a message will

be printed giving the number of files and subdirectories found on the

disk, and the program will exit back to DOS. Note that the total

number of files and subdirectories reported by HDCHEK will be greater

than the sum of the counts reported by CHKDSK. This is because HDCHEK

counts the "." and ".." entries in subdirectories and CHKDSK doesn't.

Errors and/or problems encountered, if any, will be reported on in

some detail. A number of example screen displays and log files

produced by running HDCHEK (and HDTEST) on different systems will be

found in Appendix C at the end of this documentation.

Always run the DOS CHKDSK program immediately before running HDTEST on

a disk. If CHKDSK reports any problems, its /F option (or other means)

should be used to correct them before HDTEST is run.                                         ____________________

HDTEST is invoked with the command line:

HDTEST device1 [device2] [optional parameters]

"optional parameters" may include one or more of the following:

[/A] [/B] [/Cn] [/D | /I] [/F] [/Hnnnn] [/N] [/P] [/Rn] [/Snnnnn] [/Zn]

"device1" is the name of the device to be tested, as for HDCHEK.

"device2" (which, if present, must be different from "device1") is the

name of a device on which a log file (called HDTEST.LOG) is to be

written. If "device2" is not specified, errors are reported to the

console terminal and the DOS console redirection facility should NOT                           ___________________________________________________

be used in an attempt to redirect them elsewhere. For versions of          ________________________________________________

HDTEST from V4.35, the spaces between the device identifier and the

flags (and between different flags) are optional - the command lines

"HDTEST C:/A/F" and "HDTEST C: /A /F" are equivalent.

The /A, /C, /D, /H, and /I flags have the same meaning and effect that

they do for HDCHEK. When used for HDTEST, the /D flag forces the use

of INT 25 to read from and INT 26 to write to the target disk.

The /B(oot/Partition sector test) flag forces the program to include

the boot/partition sector (track 0, head 0, sector 1) in the DOS area

for the purposes of read/write testing. The default action is only to

perform read and seek tests on this sector, even if read/write testing

of the DOS area is specified. Since the boot/partition sector is

effectively read-only under normal circumstances (it is only written                                                                       _______

to during low-level formatting and by FDISK), read/write testing is          __

less vital than for the rest of the disk. In addition, at least one

hard disk controller (a Western Digital board which allows a single

disk to be addressed as two "physical" devices) will not allow test

patterns to be written to this sector. Certain devices (chiefly

RAMdisks) appear to DOS to be hard disks but do not, in fact, possess

boot/partition sectors. On these devices, HDTEST will report

check/test results on the boot/partition sector while actually

reading/writing the device's boot sector - the latter sector will

simply end up being checked/tested twice under these circumstances.

Page 11

Documentation for HDTEST V4.45          Copyright 1988, P. R. Fletcher          _______________________________        _______________________________

The /F(ast mode) flag causes the program to use only two test patterns

(all 1s and all 0s) for read/write testing. It is intended for use on

floppy disks and for "quick and dirty" testing of other devices.

The /N(o Query) flag disables the individual prompts for confirmation

which the program normally issues before remapping clusters which are

marked as "Reserved" or which are part of a file which is marked as

"System", "Hidden", or "Readonly".  The /N flag does not prevent the                                                               ___

program from prompting for confirmation before updating the disk.

The /P(rint) flag causes the log file to be directed to the PRN device

(normally LPT1), and overrides any "device2" specification.

The /R(epeat)n flag causes HDTEST to go through n complete testing

cycles (omitting n, or setting it to zero, causes HDTEST to run

continuously until terminated by pressing <Esc>). Instead of stopping

after completing the testing of allocated data clusters, the program

immediately starts again with the DOS area. Any bad clusters found and

flagged in the first pass should be reported as "already marked as

bad" in subsequent passes. The /R flag implicitly sets the /N flag.

The /S(tart)nnnnn flag causes HDTEST to start testing occupied                                                                ________

clusters at cluster number nnnnn (expressed as a decimal number).

There must be no space between the "S" and the number. This option

will mainly be useful when a previous run of HDTEST was interrupted

(by pressing <Esc>), since it allows you to start where you left off.

Note that, in order to be sure that data is not remapped to a bad

area, HDTEST always tests all empty clusters before starting to test                       ______       ___

occupied clusters. For this reason, using the /S flag will not result

in a large time saving if your disk is nearly empty, and it will make

the most difference on a full, or nearly full, disk.

The /Z(ero bad sectors)n flag controls how HDTEST deals with sectors

that cannot be read correctly (unrecoverable read errors) when it

remaps bad clusters. Versions of HDTEST up to and including V4.10

(somewhat cavalierly) disregarded the remaining contents of the entire

cluster if one or more sectors in it were unreadable, and remapped it

to a cluster of NULLs. The default action is now to replace only the

unreadable sectors with NULLs, transferring the remaining sectors                     _______

intact to the remapped cluster. There are, however, two other options,

which can be chosen by setting n to 0 or 2 (if n is omitted, 1, or any

other number, the default action is taken). Setting n to 0 uses the

old default (remaps the bad cluster to a cluster of NULLs), while          ___                                      _______

setting n to 2 goes to the other extreme and copies all data read

(some of which is almost certainly invalid) to the remapped cluster.

It is always safe and often advisable to use the /Z2 option provided                                                                      ________

that you do not try to use any non-ASCII files that have been remapped          ______________________________________________________________________

with unrecovered read errors and this (/Z2) option. Remapped ASCII          ___________________________________________________

files can be inspected with an editor and fixed up where necessary -

non-ASCII files (e.g. .EXE, .COM, .WKS files) may appear to be

undamaged (the program may run, the worksheet be readable, etc.) but

can still contain a small change which will cause trouble later, and

they cannot easily be checked. The large "holes" which /Z options 0

and 1 leave in such files usually allows it to be much more obvious

that they are defective!

Page 12

Documentation for HDTEST V4.45          Copyright 1988, P. R. Fletcher          _______________________________        _______________________________

After displaying its copyright message, HDTEST reads the FATs and

scans the root directory and all subdirectory trees of the specified

device before initiating a short dialogue. All questions are of the

"Yes/No" type and should be answered by typing "Y" or "N" (case does

not matter) and pressing the <Return> key.  Pressing the <Return> key

alone is equivalent to giving the default response (which appears in

square brackets at the end of each prompt).  HDTEST then proceeds to

test the specified device. By default, the DOS area (boot sector,

FATs, and root directory) is not subjected to Read/Write testing,

since failures in this area usually make themselves apparent fairly

rapidly. The user can, however, choose to run these tests on the DOS

area too, or to limit testing to read and seek tests only over the

whole partition.

In its reports to the screen, HDTEST uses the convention that read-

only testing is "Checking" while read/write testing is "Testing" - it

always "checks" the boot/partition sector and the rest of the DOS

area, but only "tests" them if requested to do so.

In "checking" or "testing" a block (sector or cluster), HDTEST first

reads it (retrying, if necessary, up to five times) and then does the

longest possible seek away from it before attempting to read it again.

"Checking" is complete at this point. "Testing" continues with the

program sequentially writing 20 (or 2, for "fast mode") different bit

patterns to the block and checking that each one can be read

correctly, before rewriting the original contents of the block (and

checking to ensure that it has been restored correctly). Errors that

occur at any point in this process are reported and (if the block is

in the data area - errors in the DOS area are always treated as fatal)

the block is remapped. If an error is detected in a "Reserved" cluster

or in one that is part of a file which is marked as "System",

"Hidden", or "Readonly", the user is normally asked for confirmation

before the cluster is remapped, otherwise it is assumed that remapping

is desired.

HDTEST keeps its own copy of the FAT and a map of the device's

directory structure in memory. A bad cluster is remapped by

transferring its contents to a previously free cluster and (if the bad

cluster was in use) changing all necessary FAT and directory entries

so that the pointer chain which maintains the structure of the

affected file or subdirectory remains valid. The bad cluster's FAT

entry is then altered to identify it as bad to DOS. The device's FATs                                                                  ________

and directories are not altered until the whole device has been

scanned, and then only after the user has confirmed that it is OK to

proceed. This means that a power failure and/or an inadvertent power-

off which occurs while the program is running will cause the loss of,

at most, one cluster of data and of no FAT or root directory

information (unless you are very unlucky and it happens after you have                                      ____

confirmed that you want a damaged data area to be remapped and while

this is being done). If a power failure and/or power-off occurs while

a cluster which contains the directory information of a subdirectory

is being tested, part or all of that subdirectory will probably become

inaccessible to DOS, although the data in it will remain on the disk

and be potentially recoverable with the aid of (e.g.) the Norton

Utilities (TM).

You can safely interrupt the testing process part way through (by

Page 13

Documentation for HDTEST V4.45          Copyright 1988, P. R. Fletcher          _______________________________        _______________________________

pressing the <Esc> key) with no risk of leaving a partially updated

FAT or directory. If you do interrupt the program in this way, you

will still be given the option of updating the disk, so that bad

clusters which were detected before <Esc> was pressed can be remapped.

Errors which occur in the program's initialization phase cause it to

abort with an error message and (sometimes) a hexadecimal error code.

The meaning of the error code is context-dependent. Errors detected

during read-write testing only cause the program to abort if they

occur during testing of the DOS area, or if remapping is impossible

(only likely to happen if the disk is full). In any event, a

description of the error is written to the log file (if any) and/or to

the screen. This specifies the cluster or (for the DOS area) sector

number and the name of the affected file (if any), and defines the

type(s) of error which occurred. Appendix B contains a discussion of

some of the more technical aspects of data storage on disks (floppy

and hard), and includes more information about some of the possible

errors that may be encountered.

HDTEST takes a very long time to run to completion because of the                         ____

large number of reads and writes it does for each cluster. A typical

time for full testing ("slow" mode) of an XT disk (10 Mb) would be 2

hrs; full disk testing on a 30 Mb Enhanced AT takes at least 4 hrs.

Page 14

Documentation for HDTEST V4.45          Copyright 1988, P. R. Fletcher          _______________________________        _______________________________

DEALING WITH BAD CLUSTERS                                 _________________________

It is, unfortunately, not unusual for a new hard disk to have a small,                                                                          _____

constant, number of bad clusters. IBM's quality control standards          ________

allow, for example, 5% of the surface area of a disk to be bad (!).

These bad clusters, which correspond to imperfections in the magnetic

coating on the disk, will normally be detected and marked by the DOS

FORMAT program, and HDTEST will report them as "already marked as

bad".  This should not be regarded as evidence of any real problem

with the disk, although some suppliers (not including IBM) will                                                  ___

replace such disks under warranty if pressed to do so.  It is also not

unusual for a disk to develop a few more defects as it ages. When                                          ___

these do develop, they are also usually constant in size and position,

and will be reported as "already marked as bad" by HDTEST on runs

subsequent to that during which they are first found. The occasional                                                                    __________

appearance (and subsequent persistence) of a new bad cluster is,

therefore, not necessarily evidence of serious trouble developing.

However, if more than one or two new bad clusters are reported by

HDTEST, and especially if two consecutive runs (or passes, if the /R

flag is set) of HDTEST report different bad clusters, there is a good                                        _________

chance that the hard disk or controller is developing a serious

problem - the disk should be backed up immediately and the disk and/or

controller sent for repair at the earliest opportunity.

New bad clusters in the data area are automatically mapped out by

HDTEST, and no further action is necessary. Bad sectors in the DOS

area can be handled in two ways:

1) If you are able to do a low-level format on your hard disk (most                                     _________

makers of "add-on" hard disks supply a method of doing this, either in

the form of a short program, or by giving instructions for running a

program in the ROM BIOS from DEBUG), back up the entire hard disk                                                           ______

(including any non-DOS partitions), run the low-level format, use

FDISK to repartition the disk, and then run the DOS FORMAT program. If

HDTEST now reports no errors in the DOS area you can restore all your

files and continue to use the disk. You should, however, use HDTEST to

test (at least) the DOS area fairly frequently for a month or so to be

sure that the problems are not going to recur.

2) If you do not have the means to do a low-level format, or if you

follow the above instructions and HDTEST continues to report errors in

the DOS area, your only other choice is to back up the DOS partition,

reconfigure the disk with FDISK so that the new DOS partition starts

above the bad area, reFORMAT it, and restore all of the old files to

the new DOS partition (which will unavoidably be a little smaller than

the old).

If absolute sector zero (track 0, Head 0, sector 1) becomes bad, the

disk is useless and must be replaced. This sector is only subjected to

read/write testing if the /B flag is specified on the command line, so          __________

any reported errors (in sector -1) during normal testing represent a                                         __

problem reading the sector and are likely to be significant. Problems                  _______

encountered during read/write testing of this sector do not always                                                                  ___

indicate a bad disk - certain controller cards may not allow test

patterns to be written to it, resulting in a report of multiple errors

on read/write testing. Consequently, if multiple errors are reported

Page 15

Documentation for HDTEST V4.45          Copyright 1988, P. R. Fletcher          _______________________________        _______________________________

on read/write testing of this sector, but read-only testing proceeds

without problems and the rest of the disk seems to be OK, there is

probably no cause for concern.

BUGS AND OTHER UNPLEASANT SUBJECTS                            __________________________________

Although, like all programmers, I like to think that my product is

perfect, it is possible that there are a few bugs lurking in the

depths of the program, and likely that some combination of software

and hardware exists which HDTEST cannot figure out and/or cope with.

If you think you have found a bug and/or if you cannot get either of

the programs to work on your system, do the following:

1) Reread this documentation! Make sure that you are doing exactly

what the documentation says to do. Make sure (if you are testing a

drive with removable media) that the media is not write-protected.

2) Boot your system from a "virgin" DOS disk - preferably a copy of

your DOS distribution disk - and try to run the HD programs again. If

they run in this environment but not in your normal DOS environment,

try reconstructing the system under which they failed by adding things

one by one until the problem recurs. If you find a specific piece of

software which causes the trouble, let me know about the

incompatibility.

3) If the above approaches do not help, or if HDCHEK runs but HDTEST

bombs (the reverse should never happen), send me as detailed a

description of the problem as possible and I will do my best to

diagnose and fix it. A bug report should always include a full                                                   ______

description of the system being used, as well as the output from a run

of HDCHEK.

I can be contacted by U.S. mail at the address on the Registration

Form, by BIXMail to pfletcher, or by Compuserve EMail to 72255,305. Be

aware that, although I do pay attention to bug and problem reports

from people who are not Registered users, the latter will always be

given priority.

Page 16

Documentation for HDTEST V4.45          Copyright 1988, P. R. Fletcher          _______________________________        _______________________________

CHANGES FROM PREVIOUS VERSIONS                              ______________________________

The first version of the HD programs actually released (not including

beta-test versions) was V2.60. The programs have grown and developed

gradually, for the most part, but the releases listed below have

constituted the most significant "watersheds" - features added and

changes made between listed releases are treated here as if first                       _______

included in the next listed release.

Version 4.45 (this version)          ____________

1) Deficiency causing "fallback" to DOS interrupt level under some

circumstances when attempting to test "extended devices" on the second

physical hard drive fixed (HDCHEK and HDTEST).

2) Limited support for IBM DOS 4.00 and Compaq DOS 3.31 large

partitions added - these partitions are now recognized as valid

(allowing HDTEST to be used to test "normal" partitions on systems

which include large ones) but cannot, themselves, be tested (HDCHEK                _______

and HDTEST).

Version 4.41          ____________

1) Deficiency (introduced at V4.00) causing HDCHEK to issue an

incorrect error report on an abort resulting from a FAT comparison

error corrected.

2) Minor deficiency, causing HDTEST to create an empty log file on the

specified device when both a log device and the /P flag were

present, corrected.

3) Work-around for hardware multi-sector read ECC problem (see

Appendix B) added to HDTEST.

4) Support for small (<= 10 Mb) hard disks patched to have small

clusters and 16-bit FATs added.

5) Requirement that option flags be separated from device specifier(s)

(or each other) by one or more spaces or tabs, removed.

6) /R option added to HDTEST for multi-pass disk testing.

Version 4.30          ____________

1) Support for BIOS-level access to DOS 3.3 extended partitions added.

2) /Z option added to HDTEST to control remapping of unreadable

sectors in bad clusters.

Page 17

Documentation for HDTEST V4.45          Copyright 1988, P. R. Fletcher          _______________________________        _______________________________

Version 4.10          ____________

1) Assorted minor changes made to screen display.

2) /C option added to allow the programs to be used on some non-IBM

compatible MS-DOS systems.

3) /D option redefined - old /D option became new /I option.

4) /F option for fast read/write testing added to HDTEST.

5) /H option added to allow BIOS-level access to the disk on systems

with non-standard hard disk BIOS locations.

Version 3.13          ____________

1) Assorted minor bugs and deficiencies in error reporting and

termination routines in HDTEST fixed.

2) Minor bug in HDTEST, causing multiple recovered ECC errors

occurring during initial cluster reads to be reported as recoverable

errors without further qualification as to their type and origin,

fixed.

3) Disk reset added before retry on all errors.

4) Absolute limit on number of files and directories on device

removed.

5) Total sector or cluster count shown on screen by HDTEST during

testing along with current sector or cluster number.

6) /A option for full functionality on ATs and AT clones added.

7) /B option to include the boot/partition sector in the DOS area for

the purposes of read/write testing added to HDTEST. This sector was

previously tested with the DOS area.

Page 18

Documentation for HDTEST V4.45          Copyright 1988, P. R. Fletcher          _______________________________        _______________________________

Version 2.73          ____________

1) Bug (in V2.60) which could cause bad termination of HDTEST and

possible subdirectory inconsistencies if a sector which was not in use

was found to be bad, fixed.

2) Bug (in V2.70) which caused HDTEST to enter a terminal input loop

if ANSI.SYS was not loaded and <Esc> was entered to abort it, fixed.

3) Minor bug in V2.60 (which resulted in a failed directory scan if

HDCHEK was run on a write-protected device) fixed.

4) Internal changes made to avoid interference with expanded memory

device handlers.

5) Ability to update disk after HDTEST terminated by user (typing

<Esc>) added.

6) /S option to start checking occupied clusters at a given cluster

number added to HDTEST.

Page 19

Documentation for HDTEST V4.45          Copyright 1988, P. R. Fletcher          _______________________________        _______________________________

APPENDIX A - SUMMARY OF FLAG SETTINGS                           _____________________________________

/A(T) - used to tell the HD program that it is being run on an AT or

AT-clone (HDCHEK and HDTEST). /A is not quite equivalent to /HF000.

/B(oot/Partition sector test) - used to include the hard disk

boot/partition sector in the DOS area for the purposes of read/write

testing (HDTEST only).

/C(lone BIOS)n - used in systems which do not support the IBM BIOS's

INT 11 "equipment determination" BIOS function (e.g. the Tandy 2000).

The parameter ("n") is the number of floppy drives attached to the

system, which is otherwise determined automatically.

/D(OS interrupts) forces the program to use only DOS interrupts

(INT 25 and INT 26) for access to the target device. May not be

combined with the /I flag (HDCHEK and HDTEST).

/F(ast mode) limits read/write testing to the use of two patterns

only (HDTEST only).

/H(ex BIOS address)nnnn - used to tell the HD programs that the ROM

BIOS of the device being tested is located at nnnn, which is

interpreted as a Hexadecimal segment address, rather than at C800,

which is the default.

/I(nterrupt 13) - forces the program to use the existing INT 13

vector.  This is the default under DOS 2.x. May not be combined with

the /D flag (HDCHEK and HDTEST).

/N(o Query) - disables the individual prompts for confirmation which

the program normally issues before remapping "special" clusters - does

not prevent the program from prompting for confirmation before          ___

updating the disk (HDTEST only).

/P(rint) - causes the log file to be directed to the PRN device

(HDTEST only).

/Rn - causes HDTEST to perform n complete testing cycles (or to

continue until interrupted for n=0) (HDTEST only).

/Snnnnn - causes HDTEST to start testing occupied clusters at cluster                                                   ________

number nnnnn (expressed as a decimal number) (HDTEST only).

/Zn - controls remapping of unreadable sectors in bad clusters (HDTEST

only). n may have one of the following values:

0 - Remap clusters with unreadable sectors to all NULLs

1 (default) - Remap only unreadable sectors to NULLs

2 - Transfer all data read to new cluster

Page 20

Documentation for HDTEST V4.45          Copyright 1988, P. R. Fletcher          _______________________________        _______________________________

APPENDIX B - DISK ERROR MESSAGES                             ________________________________

Users of earlier versions of this software have repeatedly requested

that I provide a more detailed explanation of the significance of some

of the error messages displayed by HDTEST when it encounters a bad

sector or cluster on the disk. It is not possible to do this without

assuming (or imparting) a more detailed understanding of the "nitty-

gritty" details of data storage on hard and floppy disks than most

people want or need, let alone possess. This Appendix is not,

therefore, "required reading" for the safe or effective use of the HD

programs. It is intended as a tutorial for those who would like to

understand something more of what is really going on on their hard or                                               ______

floppy disk drive when the little red light comes on.

For the purposes of what follows, it is (fortunately) not necessary to

know or understand exactly how changes in magnetic flux on the surface

of a hard (or floppy) disk are translated into binary data, or vice

versa. That could be the subject of another long dissertation. I will

simply take for granted that binary data, conceptually organised into

8-bit bytes, can be read and written by the disk drive hardware under

the command of its controller (which usually resides on a separate

card plugged into one of the computer's expansion slots). I will be

describing how blocks of data are physically arranged on a disk and

how the disk controller goes about ensuring that the data it reads has

not been corrupted since it was written.

I propose to start off by running over some of the error-detection and

error-correction techniques and procedures that are applied to data

stored on magnetic media. These techniques and procedures are

necessary because, although magnetic media are very reliable and the

technology of recording and playback is well understood, there always

remains the possibility that a defect in the medium or some other

intercurrent problem may cause the sequence of bytes read back from a

disk or tape to be something other than a perfect duplicate of the

sequence that was stored. If a single copy of the raw data were all

that were recorded, there would be no way of immediately detecting

(let alone correcting) such an error. It might be brought to light

when the file that was changed caused problems when it is used by or

executed as a program, but it might not become apparent until it had

propagated itself through and caused problems or errors in multiple

other programs or files. This would clearly be an unacceptable state

of affairs. In order to prevent this from occurring, then, there

should be some means of checking data which is read in from storage

for errors, and/or (preferably) of correcting any errors which may

have occurred before any use is made of the data.                        ______

Although error checking and/or correction can (and sometimes does)

take place at any level from that of the application program down to

that of the device controller, I am only going to describe the means

that are used by hard and floppy disk controllers. When a block of

data is passed to the controller to be written to the disk, a series

of arithmetical operations is performed in sequence on all the bytes

in the block, to produce a 2-byte (for diskettes) or multi-byte (for

hard drives) code. The diskette controller implements what is known as

a cyclic redundancy check (CRC) algorithm, which is optimized to allow

detection of all single bit errors and many longer error bursts (it is          _________

Page 21

Documentation for HDTEST V4.45          Copyright 1988, P. R. Fletcher          _______________________________        _______________________________

reasonable to assume that errors will most probably either affect a

single bit at random or, if they result from media defects, a number

of adjacent bits). The longer (at least 32-bit) error correcting code

(ECC) calculated by a hard disk controller gives enough information to

allow detection and correction of, at least, all single bit errors and                          ______________

detection of most likely longer error bursts. The CRC or ECC is saved

on disk with the data and is read back in by the controller when the

data is read. The controller then compares the saved CRC or ECC with

one newly calculated from the data. If the two are identical, the data

is assumed to be valid and a successful read is signalled. If there is

a discrepancy, a diskette controller always returns the data with a

"CRC error", since the CRC code does not contain enough information to

allow the incorrect bit or bits to be identified. On a hard disk, if

the ECC comparison suggests a correctable error, it will be corrected,

and the read call will return with a "Corrected ECC Error" code but

with (presumably) valid data in the transfer buffer. If a more

extensive error is detected, on the other hand, the call returns with

a "Uncorrectable ECC Error" code (which is the same as the CRC error

code).

The 32-bit ECC used by IBM and many compatible disk manufacturers

allows correction of single-bit errors only. Manufacturers of other,                                                 ____

usually more expensive, controllers use longer and more complex ECCs,

to allow the correction of some multi-bit errors, but the necessity to

minimise the time taken to calculate the ECC (which is part of "access

time") sets tight limits on how complex it can be. ECC error

correction is unfortunately not infallible. It is possible for a                                      ___

complex, uncorrectable error to result in the same ECC disparity as a                   __

different, correctable, error, so that invalid "corrected" data can be                                                                          ______

returned. The longer and more complex the ECC, the less likely this

becomes, but there is no way of completely avoiding this ambiguity,

short of keeping multiple copies of the data on the disk. In addition,

some hard disk controllers* apparently abort with the "corrected ECC

error" code immediately on encountering a sector containing such an                      ___________

error, even if it is not the last sector in the block specified by a                               ___

(multi-sector) read request. This non-intuitive (to say the least of

it) behavior can combine with DOS 3.x's habit of ignoring corrected                                                           ________

ECC errors to result in apparently inexplicable disk-related problems,

up to and including system crashes (if the file in which the problem

occurred contained executable code). This hardware deficiency may be

related to the problems which have plagued users of DOS 3.3 on some

(previously) apparently IBM-compatible non-IBM hardware, since these

also appear to result from deficiencies in the clones' handling of

multi-sector disk reads. Prior to V4.36, HDTEST did retry corrected                                                          ___

ECC errors which were reported on the first read of a cluster, but

would eventually (if the corrected error persisted) accept the data

read as valid and copy and remap it to a "good" cluster. This could

cause trouble if the controller had the "immediate abort on corrected

ECC error" problem described above and the ECC error were not in the                                             ___

last sector of the cluster, since the sectors after the "bad" one

would not actually have been read, and would be lost (actually

replaced by the corresponding sectors from the previous cluster read)                                                         ________

_____________________________________________________________________

* This behavior is known to occur with some early Western Digital

controllers (1002A-WX1, 1002S-WX2) but it may also occur with others.

Page 22

Documentation for HDTEST V4.45          Copyright 1988, P. R. Fletcher          _______________________________        _______________________________

after remapping. On a machine with this problem, DOS would not, of

course, have done any better than the earlier versions of HDTEST at

reading the bad cluster, and HDTEST at least warned you that there was

a problem, but the behavior described was certainly not ideal. From

V4.36, HDTEST rereads clusters in which a persistent corrected ECC

error is reported on the initial read sector by sector, to ensure that                                                ________________

all the data in the cluster is transferred when it is remapped. In          ___

most cases (as long as the ECC error was really correctable), this                                                   ______

will save the cluster (and the file of which it is a part) intact.

DOS allows the programmer and/or computer user to think of and deal

with disk data in terms of files, lines, and/or records, but data is

always moved between a disk and memory a sector at a time. Any disk

read or write request passed to DOS eventually gets translated into a

command to read or write one or more sectors on the disk, commencing

at a specified head, track, and sector. A floppy disk may have either

one or two usable surfaces, corresponding to the top and bottom of the                            ________

disk. A hard disk usually has at least two surfaces, and may have

more, since multiple physical "platters" are frequently incorporated

in a single device. Each surface is normally accessed by and

corresponds to a single read/write head, which moves radially across                                             ____

it, although one surface on newer disks is often reserved for "servo"

tracks, which are written with a special pattern that allows the

controller to ensure that the read/write heads are kept exactly

aligned with the data tracks. For a single device, the (data) heads

are, by convention, numbered sequentially from zero up to one less

than the total number present. Each surface is further divided into a

number of circumferential tracks (or cylinders) and each track is                                    ______     _________

divided into a number of sectors. By convention, the tracks on each                                   _______

surface are also numbered sequentially from zero, with track zero

being the outermost track. The sectors on each track are normally

numbered from one to a number corresponding to the total number of                        ___

sectors on the track, but adjacent sectors do not normally have                                                        ___

consecutive sector numbers. A typical arrangement of the seventeen

sectors on a single track of an XT hard disk, for example, would be as

follows: 1,8,15,5,12,2,9,16,6,13,3,10,17,7,14,4,11. This "inter-                                                                    ______

leaving" of sectors helps to make multi-sector disk reads and writes          _______

more efficient. The controller requires a finite time to process the

data from one sector before it can start reading (or writing) the

next. During this time, of course, the disk continues to rotate and

one or more of the sectors which physically "follow" the one that was

last accessed pass under the read/write head unread. Ideally, the

interleave (number of physical sectors separating consecutively

numbered sectors) should be such that the next sector is just about to

come under the read-write head when the controller is free to deal

with it, but, because this time is not exactly predictable, it is

better to be ultra-conservative (as IBM is) and make it one or two

sectors longer than necessary than to be too optimistic. Too small an

interleave value is very undesirable, because it results in the                              ____

controller having to wait almost a complete disk rotation time for

each sector it misses to come round again.

At the beginning of each disk sector is an address mark, a short,                                                     ____________

recognizable (to the controller) bit sequence containing the track and

sector number. The process of writing the address marks for each

sector on a disk is known as "low-level" or "sector" formatting. The

DOS FORMAT program does this when it is run on floppy disks, but on

Page 23

Documentation for HDTEST V4.45          Copyright 1988, P. R. Fletcher          _______________________________        _______________________________

hard disks the DOS program merely makes sure that all required address

marks are present - they are created by the disk manufacturer or

supplier's low-level format program, which may or may not be available

to the end user. After each address mark there follows a short gap (to

allow the controller time to recognise and respond to the address

mark), then the raw data, then the CRC or ECC data. A sector normally

contains 512 bytes of raw data, but DOS (and HDTEST) can recognise and

deal appropriately with devices with non-standard sector sizes.

When a sector read/write command is passed to the disk controller, it

(the controller) causes the disk's read/write head to move (seek) to                                                                      ____

the specified track, waits a few milliseconds for it to settle down,

and then starts to "pay attention to" the data passing underneath it.

The controller locates and reads the address marks to confirm that the

read/write head is positioned at the correct track and to identify

each sector. If data is being read from the disk, the raw data and

recorded CRC or ECC from the requested sector(s) are transferred into

the disk controller's internal buffers and (assuming that no errors

are detected), thence to main memory. If data is being written, the

controller derives the error-checking data from the raw data and

writes both to the disk. Some of this process is under the direct

control of the controller hardware, but it is supervised by the

device's ROM BIOS, which is responsible for error-reporting. Errors

can occur at almost any stage of the read/write process, but (if the

disk is working at all) usually result from media defects or problems

developing in the mechanical components of the hard disk drive. Media

defects, once present, are generally constant in size and position

(although jarring a disk while it is being written to may sometimes

generate a temporary bad sector).                     _________

When HDTEST detects an error during read/write testing of a sector or

cluster which has not previously been marked as bad, a (normally)

four-line error message is printed, either to the screen (by default),

to the printer (/P option), or to the logging file (if an output

device was specified). The first line of the message simply indicates

the sector or cluster number at which the error occurred. The second

line indicates what sort of error was detected and during what phase

of read/write testing it occurred. The third line indicates whether

HDTEST was able to recover and/or remap the data from the bad sector

or cluster, and the fourth indicates either the file which a bad

cluster formed part of or that the cluster was not in use. If it was

not possible to read one or more sectors from a cluster which

contained data (unrecoverable read error), a further two-line message

is printed which notes how many sectors were unreadable and how the                                          _______

data from the bad cluster has been remapped (which depends on whether

a /Zn option was specified on the command line).

When the program detects an error during read/write testing of a

sector in the DOS area, only the first three lines described above are

printed. The remainder of this Appendix contains more details of and

further information on the possible contents of lines two and three of

HDTEST's disk error message.

Page 24

Documentation for HDTEST V4.45          Copyright 1988, P. R. Fletcher          _______________________________        _______________________________

The second line of the disk read/write error message will usually take

the form: "[Error Type] occurred during [phase of testing]". Possible

values for [Error Type] are as follows (in alphabetical order):

Address Mark Not Found - this is a fairly common error reported for

bad blocks which are present from "scratch" on a new disk, since a

significant media defect may well not retain sector formatting data

and/or the manufacturer may have made sure that a known bad track on

the disk will not be used by deliberately not writing address marks

there. If new bad clusters are reported with this error message, it is

likely that the hard disk surface has been physically damaged (perhaps

by a shock to the drive) but possible that the controller has a

problem.

Bad Controller - this means what it says, but check that all boards

and cables are properly seated before giving up.

Bad Sector - the BIOS was instructed to read a sector number which (it

believes) does not exist on the device. This indicates either an

internal program error or that the program is having a problem

determining the physical layout of the disk. Check the values reported

by HDCHEK against those in the disk documentation.

Bad Track - analogous to "Bad Sector".

Buffer Comparison - the data read from a sector differs from that

written to it. This should not happen, since it suggests either that

the device's error-detection functionality has failed or that it has

been overwhelmed. If it does occur, the most likely causes are

controller and memory problems (in that order of likelihood).

Corrected ECC - as discussed at some length above, the data retrieved

from a hard disk was incorrect but the device's error-correction

hardware was (probably) able to correct it.

DMA Boundary Violation - should not occur (HDTEST positions its

buffers to avoid crossing absolute segment boundaries). Suggests

either an internal program error or controller/BIOS problem.

Hardware Timeout - a seek or other operation was not completed within

the maximum time allowed by the BIOS. Check connections, reseat

boards, otherwise suspect the mechanics of the disk.

Init Fail - should not occur (on a working disk).

Internal - internal program error. Make sure that your working copy of

HDTEST has not been corrupted. The disk is probably not defective.

Invalid Command - should not occur. Treat as Internal Error.

Multiple Errors - one or more additional errors occurred while HDTEST

was attempting to recover from an initial one. Multiple phase report

lines will be printed but the error types cannot be further specified.

Recovered - HDTEST recovered from the error and did not report on the

error type (should not occur).

Page 25

Documentation for HDTEST V4.45          Copyright 1988, P. R. Fletcher          _______________________________        _______________________________

Reset Fail - The device could not be reset (after an error). Usually

indicates major problems with the disk or controller, but check

connections, etc.

Sector Not Found - the requested sector's address mark was not found.

Likely to result from a media defect.

Seek Fail - after a seek to a track, the controller found that the

read/write head was not positioned correctly. Usually indicates a

significant problem with the (drive) hardware.

Sense Fail - the controller cannot find the drive! Check connections,

etc.

Uncorrected ECC - data error too major to be corrected. Usually a

media defect. This message will also be produced if a CRC error is

detected on a floppy disk.

Undefined - the BIOS cannot figure out what went wrong, but something

did!

Unknown - likewise!

Write Protection - HDTEST is attempting to write to a write-protected

device.

Possible forms for the rest of the second line are as follows:

... Error (unrecovered) occurred during first read - the cluster could

not be read without error in five tries before a seek to the other

"end" of the disk.

... Error occurred during first read - the error occurred during the

first read but was recovered after one or more retries.

... Error occurred during read after seek - the error occurred when

the cluster was read after a seek away.

... Error occurred during pattern read/writes - the error occurred

during the pattern read/write phase of testing.

... Error occurred during final write - the error occurred when the

saved copy of the cluster was being restored to the disk.

... Error occurred during read after final write - the error occurred

when the restored cluster was being read back.

... (Buffer comparison) error occurred after final read - the copy of

the cluster read back after restoration to the disk differed from its

original memory image.

Page 26

Documentation for HDTEST V4.45          Copyright 1988, P. R. Fletcher          _______________________________        _______________________________

There are only three basic forms of the third line:

This error is (or "These errors are") recoverable (no data loss has

occurred). - the bad cluster was read successfully and has been

successfully restored to its original form, but it remains suspect and

will (with your permission) be remapped.

This error is unrecoverable, but remapping is possible. - the bad

cluster was read successfully but has not been restored to its                                                ___

original form. Remapping should be successful, but the data will be

lost if this is not permitted.

This error is unrecoverable, and data may be lost! - the bad cluster

was not read successfully and will be remapped as provided by the

value of the /Z switch parameter if remapping is permitted. In any

event, some data loss has almost certainly occurred.

Page 27

Documentation for HDTEST V4.45          Copyright 1988, P. R. Fletcher     _______________________________        _______________________________

APPENDIX C - EXAMPLE PROGRAM RUNS                   _________________________________

Screen display from successful run of HDCHEK on partition occupying the

whole of a 20 Mb hard disk:

Device C: appears to be the DOS partition of a hard disk

It contains 41531 sectors

It starts at absolute sector 17 on the disk      {THIS IS CORRECT}                                                  _______________

The total partition size is approximately 20 Mb

There are 4 heads and 17 sectors/track

Sector size is 512 bytes

Each of the 2 (16 bit) FATs occupies 41 sectors

The Root Directory occupies 32 sectors

The data area starts at DOS sector 115, and contains 10354 clusters, each

consisting of 4 sectors

The directory scan was successful; a total of 1260 files and subdirectories

was found.

Screen display from successful run of HDCHEK on a freshly FORMATted DSDD

9-sector floppy disk:

Device A: is not a hard disk - using DOS interrupts

Device A: is a floppy disk

The disk contains 720 sectors

There are 2 heads and 9 sectors/track

Sector size is 512 bytes

Each of the 2 (12 bit) FATs occupies 2 sectors

The Root Directory occupies 7 sectors

The data area starts at DOS sector 12, and contains 354 clusters, each

consisting of 2 sectors

The directory scan was successful; a total of 0 files and subdirectories

was found.

Screen display from run of HDCHEK on 20 Mb DOS partition - directory

structure bad or unrecognizable:

Device C: appears to be the DOS partition of a hard disk

It contains 41531 sectors

It starts at absolute sector 17 on the disk

The total partition size is approximately 20 Mb

There are 4 heads and 17 sectors/track

Sector size is 512 bytes

Each of the 2 (16 bit) FATs occupies 41 sectors

The Root Directory occupies 32 sectors

The data area starts at DOS sector 115, and contains 10354 clusters, each

consisting of 4 sectors

The preliminary directory scan failed.

Page 28

Documentation for HDTEST V4.45          Copyright 1988, P. R. Fletcher     _______________________________        _______________________________

Screen display from run of HDCHEK on 20 Mb DOS partition - FATs not

identical:

FATS are different

FAT discrepancy at byte 20F - FAT 1 has FF; FAT 2 has F8

Continue comparison (Y/N)? [Y]: n     {user input}                                _

Device C: appears to be the DOS partition of a hard disk

It contains 41531 sectors

It starts at absolute sector 17 on the disk

The total partition size is approximately 20 Mb

There are 4 heads and 17 sectors/track

Sector size is 512 bytes

Each of the 2 (16 bit) FATs occupies 41 sectors

The Root Directory occupies 32 sectors

The data area starts at DOS sector 115, and contains 10354 clusters, each

consisting of 4 sectors

Screen display from run of HDCHEK - program unable to deduce disk

structure - disk not recognizable as a DOS device:

Unrecognised media descriptor byte! - see error code

Error Code = F1      {F1 is not a media descriptor byte used by DOS}

Page 29

Documentation for HDTEST V4.45          Copyright 1988, P. R. Fletcher     _______________________________        _______________________________

Log file from successful run of HDTEST on 20 Mb DOS partition:

HDTEST Log File - Wed Jan 07 11:44:52 1986

Testing Device C:

No new bad clusters identified - HDTEST finished.

Wed Jan 07 16:10:41 1987

Log file from run of HDTEST on 20 Mb DOS partition - one bad cluster

detected and remapped:

HDTEST Log File - Mon Jan 05 21:09:32 1987

Testing Device C:

Read/Write error at cluster number 7623!!

Corrected ECC error occurred during pattern read/writes

This error is recoverable (no data loss has occurred).

Bad cluster is in file \MYFILES\DATA5\FFT1234.DAT

1 new bad clusters identified, marked, and remapped - HDTEST finished.

Tue Jan 06 05:08:04 1987

Page 30

REGISTRATION FORM - HDTEST V4.45                        ________________________________

Please complete this form and return it with a check or money order for

$35.00 (+ $5.00 for shipping outside North America) or equivalent to:

Peter R. Fletcher

1515 West Montgomery Avenue

Rosemont

PA 19010

U.S.A.

European users should send Registrations to the address on the

program's copyright screen.

None of the information on this form will be passed on to any other

individual or commercial organization.

--------------------------------------------------------------------------

This information is required to process your Registration:

Last Name:____________________________  Other Names:______________________

Address:______________________________  Town/City:________________________

State:_____________  Post Code:____________  Telephone:_________________

(evenings/weekends)

I understand that this software is supplied without warranty, express or

implied, and agree not to hold its author liable for any direct or

consequential loss arising out of my use of it.

Signature:______________________________

-------------------------------------------------------------------------

You are requested, but not required, also to answer the following

questions:

On What type(s) of computer are you using HDTEST? _________________________

CPU (8088, 8086, etc.)? __________   Amount of RAM (kB)? ____________

Type & Size of Hard Disk(s)? ____________________________________

Make and Type of Hard Disk Controller(s)? ______________________________

Do you own a Modem?                                             Y  N

Are you a member of BIX?   Y  N        Compuserve?              Y  N

How did you obtain/hear about this program?

Did you think that the $35.00 registration fee for this program was:

Very Good Value?              About Right?             Excessively High?

Are there any other utility programs that you would like to see made

available as "User Supported Software"? What sort of programs?