SBCD2.ADD can be used in OS/2 system which  supports  layered

CD-ROM  device  management structure.  Supported by OS/2 2.1,

existing OS/2 2.0 users can also access the structure by using

OS2CDROM.DMD in the OS2CDR package for OS/2 2.0.  To get the

package, type GO IBMOS2 in Compuserve network.

INSTALLATION OF SBCD2.ADD, CD-ROM DEVICE DRIVERS

------------------------------------------------

1) If the following line is in your OS/2 CONFIG.SYS file, delete it or

"REM" it out.

DEVICE=C:\OS2\CDROM.SYS ...  (where "..." are parameters)

2) If the OS2CDROM.DMD file is not already in the C:\OS2 directory,

copy that file form disk 1 of your OS/2 Ver. 2.1 installation

diskette to the C:\OS2 directory.

3) Copy the SBCD2.ADD file to C:\OS2

4) Put the following lines in your OS/2 CONFIG.SYS file in the given

order near the bottom of the OS/2 CONFIG.SYS file.  (Putting these

lines near the top could cause problems.)  Make sure that none of

these lines are already in your OS/2 CONFIG.SYS file.  If they are,

delete them and put them with the lines shown below.

BASEDEV=SBCD2.ADD /P:220

BASEDEV=OS2CRDOM.DMD

IFS=C:\OS2\CDFS.IFS /Q

DEVICE=C:\OS2\MDOS\VCDROM.SYS

LASTDRIVE=H

NOTE: A) The last line (LASTDRVIE=H) will problably be different for

your system.  For example, if the current last drive of your

system is "D: ", your CD-Rom drive will be "E:" and the last

line should read: LASTDRIVE=E

This line may be optional.  If you have problems, try eliminating

this line and see what happens.

B) The first line contains the I/O address of the Sound Blaster

card as a parameter (/P:220).  If your I/O address is different

(not 220), change the number on the first line to reflect your

setting.  Check "DESCRIPTION OF SBCD2.ADD SWITCHES" at the end

of this document for info. on how to set other switches.

5)  Reboot your system.  The SBCD2.ADD driver should now work.

DRIVER INSTALLATION PROCEDURE IF YOU HAVE NOT INSTALLED OS/2 2.1

----------------------------------------------------------------

1. Copy the SBCD2.ADD CD-ROM driver to the OS/2 2.1 installation disk 1.

2) Add the following line to the CONFIG.SYS file contained on

the OS/2 installation disk 1.  (Put it on the last line of

the CONFIG.SYS file.)

BASEDEV=SBCD2.ADD /P:220

3) Install OS/2 2.1 on your system.  When selecting installation

options, do NOT install the CD ROM option!  (OS/2 2.1 does

not include the SBCD2.ADD driver.)

4) Copy the SBCD2.ADD file to the C:\OS2 directory.

5) If the OS2CDROM.DMD file is not already in the C:\OS2 directory,

copy that file from disk 1 of your OS/2 Ver. 2.1 installation

diskette to the C:\OS2 directory.

6) If the CDFS.IFS file is not already in the C:\OS2 directory,

copy that file from disk 1 of your OS/2 Ver. 2.1 installation

diskette to the C:\OS2 directory.

7) Make certain the following lines are in the OS/2 CONFIG.SYS

file on your hard disk in the given order.

BASEDEV=SBCD2.ADD  /P:220

IFS=C:\CDFS.IFS  /Q

BASEDEV=OS2CDROM.DMD

DEVICE=C:\OS2\MDOS\VCDROM.SYS

LASTDRIVE=H

NOTE:  A) The last line (LASTDRIVE=H) will probably be different

for your system.  For example, if the current last drive

of your system is "D:", your CD-ROM drive will be "E:"

and the last line should read: LASTDRIVE=E

This line may be optional.  If you have problems, try

eliminating this line and see what happens.

B) The first line contains the I/O address of the Sound

Blaster card as a parameter (/P:220).  If your

I/O address is different (not 220), change the

number on the first line to reflect your setting.

Check "DESCRIPTION OF SBCD2.ADD SWITCHES" at the

end of this document for info. on how to set other

switches.

8) Reboot your system.  The SBCD2.ADD driver should now work.

DESCRIPTION OF SBCD2.ADD SWITCHES

---------------------------------

/P:xxx

------

This switch specifies the port address(in hex) of Sound Blaster card

used. Options for xxx are 220 and 240.  Default setting for xxx is

220.

Example:

BASEDEV=SBCD2.ADD /P:220

/M:xxx

------

This switch specifies the port address(in hex) of MKE CD-ROM interface

card used.  Options for xxx are 300, 310, 320 and 330.  Default

setting for xxx is 300.

Example:

BASEDEV=SBCD2.ADD /M:300

NOTE : /P and /M are mutually exclusive.  If both are specified, the

latter one will be effective.  If none is specified, default

is Sound Blaster.

/D:xxx

------

This switch specifies the drives to be run at double transfer speed

(300KB/s).  The option /D:xxx specifies all daisy chained drives to

run at double speed.  You can specify individual drive using

/D:0, /D:1, /D:2, or /D:3 where 0, 1, 2, and 3 are the ID numbers of

drives you want to run at double speed.  If you have only one drive,

use the following format:

BASEDEV=SBCD2.ADD /D:1    (1 is the jumper setting on the drive)

If you have 2 or more drives, use this format:

BASEDEV=SBCD2.ADD /D:1,3  (1 and 3 are jumper settings on the drives)

/N:xxx

This switch specifies the drives to be run at normal transfer speed

(150KB/s).  The option /N:xxx specifies all daisy chained drives to

run at normal speed.  You can specify individual drive using

/N:0, /N:1, /N:2, or /N:3 where 0, 1, 2, and 3 are the ID numbers of

drives that you want to run at normal speed.  If you have only one drive,

use the following format:

BASEDEV=SBCD2.ADD /N:1    (1 is the jumper setting on the drive)

If you have 2 or more drives, use this format:

BASEDEV=SBCD2.ADD /N:0,2  (0 and 2 are jumper settings on the drives)

NOTE : The default is double transfer speed wherever it is possible.

If the same drive has been set more than once, the latest

setting will take effect.

********** End of README.TXT **********