SoftBlue V.1.00 - the first Windows dialer

-----------------------------------------------------------------

Conception

SoftBlue is as far as I know the first bluebox facility

available under Windows. The battle of GUIs isn't over, but

Windows is in fact the most popular operating system on PC, so I

thought it would be quite convenient to combine the multitasking

features of Windows - you may write a text or play one of the

stupid games while you download - with a nice easy-to-use blue

box.

SoftBlue runs with Windows 3.1 and above only. It uses Multimedia

Extensions, common dialog boxes and 286 code - and in fact I refuse

to support lamers with Win 3.0...

How to use SoftBlue

The main window

SoftBlue is similar to any Windows programs. In fact, there are

two list boxes, on the left you see the available break

sequences (trunks), on the right there are the telephone

numbers. If you click on a telephone number, the selected break

sequence changes to the standard sequence of this number. If you

click on a break sequence, you simply select it.

The button BREAK plays the break sequence only, without a number

after it, naturally also leaving out prefix and suffix of this

number. If you press DIAL the selected number is played, with

the prefix and suffix of the selected break sequence. If there

is no break sequence selected, the standard dial set is used

(see below). BREAK & DIAL (as you may guess) plays break

sequence and number together, while CANCEL stops playing. You

have to select both a break sequence and a phone number before

you select BREAK & DIAL.

Dialog and menu items

The Break Sequence pull down menu allows to edit or delete a

selected break sequence or to create a new one.  You can edit

the following break sequence elements:

- a unique name to identify the break sequence

- phone number to dial before the break

- a starting delay

- a set of 9 sinus tones, each with max. three tones played

together, a duration and a pause.

- prefix and suffix of a number dialed afterwards

For the numbers dialed before and after the break you may choose

a different dial set. For example, you start with DTMF (the same

tones your handset produces) and go on with CCITT-5.

Alternatively, you can also define your own dial sets.

The same actions are available for phone numbers in the Phone

Number pull down menu.

A phone number consists of

- again a unique name

- the phone number

- the number's standard break sequence, which is automatically

selected in the break sequence list box when you click on that

phone number

With Edit Dial Set! you can edit existing dial sets (DTMF and

CCITT-5 are predefined) or create a new one. You can also select

the default dial set, which is used whenever a number is dialed

and no break sequence is selected. This default dial set is also

used when numbers are dialed manually with the Dial Pad.

In the Options menu you can select whether or not the Dial Pad,

a window similar to dial buttons on a phone, is displayed, and

if the SoftBlue window should always be in the foreground. It

cannot be overlapped by other application windows then.

Additionally you can select the output device and options

concerning the device there. The following devices are

supported:

- Adlib OPL-3 chip (if not used by windows as MIDI device for

example). This device is not recommended, because playing is not

interruptable and Windows stops working while SoftBlue plays a

sound. Don't worry if your speaker keeps silent, just uninstall

your MIDI driver from Windows, it should work then alright.

- the standard wave ouput device which is included in windows.

For nearly any sound cards there are drivers available. Check if

their sound quality is sufficient, some kind of clicks while

playing are critical. This problem occurred with Gravis Ultra Sound

drivers.

Using this device, SoftBlue calculates all data which is

necessary and gives them to Windows for playing. The sinus

creating algorithm is fast as ass (on a 486/66 MHz 20 s 44 kHz

sound takes probably .2 s to calculate), playing itself is done

in the background. You can, once sinus data is calculated, abort

playing at any time. Please beware, that the sample data has to

be stored somewhere, and 20 s at 44 kHz means 880 KB which are

kept in memory, so use lower sample rates, if the sound isn't

played completely and you are low on memory. Normaly, 11 kHz are

enough for sinus tones of 3000 Hz frequency. Also check if the

sample rate used is supported by card and driver, SoftBlue does

not do that for now, I will include it later.

- (still VERY beta, don't use it) output via the voice function

of a ZyXEL modem, the function is partly implemented, but is in

fact as buggy as your cellar, so be careful, I will work on that

further on.

With ZyXEL modems pulse dialing will be no longer a problem.

Probably I will try to implement an automatic voice recognition,

so if an operator talks at the end of the line, the modem will

automatically hang up.

I have not removed this menu point although it is that beta - I have

better things to do than to cripple my programs... So don't be silly

and keep your hands off this point.

Wait for V. 2.00 to do blueboxing with the ZyXEL.

I hope you get fine connections using SoftBlue. I am rather a

coder than a phreak, so you have to get trunks and numbers on

your own. But if you miss a convenient function tell me at

Madman's Sanctuary. I will see what I can do.

Have fun and good trading!

Contact the Coder (Heatchy) on:

Madman's Sanctuary - Police WHQ - Cybrix EHQ - TRSI/Faith GHQ

Soon 5 Nodes USR Dual + 2 Nodes ISDN - +49-30-ELITE

or Maverick on: Concept Elite - TRSI/Faith WHQ ^ Mirage - Razor WHQ

Ice Station Zebra - Pentagramm WHQ

P.S. Sorry for the mistakes in that document, it is rather a leaflet than

a manual, but handling is too easy to write novels about it.