CHIPS Version 1.8

(c) 1990 William C. Parke

CHIPS is an program designed to explore the hardware on an IBM PC

compatible.  It will make a series of tests to determine, as far as

possible, the kind of computer chips present in the particular machine.

These include the Central Processing Unit (CPU), the Numeric Data

Processor (NDP), and the various Asynchronous Communication Elements

(ACEs) used on the serial ports.  CHIPS demonstrates that software is

capable of checking detailed hardware capabilities directly. Programmers

can use more sophisticated coding when more advanced hardware is found.

Run CHIPS with the DOS command:

CHIPS

To save a copy of the hardware configuration, use DOS redirection:

CHIPS >chips.scn

Although the hardware configuration of a computer can be verified by

internal visual inspection, it is not a particularly easy task.

Moreover, the critical question of whether the hardware will respond to

software can be at issue.  CHIPS uses software to make direct tests of

hardware elements.  It goes beyond the assumptions of the operating

system, whose configuration may be at variance with the actual

situation.  Thus, CHIPS will allow one to determine possible conflicts

between the hardware and the settings of the configuration DIP switches

on the motherboard of a PC or the CMOS memory in the AT computer.

The CPU: Central Processing Unit

Because of various quirks, design changes, and design improvements, it

is possible to distinguish between the following types of CPUs:

8088    The original 8/16 bit Intel CPU used in the IBM PC

8086    The full 16 bit version

V20     NEC's clone of the 8088 with added 8080 mode

V30     NEC's clone of the 8086 with added 8080 mode

80188   An enhanced 8088 with expanded i/o instructions

80186   An enhanced 8086 with expanded i/o instructions

80286   The first micro CPU with protected mode

80386   Extended with 32 bit registers, new instructions

80486   Integrated with NDP, cache buffer, extended addressing

For the 8088, CHIPS will test for a serious hardware bug in the early

production chips which could cause the computer to halt during a program

execution.  For the 80386, CHIPS looks for the infamous 'multiply bug'

in early production runs.  A double sigma (printed on the chip) means

the chip passed Intel's tests.  If CHIPS confirms this test, it will

print a double sigma after the chip name.

CHIPS next determines the speed of the CPU and the number of effective

wait states currently used by the CPU in accessing memory.  These

are printed after the CPU identification.

The Interrupt Controllers

The orginal IBM PC used the Intel 8259 chip to manage the various

hardware interrupts from the keyboard, drives, clock, ports, etc.  The

AT machine cascaded the master 8259 with a slave 8259 controller,

expanding the number of hardware interrupt lines from eight to

fifteen.  CHIPS will report on the existence of one or both of

these controllers.

The NDP: Numeric Data coProcessor

CHIPS will also test for the Numeric Data Processor (math coprocessor)

to see if it is available to accompany the CPU.  [Note: The NDP is

integrated into the 80486.]   Function and designed improvements in the

NDP allow software to distinguish between the following possibilities:

no coprocessor

8087    NDP to accompany the 8088/8086

80287   NDP to accompany the 80286 or the 80386

80387   NDP to accompany the 80386

The ACEs: Asynchronous Communication Elements

The serial ports on the IBM PC are controlled by an Intel 8250 or its

equivalent.  Most PCs have at least one 8250.  Some serial cards use a

faster version, the 16450 or the 16550.  A more advanced chip, the

16550AN, contains an internal 16 byte buffer, useful in multitasking

operations.  CHIPS can distinguish:

no ACEs on any ports

8250    Serial controllers

16450   Serial controllers, extended version of the 8250

16550   Serial controllers, faster version of the 8250

16550AN Serial controllers, with FIFO buffer.

The Parallel and Game Ports

The IBM PC add-on cards generally do not use an intelligent chip to

control the printer ports or the game port.  Instead, these ports, when

available, are controlled by logic chips.  CHIPS will search for real

ports at the default locations.

Display of IBM BIOS Assumptions for Attached Devices

At boot time, the IBM PC and AT reads the user configuration settings

for various devices which may be attached to the system.  Some of this

information is stored in the BIOS (Basic Input/Output System) memory.

CHIPS will allow you to compare the actual hardware it found with the

settings recorded in the BIOS memory.  In the case of the NDP, CHIPS

shows the BIOS NDP-present flag (yes or no).  For the serial ports,

CHIPS will give the current name assigned by DOS to the ports used for

serial devices.  This assignment may vary, as it is can be changed using

various swapping or redirection utilities.  CHIPS reports the same kind

of information for the parallel (printer) ports.  After showing if the

game port is active, CHIPS gives the setting of the BIOS equipment flag

for this port.  A test is also made to see if a mouse driver has been

installed in the memory of the machine.

Mouse Support

Chips can distinguish a serial, bus, and PS/2 mouse when a mouse

driver is present.

Sound Support

Chips will test for a MIDI or AdLib port and accompanying driver.

Example of CHIPS Display:

Chip Identification V1.8� (c) William C. Parke 1990

Central Processing Unit: -8088-- 8 MHz with 0 wait states.

8088 interrupt test: Passed.

Math coprocessor: -8087-- Present and flagged in BIOS.

Interrupt Controllers: Master 8259 Found, Slave 8259 Not Found.

Serial port  3F8H: -8250-- Active.

port  2F8H: -8250-- Active.

port  3E8H: Not Active.

port  2E8H: Not Active.

Parallel port  3BCH: ------- Active as LPT1:

port  378H: ------- Active as LPT2:

port  278H: Not Active.

Game port  201H: Not Active and not flagged in BIOS.

Sound I/O support: PC Speaker.

Mouse: No Driver Found.

Technical Details:

Distinguishing CPUs

"8088 programmable registers and addressing modes are

identical to the 8086 in every way.  Except for execution

speed, to the programmer the 8088 is identical to the 8086."

(Osborne/McGraw-Hill, Russell Rector and George Alexy, The

8086 Book, 1980)

A statement such as the above is a challenge to programmers!  The

second sentence turns out to be wrong.  There is a trivial software test

one can make to distinguish the 8088 from the 8086 (and the 80188 from

the 80186), despite the claims that the only difference between these

chips is the treatment of the external data hardware lines.

The test in question takes advantage of the fact that the 8088 has

a 4-byte prefetch queue, while the 8086 has a 6-byte prefetch queue.

This means that these chips pull the next set of instructions to

be executed into an internal buffer.  Now suppose we instruct the CPU

to change one of the memory locations which had just been prefetched.

Then the code at that location will not be the same as the prefetched

code.  On the other hand, if the change is made to the code in memory

just out of reach of the 8088, but still within reach of the 8086

prefetch operations, these chips will execute different instructions.

For example,

XOR     CX,CX

CLI

MOV     BYTE PTR [NEXT],NOP

NOP

NEXT:   INC     CX

STI

JCXZ    8088

will result in CX=1 on the 8086, but CX=0 on the 8088.

The 80186 can be distinquished from the 8086 using a variety of

software differences.  One of the simplest is the shift operation.

Chips after the 8086 do not waste time bit shifting more than 32

times (the maximum size of any one register), so the shift value

(in the CL register) is truncated modulo 32 before the operation.

Thus:

MOV     CL,33                   ; 8018x shifts modulo 32

SHL     CL,CL

JCXZ    8088

will result in a non-zero flag being set if the chip is above the 8088

generation, since only a single bit shift occurs in that case.

The 8088 and 8086 differ from the 80286 and later chips in how they

handle the value of the stack pointer when it is pushed onto the

stack:

PUSH    SP

POP     AX

CMP     AX,SP

JNZ     8088

After the above operation, AX will contain the value of the stack pointer

before the push if the chip is an 80286 or later.

The V20/V30 chips can be detected by executing extended V20 code which

also is meaningful on the 8088.  For example,

SET1    CX,0            ; v20 code

sets a bit in the CX register.  On the 8088 this code is

POP     CS

SBB     AX,0C1H

which does not affect the CX register.  However, this code must be avoided

on an 80386 machine, which can lock at the POP CS.  A simpler test

is the following:

XOR     AL,AL

MOV     AL,40H

MUL     AL

JZ      NEC

since the 8088 will set NZ on multiply while the NEC does not.

The 80386 handles more flag register bits than the 80286.  By trying

to set the upper four bits of the flag register and then testing:

MOV     AX,0F000H

PUSH    AX

POPF

PUSHF

POP     AX

AND     AX,0F000H

JZ      80286

the upper four bits will be reset on chips below the 80386.

The 80486 can be distinquished from the 80386 by the fact that its

prefetch queue is 32 bytes in size instead of 16 bytes.  The test

code is similar to that above which separates the 8088 from the

8086, except one needs 15 nops instead of one.  Alternatively, one

can attempt to set the 29th bit of the CR0 register (cache write-through

bit), which cannot be set on the 80386:

MOV     EAX,CR0         ; 0FH,20H,0C0H

MOV     EBX,EAX         ; 66H,8BH,0D8H

XOR     EAX,20000000H   ; 66H,35H,00H,00H,00H,20H

MOV     CR0,EAX         ; 0FH,22H,0C0H

MOV     EAX,CR0         ; 0FH,20H,0C0H

CMP     EAX,EBX         ; 66H,3BH,0C3H

JZ      80386

CPU speed tests are made by timing repeated CPU divide instructions.

Wait states are extimated by the time required to make repeated

memory-to-memory moves.

Finding the Numeric Data coProcessor:

Attempting to execute commands designed for the math coprocessor

on a system without one is a delicate matter.  Some instructions

will cause particular CPUs to lock up.  (Unfortunately, FSTENV, which

could be used to save the current state of the coprocessor, freezes

PS/2-55 80386SX machines).  Others require the CPU to wait for the

coprocessor to finish its task before continuing.  Assemblers and

compilers often insert the WAIT instruction in such cases.  However,

asking a CPU to WAIT when there is no coprocessor is asking it to wait

forever.  It appears that

FINIT                   ; initialize coprocessor

JMP $+2

JMP $+2

JMP $+2                 ; imitate a wait

FSTCW WORD PTR NDPCW    ; store NDP control word

MOV AX,WORD PTR NDPCW

AND AH,11111B           ; isolate inf,rounding,precision bits

CMP AH,,00011B

JZ GOT_NDP

will pass through the digestive tracks of the 80x86 series without

blockage, even when no coprocessor is connected.  When there is a

coprocessor, the control word bits 8-12 will be set to indicate the

default 64 bit precision, nearest rounding, and projective infinity.

To differentiate an 8087 from an 80287 or later, one can drop the

interrupt-enable-mask bit of the control register, disable interrupts,

read the register back, and check if the mask bit is still 0 (interrupts

still enabled):

AND     WORD PTR NDPCW,0FF7FH    ; drop bit 7 IEM inter enable mask

FLDCW   WORD PTR NDPCW           ; load control word into NDP

FDISI                            ; disable interrupts (ignored on 287)

FSTCW   WORD PTR NDPCW           ; store control word in memory

TEST    WORD PTR NDPCW,80H       ; is IEM of control word back again?

JNZ     80287

The 80387 will reset the zero flag bit in its status register on

initialization, but the 80287 will not:

FLDZ                            ; zero st(0)

FXAM                            ; set status flags

FINIT                           ; reset 80x87

FSTSW   WORD PTR NDPSW          ; get status word

TEST    BYTE PTR NDPSW+1,01000000B      ; check C3 = ZF

JNZ     80287                   ; 80287 does not reset ZF

Testing for Real Serial Ports:

A relatively straight foward test can determine if the standard

serial ports have an active chip controlling them.  For example, reading

in the line-control register, reversing its first six bits, sending the

reversed set out, reading it back in, then checking if the new bits

are as they were sent out.  Of course, this testing should be done

with interrupts turned off, and the original register value restored

before turning them back on:

MOV     DX,3FCH         ; first serial port + 2

IN      AL,DX           ; get line control reg

MOV     BL,AL           ; save

CLI

XOR     AL,0111111B     ; reverse bits

MOV     BH,AL           ; save reversed value

OUT     DX,AL

JMP     $+2

IN      AL,DX

MOV     AH,AL           ; save read value

MOV     AL,BL           ; get original line control

OUT     DX,AL           ; restore original

STI

CMP     AH,BH           ; see if it changed

JZ      GOT_COM

The 16450 chips has an additional eighth read/write port, which

can be tested for.

To tell if one of the newer 16550 or 16550AN chips is present,

one can use the two new bits (6-7) in the interrupt-identification

register.  These bits, when set, clear and enable the FIFO internal

receive buffer.  These chips are distinquishable by whether these

bits can be set:

bits 7  6

0  0   8250 ACE

0  1   unknown

1  0   16550

1  1   16550AN

Parallel Port Testing:

A live parallel port can have its interrupt-enable bit set at the

control register port.  Testing if this bit can be toggled serves as an

appropriate probe.  The testing must be done with software interrupts

turned off as there may not be any interrupt handler for the parallel

port.  (Moreover, the original IBM PC had a hardware fault which

prevented proper parallel port interrupt handling.)

Mouse Presence:

There is no standard mouse port on the IBM PC.  One can operate a

mouse from a serial port, a bus card, or through the PS/2 keyboard

controller.

CHIPS can look for a mouse driver interface in software.  If the

driver is present, there is a very good chance the system has the

necessary hardware for a mouse.  CHIPS attempts to find if the mouse

driver has taken over a serial port interrupt.  It will look at

the keyboard controller for a PS/2 mouse.

Sound Ports:

The MIDI-401 (musical instrument digital interface) takes the ports

330H and 331H.  CHIPS tries to get an acknowledgement to a status read

of a possible MIDI card.  An AdLib music card uses the port 388H.  CHIPS

attempts to initialize the AdLib music card.  If successful, it will

look for an AdLib driver, which uses interrupt 65h.