############################################################################

C25SIM

An Affordable TMS320C25 Simulator

############################################################################

GETTING STARTED

C25SIM.EXE is a menu-based C25 simulator. To use it, type

c25sim foo

and it will load the files FOO.HI, FOO.LO, FOO.MAP, and FOO.LST. You will

see a "READY>" prompt. You can then press F1 to see the help screen, of F2

to see the source listing. In both the help screen and the source listing,

you can navigate with the Home, End, PageUp, PageDown, and up and down arrow

keys. Any other key will return you to the simulator.

Notice that the C25's registers and flags all appear on the right side of

your screen. Whenever the simulator executes code, these are updated.

Type "symbols" (and press Enter) to see the symbol table. This is extracted

from FOO.MAP. The simulator commands that require numerical arguments will

also accept symbol names, and will look up the value of the symbol.

Type "bp _interrupt_handler" to set a breakpoint at the beginning of the

interrupt handler. FOO.ASM sets up a timed interrupt, which saves the C25's

registers on a stack, increments the _L variable, restores the registers,

and returns to the main program. You will see interrupts occuring about

every 500 processor cycles.

Type "go". This command will cause the simulator to execute code until a

breakpoint is hit, or until you press a key on your keyboard. When the

simulator stops, it will notify you that it reached a breakpoint, tell you

the address, and print the names of any symbols whose values match that

address. You will see:

READY> go

Hit a breakpoint at 0053

_interrupt_handler

READY>

Type "go" again. The simulator will continue executing until it hits the

same breakpoint again. Type "rd _L" to read the data memory value at the

address for the _L variable. Type "go" and "rd _L" a few more times to see

that the interrupt handler is indeed incrementing _L.

The timer is displayed on the right side of the screen, by the name TIM. The

PRD register, or period register, is used to reload the timer after it

counts down to zero. At this point, TIM will have a value close to 0x1F4,

since it was recently reloaded when the interrupt occured. Type "cycles 100"

to run the simulator for 100 processor cycles. The value of TIM will be

around 0x190 at this point, indicating that it will time out in about 400

processor cycles.

############################################################################

SHAREWARE

C25SIM and its associated files represent significant effort. If you try

them for a couple weeks and find they are useful, you are invited to send

a check or money order for $25.00 to

Will Ware

26 Beacon St., Apt. 34B

Burlington, MA  01803

My email address is:

72143,315                       on Compuserve

wware@world.std.com             on Internet

############################################################################

THE HELP SCREEN

The following two pages are the help screen, which is the file C25SIM.HLP,

displayed in C25SIM.EXE when you press the F1 key. This is always available

while you are using the simulator. For your convenience, a copy of the help

screen follows.

QUICK COMMAND REFERENCE

load            Load hex and map files

listing         Load listing file (default extension: LST)

wp              Write program memory

rp              Read program memory

wd              Write data memory

rd              Read data memory

mr              Modify a C25 register

pwait           Program memory wait state

dwait           Data memory wait state

bp              Set a breakpoint

clrbps          Clear all breakpoint

go              Run N steps or until next breakpoint

cycles          Run for N processor cycles

reset           Reset the C25

bkgnd           Run C25 continuously as background process (or stop)

int             Manually trigger an interrupt

symbols         Show symbol table

match?          Find all global labels matching current PC

quit            Quit this program

Use Home, End, PageUp, PageDown, and up and down arrows to navigate, any

other key returns to the simulator.

Function keys:  F1 => view this help file

F2 => view source listing file

F3 => view symbol table

F4 => execute one instruction (active only in simulator)

F5 => string search (active only in viewer)

F6 => repeat string search (active only in viewer)

MORE DETAILS ON COMMANDS

Commands are case-insensitive. "WP", "Wp", "wP", and "wp" are all valid.

Many commands take numerical arguments. Numbers are interpreted as decimal

by default, and are interpreted as hexadecimal if they have the prefix "0x",

as in C. Numerical arguments can be decimal numbers (29), hexadecimal

numbers (0x1234), or global symbols defined in the MAP file. Assuming there

is a global symbol "_interrupt_handler", these would all be acceptable ways

to set a breakpoint.

bp 0x53

bp 83

bp _interrupt_handler

"Go" takes an optional numerical argument. If no argument is used, "go" will

execute code until either a breakpoint is hit, or a key is pressed on the

keyboard. If an argument is given, "go" will stop on either of those

conditions, or when that number of instructions have been executed.

"steps" and "cycles" each take a number, respectively specifying how many

instructions or processor cycles to execute.

"wp" and "wd" take two numerical arguments, an address followed by data.

"bp" takes on numerical argument, an address, and sets a breakpoint there.

"rp" and "rd" take an address, and optionally a second address. If two

addresses are specified, then a range of memory values is displayed, up to

but not including the second address, for example:

READY> rd 0x1C00 0x1C10

Data memory

1C00 - 1234 5678 0246 1357 1234 5678 0246 1357

1C08 - 1234 5678 0246 1357 1234 5678 0246 1357

"load" takes a filename, and loads Intel hex files for C25 code, and a Texas

Instruments map file, if one exists. For more details, see the description

below of the functions load_hex_files() and load_map_file().

"listing" takes a filename, and loads a new source listing file. This can be

viewed by pressing the F2 key. While in the listing view screen, you can

navigate with Home, End, PageUp, PageDown, and the up and down arrow keys.

Press any other key to return to the simulator. If the simulator is invoked

with a filename argument, it will try to load a listing file automatically.

If the argument is "FOO", the simulator will look for "FOO.LST". If an

extension is specified, it will use that.

"symbols" prints the symbol table (extracted from the MAP file), showing the

names of globally defined symbols and their hexadecimal values.

"match?" prints the names of all globally defined symbols whose values match

the current PC.

"mr" takes the name of a register, followed by a new value to assign that

register. The valid register names are: arp ov ovm intm dp arb cnf tc sxm c

hm fsm xf fo txm pm pc t sp ar0 ar1 ar2 ar3 ar4 ar5 ar6 ar7 bio acc p drr

dxr tim prd imr greg.

"pwait" and "dwait" set up wait states for ranges of program and data

memory. To specify a range of program memory with two wait states from

address 0x800 to 0xFFF, type "pwait 0x800 0xFFF 2". "dwait" works the same

way for data memory. Up to twenty ranges can be specified for program

memory, and another twenty for data memory.

"int" takes an interrupt type name, and manually triggers an interrupt of

that type. The valid C25 interrupts are: RS, INT0, INT1, INT2, TINT, RINT,

XINT, TRAP.

MORE DETAILS ON THE FILE VIEWER

The file viewer lets you conveniently view text files from the simulator.

At any time while simulating, you may press the F1 key to view this help

file, the F2 key to view the source listing, or the F3 key to view the

symbol table. When in the file viewer, you can navigate using the Home,

End, PageUp, PageDown, and up and down arrow keys.

Within the file viewer you can search for a string of text. Press F5 and

you will be prompted for the string to be searched for. The first line, at

or after your current position, that contains that string will be moved to

the top of the screen. When you press the F6 key, the next line containing

that string will be moved to the top of the screen. If the string is not

found, a warning to that effect will appear.

############################################################################

HOW TO ASSEMBLE AND LINK FOO.ASM

Files with the prefix FOO are some sample C25 code, which were assembled and

linked with TI's C25 assembly language tools. The procedure for assembling

FOO.ASM is illustrated below. On the distribution disk, you will find

FOO.ASM, FOO.LST, FOO.CMD, FOO.HI, and FOO.LO. FOO.HI and FOO.LO are Intel

hex files that can be used to program high-byte and low-byte EPROMs.

dspa foo.asm foo.obj foo.lst

dsplnk foo.cmd foo.obj -m foo.map

dsprom -intel a.out foo

del a.out

############################################################################

WHAT DOES FOO.ASM DO?

FOO.ASM exercises the C25's timer, interrupts, bit-reversed addressing mode,

and idle mode. The results of all this appear in two address registers, AR4

and AR5, and in three data memory locations labelled "_i", "_k" and "_L".

There is also an array of data memory locations called "rstring" used for

the recieve interrupt.

First FOO establishes AR1 as a stack pointer, and sets up a timed interrupt

to occur every 500 processor cycles. rstring is initialized to zeros. AR4,

AR5, _k, and _L are then initialized to zero. FOO then enters a loop (L1),

where it respectively increments and decrements AR4 and AR5 using

four-bit-wide bit reversal, and goes on to call a do-nothing subroutine.

After the subroutine call, FOO sets _i to zero and enters an inner loop (L2)

which will run ten times using AR0 as a loop counter. Inside the L2 loop, _k

is set to 17 times _i, and then _i is incremented. After the L2 loop runs

ten times, FOO calls a subroutine which checks the value of _L, and executes

an IDLE instruction if _L has reached 15. _L is incremented periodically by

the timed interrupt.

There is also a receive interrupt which fills rstring with incoming data.

When a data word of zero is received, rstring is refilled with zeroes and

the receive interrupt starts at the beginning again.

############################################################################

WHAT DOES SAMPLE.C DO?

SAMPLE.C is an example of how to use the simulator library functions to

write a special-purpose simulator. SAMPLE looks specifically for changes in

_k, _L, AR4, and AR5, and reports the changes, and how many processor cycles

have passed when they occur. SAMPLE reports when interrupts occur, and when

the C25 hits an IDLE instruction.

SAMPLE also simulates the operation of the C25's serial port by writing the

DRR register and triggering periodic receive interrupts.

As SAMPLE is running, you may:

Press the 'Q' key to quit and return to DOS,

Press 'R' to get a full report of the C25's registers, flags,

and status

Press 'M' to toggle a mode bit which is initialized to zero.

When the mode bit is one, changes in _k, _L, AR4, and AR5

are displayed on your screen.

Press 'S' to print the current contents of rstring, which is filled

by the receive interrupt with characters from the string

"Supercalifragilistic".

Press any other key to pause SAMPLE and get a shorter report

of C25 status

############################################################################

WRITING A CUSTOMIZED SIMULATOR

C25LIB.C is a library of routines for creating your own simulator very

easily. The effort of doing a small amount of C programming will be vastly

offset by the flexibility of being able to design a simulator specifically

to address your own application.

SAMPLE.C is an example C program that illustrates how to use the routines in

C25LIB.OBJ. SAMPLE.C was written specifically in conjunction with FOO.ASM.

The variables and routines available in C25LIB.OBJ are spelled out in the

header file C25LIB.H.

WINDOWS.C provides some handy windowing functions that were used to define

the user interface for C25SIM. These will be available to you if you are

compiling with Microsoft C, but are not available with Borland Turbo C.

############################################################################

IMPORTANT THINGS

A.      Your code must call the routine initialize_simulator() before

attempting to execute any C25 code. This routine initializes the

table that allows the simulator to disassemble and execute C25

instructions.

B.      To accomodate the windowing done in C25SIM, all the simulator

library functions route their printed output thru the routine

void __write(char *);   /* two underscores */

which you must provide. For many applications, you can simply

use the following definition, taken from SAMPLE.C:

void __write(char *s) { printf("%s", s); }

############################################################################

1. Registers and Flags

The C25 flags and registers are as follows, and are available to be inquired

or altered in your C program.

extern int arp, ov, ovm, intm, dp;

extern int arb, cnf, tc, sxm, c, hm, fsm, xf, fo, txm, pm;

extern long int accumulator, p_register;

extern unsigned int pc;

extern int t_register, _stack[8], _sp, ar[8];

extern int mp_mc_mode;

extern int ports[16], bio;

extern int drr, dxr, tim, prd, imr, greg;

All the C25's registers and flags are available as variables in C. Most of

these variables are 16 bits (int). The accumulator and the P register are 32

bits (long int). The following registers are odd lengths: ARP (3 bits), DP

(9 bits), ARB (3 bits), and PM (2 bits). The upper bits of these registers

are ignored by the simulator. The following flags are only one bit, and the

upper 15 bits are ignored: ov, ovm, intm, dp, cnf, tc, sxm, c, hm, fsm, xf,

fo, txm, mp_mc_mode, bio. xf and bio are external pins on the C25.

mp_mc_mode is also an external pin, which determines whether code is fetched

from the C25's internal ROM (microcontroller mode) or from external ROM

(microprocessor mode). By default it is set to 1, indicating that the C25 is

in microprocessor mode.

############################################################################

2. Steps and Cycles

extern unsigned long int steps_taken, cycles;

There are two long integers that mark time in the simulator. "steps_taken"

measures how many instructions have been executed. Repeated instructions

(controlled by the RPT or RPTK instruction) will not count: only the repeat

instruction itself will be recorded. "cycles" measures the actual execution

time for your code, in processor cycles. If you clock your C25 at 40 MHz,

each clock cycle is 100 nanoseconds.

############################################################################

3. Wait States

typedef int (*iufunc) (unsigned int);

iufunc pwaitf, dwaitf, iwaitf;

C25SIM normally assumes that all memory is zero-wait-state. This assumption

can be easily changed to match your hardware. This affects the "cycles"

variable. There are three pointers to functions: pwaitf, dwaitf, iwaitf,

respectively regarding program memory, data memory, and I/O ports. These

pointers are initialized to NULL. To simulate non-zero-wait-state memory,

write a C routine that accepts an unsigned integer (a memory address) and

returns an integer (the number of wait states for writing or reading that

address), and assign the appropriate pointer to point at that function.

Here is a sample usage.

/* convenient way to specify memory address ranges */

#define ui(x)  ((unsigned int) (x))

#define range(x,lo,hi)  ((ui(x) >= ui(lo)) && (ui(x) <= ui(hi)))

int program_wait_states(unsigned int addr)

{

/* a small block of REALLY slow memory */

if (range(addr, 0x100, 0x1FF)) return 10;

/* everything else is zero-wait-state */

return 0;

}

void main(void)

{

initialize_simulator();

....

pwaitf = program_wait_states;

....

/* run simulation */

}

############################################################################

4. Accessing Program and Data Memory from C

extern int warn_progmem_writes;

extern int *program_memory_address(int a);

extern void write_program_memory(int a, int d);

extern int read_program_memory(int a);

extern int *data_memory_address(int a);

extern void write_data_memory(int a, int d);

extern int read_data_memory(int a);

As you load code into the simulator, C25SIM allocates blocks of your PC's

memory to function as blocks of virtual C25 program memory. Likewise, when

your code starts using data memory, C25SIM allocates PC memory for virtual

data memory. If you want to write or read C25 memory directly, you can use

program_memory_address() and data_memory_address() to convert C25 addresses

to PC addresses (pointers to integers).

Alternatively, you can use write_program_memory() and read_program_memory()

to access program memory, and write_data_memory() and read_data_memory() to

access data memory. The only difference in using these functions is that

they will warn you if you are reading uninitialized memory, or writing

program memory (a potential bozo no-no in the C25's Harvard architecture).

If you want to use write_program_memory() to write to program memory without

getting warnings, set warn_progmem_writes to zero. By default it is set to

one, and is only cleared temporarily when hex files are loaded.

############################################################################

5. Starting the simulator, stepping the simulator

extern void initialize_simulator(void);

extern void advance(void);

initialize_simulator() sets all the simulator's registers, flags and

variables to zero, except for mp_mc_mode and warn_progmem_writes which are

set to one. It also sets up the table which the simulator uses to

disassemble code as it executes.

! ! ! ! ! ! ! !

NOTE: You *MUST* run initialize_simulator() before stepping the

simulator. Otherwise the simulator will not recognize any C25

instructions.

! ! ! ! ! ! ! !

advance() causes the simulator to execute one instruction.

When executing any repeated instruction, the "steps_taken" variable will

only be incremented once. The "cycles" variable will be incremented thru the

correct number of processor cycles that would physically occur.

############################################################################

6. Loading files, using global symbols

extern void load_hex_files(char *word);

extern void load_map_file(char *word);

extern int lookup_symbol(char *name);

extern void print_matching_symbols(int d);

load_hex_files() loads executable C25 code into the simulator.

If your code is in high and low Intel hex files (e.g. for programming 8-bit

eproms), with the filename extensions "HI" and "LO" respectively, then you

can load both files by calling load_hex_files() and passing it the prefix of

the filename (everything up to, but not including, the period).

If your code is a single Intel hex file with 16-bit data, with highbytes

followed by lowbytes, you can load it by specifying an entire filename, with

the only restriction that the filename extension must be "HEX".

If your code was linked with TI's C25 linker, and you produced a map file,

you can load the names and values of all globally defined symbols (routines,

labels, and variables) using the function load_map_file(). Pass the filename

as an argument; if no filename extension is given, the function will assume

it is "MAP".

Once the symbols are loaded, you can look up the value of a symbol with the

function lookup_symbol(), which takes the symbol's name as a string and

returns the value as an integer. Another handy function is

print_matching_symbols(), which accepts an integer and prints the names of

all symbols with that value. This can be handy for identifying your location

in some code.

############################################################################

7. Breakpoints

The current version of C25SIM supports 100 breakpoints.

extern void add_breakpoint(int d);

extern void delete_breakpoint(int i);

extern void clear_all_breakpoints(void);

extern int go_til_breakpoint(long int n);

add_breakpoint() will set up a breakpoint at an address in program memory,

and delete_breakpoint() will delete a breakpoint at an address, if one

exists. clear_all_breakpoints() will delete all existing breakpoints.

The function go_til_breakpoint() accepts a numerical argument n. If n = -1,

then go_til_breakpoint runs C25 code until either of two conditions:

1. the simulator hits a breakpoint,

in which case go_til_breakpoint returns -1,

2. or a key is pressed on the PC's keyboard,

in which case go_til_breakpoint returns the ASCII

value for the key that was pressed.

If n is not -1, then go_til_breakpoint will also terminate on a third

condition:

3. n instructions have been executed,

in which case go_til_breakpoint returns -2.

############################################################################

8. Resets and Interrupts

extern void c25_reset(void);

extern void c25_interrupt(int n);

These functions are used to reset or interrupt the simulated C25. The C25

has eight kinds of interrupts:

/* Interrupt types */

enum int_type { RS, INT0, INT1, INT2, TINT, RINT, XINT, TRAP };

The RS interrupt is simply a processor reset. The TRAP interrupt is normally

invoked by a software instruction. RS and TRAP are non-maskable interrupts.

The timer can be configured in the C25 code to provide timed interrupts

(TINT).

The simulator does not attempt to model the C25's on-chip serial ports. You

can model a receive-buffer-full event by loading a value into the "drr"

variable and then performing a RINT interrupt. Likewise you can simulate a

transmit-buffer-empty event by triggering a TINT interrupt. If you are using

the serial ports, your code will probably want to respond by loading the

"dxr" variable with data to be shifted out.

If an interrupt is not enabled when c25_interrupt() is invoked, a flag is

set to record the fact that the interrupt is pending, and the interrupt is

triggered as soon as it is enabled.

############################################################################

9. Reporting processor status

void short_report(void);

void long_report(void);

These functions respectively print out short and long reports of processor

status. A short report shows the program counter and the instruction

currently being executed, both in hexadecimal, and the number of

instructions executed and processor cycles both in decimal.

PC:0096   Instructions executed: 622   Processor cycles: 1000

Instruction: 70A0    SAR

A long report shows the same information from the short report, plus the

values of the C25's registers and flags. All registers are shown in

hexadecimal, except for ARP, DP, ARB and PM, which are shown in binary, with

a decimal value in parentheses. Single-bit flags are shown as 0 or 1.

Sixteen-bit registers are shown as four digits of hex, and 32-bit registers

are shown as eight digits of hex.

PC:0041   Instructions executed: 154   Processor cycles: 300

Instruction: FE80    CALL

ACC:00000000 ARP:000(0)

ARs: 000A 0403 0000 0000 0004 0007 0000 0000

Hardware stack: empty

OV:0 OVM:0 INTM:0 DP:000001000(8)

ARB:101(5) CNF:0 TC:0 SXM:0 C:0 HM:0 FSM:0 XF:0 FO:0 TXM:0 PM:00(0)

T:0000 P:00000000

DRR:0000 DXR:0000 TIM:00D0 PRD:01F4 IMR:0008 GREG:0000