Documentation for GBSIM 0.99

============================

GBSIM is a GameBoy emulation for the PC.

-- PLEASE NOTE: ---------------------------------------------------------------

This program is just an experiment and only interesting for freaks with

hardware knowledge.

I decided to stop the development of it, because there is another (better)

emulation of the GB available (VGB by Marat Fayzullin).

I'm going to port the debugger of GBSIM to C and add it to VGB, but until

this is done, GBSIM is the easiest way to debug GameBoy-programs.

I'm going to release the code (TP6.0 + ASM) if enough people are interested.

Please write to:

Jens Ch. Restemeier <jchrr@hrz.uni-bielefeld.de>

If you want to send comments or suggestions for the GB-debugger, or you are

completely bored, you may write, too.

A added a music-record function, and changed the extension from ".GBS" to

".GB".

-------------------------------------------------------------------------------

GBSIM doesn't expect a "specially optimized" configuration, as long as EMM386

is installed with EMS. I think 1 MB EMS is enough.

You need a binary game-image to see GBSIM work, but don't ask me about how to

get one ...

Copy everything into a directory and start it. GBSIM will load the file

given as parameter. Without a parameter, it will look for TETRIS.GBS.

You'll see the debugger (See below for commands). Press L and look.

(See below for keyboard controls).

If present, it loads a label file (<parameter>+'.L'). Look into GAMEBOY.L

for syntax (GAMEBOY.L is always loaded !).

Amazing, isn't it ?

Don't be surprized, if your PC locks up, while or after GBSIM runs. I do really

try to clean up, when GBSIM exits.

This version isn't optimized for speed, it's just a quick & dirty

"It's-possible-!"-beta-version. Don't expect, that it runs with MS-Windows [95].

It works on a 386/486/Pentium, maybe you have to switch off the cache, remove

any TSRs, turn off your coffee-machine, darken the room, cast a magic spell,

light a black candle, burn incense or just wait for the next release...

It DOES NOT work on a 286, because I'm using 32-bit registers.

==============================================================================

Why did I waste my time developing a GB-emulator ? Well, first of all, it

wasn't _that_ work. The main coding was done at just one weekend.

I met people, who developed devices to read GB-cartridges (to C64 and Amiga)

and I built this for PC from the "c't Printer-Server" with two 8255 PIOs. (Then

a friend built this for the PC-printer Port. Needs 9 74xx logic chips.)

I heard that a Z80 is working in the GB, I tried to decode it with an

Z80-Disassembler on CP/M and the code seemed to be "runable".

So I wrote a simple Z80 disassembler with label support and tried to decode

Tetris. That did not work, so I modified the disassembler, until the code

seemed to be good. After discovering some internals (new opcodes, IO, Video), I

closed the project. Then, month later, after closing my first 100% assembler

project, I had a great idea how to write the CPU emulator, and a day later

I had the title of Tetris working.

The final improvements where month ago, I've got no new information, so

I think: Show it to everybody, maybe somebody gives me THE information

to finish it.

I got a "GB-FAQ" recently, giving me all the info I need ! Shit, this Guy

developed a GB-Emulator, too. While I was first, there is no way to proove

it :-(

But now I have completed the CPU, and a documentation of the missing

registers...

I'm interested in internal information of other hardware, too, like SNES,

PC-Engine, Mega Drive (Genesis), NES, Atari VCS, Vectrex, PSX, 32x, Saturn,

Jaguar, Nintendo 64, Master System, FM-Towns, Super TwinCom...

==============================================================================

Commands:

=========

Debug-Mode:

===========

All numbers must be entered in HEX.

T       => Trace

S       => Step

H       => Here         Run, until here

G       => Goto         Set Cursor to

W       => Write        Set the value for the joypad:

0 => nothing

1 => A

2 => B

4 => Select

8 => Start

16 => right

32 => left

64 => Up

128 => Down

N       => Reset

M       => Memory  Set the position of the memory-window

R       => Run     The code is emulated, until an illegal opcode or the

HALT - opcode appears

I       => Look, whats on the screen (the GameBoy's of course...)

L       => Look-Mode    Show the screen while emulation.

O       => Output       Write a value to an adress.

Delete  => Set the actual opcode to NOP.

+       => Log $FF00-$FF4F (IO-Area) to \GBSIM.LOG (Hex)

B       => Set/Clear breakpoint ( max. 9)

C       => Clear all breakpoints

F       => Find next breakpoint from cursor

F1      => Save memory+registers (Snapshot)

F2      => Load memory+registers

F3      => Save Screen (IFF-Format, extension will be added)

Deluxe-Paint 2 PC does not like those Files, but

Graphics Workshop (V7.0f DOS) can show/convert them !

This function will be good for making solutions or magazines.

F5      => ROM information, shows the Logo

ESC     => Exit the emulation

Down    => Move the cursor down

(Left/Right => Inc/Dec cursor-adress)

Look-mode:

==========

Use a 4-button joystick

Breakpoints don't work here.

Breaks on errors, ESC or HALT-instructions

==============================================================================

Technical info:

===============

CPU:

====

The Gameboy is based on a Z80-like CPU. Some opcodes are left out,

replaced or relocated. There is no second register-set and IX and IY are

missing. The $ED-Opcodes are killed.

The "Halt" instruction seems to be changed. If the emulation breaks there,

kill it. I could fix this, but my breakpoint-stuff works with "Halt", and

I'm too lazy to rewrite it. "Halt" is normaly coded as "ld (HL),(HL)", other

instructions of this kind seem to be changed, too: Tetris uses "ld b,b".

Any ideas ?

MEMORY:

=======

The first 32K of Memory are on the Cartridge, then there are 8K video-RAM.

From $C000 are 8K RAM. There is a "special" RAM-area from $FE00-$FFFE, while

the area from $FF00-$FF4f holds I/O registers.

The GB uses an MMU for Cartridges with more than 32KB on it. It switches the

memory configuration on a memory write to $2xxx. This adress is not full

decoded: If /WE and ((address AND $3000)=$2000) the MMU selects a new page.

Extension-memory banking is not done, yet.

I/O:

====

The IO-chips are spread from $FF00-$FF4F. I found the joypad-port at $FF00 and

the display-options from $FF40 to $FF4B.

VIDEO:

======

The font and the screen are stored in video-RAM. The font can have 3*128 chars

and there are two 32*32 screens (20*18 visible). The GB has 40 Sprites, 8x8 or

8x16 great. They are normal chars from the font.

For more details read the GB-FAQ !

==============================================================================

The emulator is a Turbo Pascal (6.0, I don't like 7.0) program with a

Turbo Assembler kernal for CPU-emulation.

==============================================================================

Plans for the final Version (which will never get done...):

- 100% Assembler, P-Mode

- Cool GUI

- Better name than GBSIM: something like Super-Boy or Mega-Gameboy or

Masterboy or Home-Boy or PC-Boy or Brain-Boy ...

- Trainer (Lifes, Energy, Level), like Action Replay

- Sound (SB, GUS)

- support for the Super-GB games (showing the Frame, Colors)

- IPS-Support, integrated cheats/games database

- PC-Joypad/SNES-Joypad (sure)/Genesis-Joypad (maybe) support

- PC<->Gameboy link option

- Double GB-emulation (for two player action on REAL fast machines)

------ Cut here --------

Plans for the VGB-debugger:

1. everything, that GBSIM can:

- single step execution

- memory manipulation

- breakpoints

- RAM/register snapshot

- intruction "NOP"-out

- screen snapshot (I think about using Targa instead of IFF)

- IO-Dump to file

- label-support

2. what GBSIM can not:

- source level debugging (ASM and C)

- register manipulation

- direct support for TASM and GBC symbols

- disassemble into file

- label manager (to add/modify labels)

I'm planning to write it "system independent" like VGB: all you need to port

it, is to write a "driver" for your system.

==============================================================================

History:

Beta 1:

- Quick and dirty first release (SSSSSLLLLLLOOOOOWWWWW).

- 286 Code

- internal debugger

Beta 2 (only shown/given to some friends):

- MS-Windows test: IT WILL NOT RUN WITH WINDOWS ;-)

- CPU emulation uses 386 code

- New CPU-engine.(Faster)

- Save+Loadgame

- Breakpoints

- Better keyboard-control in Look-mode

- EMM386.EXE used (VCPI:Breakpoints,Expanded Memory).

Beta 3 (only shown/given to some friends):

- Grafix-engine reworked (80% of display-options used, the rest would need a

complete rework)

- CPU-Engine reworked (faster)

- additions to the debugger

Beta 4 (only shown/given to some friends):

- Reworked IO-emulation

Beta 5

- New GB-MMU emulation (other EMM386-Feature used, now it is impossible

to write into the ROM-area).

Beta 6

- CPU-Engine optimized (smaller, a little faster)

- Skips Smartcard-header, if present

- Auto-MMU-Patcher

- IO is done in Assembler !

- new GFX-engine !!!!!

- Screen Grabber

Beta 7

- even newer GFX-engine !

- killed keyboard driver

- added IBM-joystick driver, SF-pad driver comming soon !

- you'll see the Nintendo-Logo when getting cartridge-info !

Beta 8

- Lots of debugging and rewriting

P.S.:

- Does anybody know, how the "Super Gameboy" works ? It's a complete GB

without a display. The SNES seems to read the LCD-datastream and gives the

joypad-info back. It should be possible to build a PC-adapter for it (ignoring

the SF-ROM), and running it from DOS/OS2/Windows/Linux (OR an Amiga-adapter to

run it from the Workbench OR a [Power]Mac adapter to run it with MacOS).

You'll need to decode it's registers to host-IO and a Smartcard.

Maybe I'll try it, when I've got a Super-GB. What I know is this:

Super Donkey Kong checks it's presence by writing 030h to 0ff00h, then reading

it back: 0fxh: GB, 03xh: Super GB. It tries some serial transfer on the

joypad register: 030h Stop-Bit, 020h=Hi, 010h=low. The frame seems to be

transfered through the VRAM (Had no time to check this).

- I need a PAL/GAL-listing for a GB-MMU replacement. I don't like killing

cartridges to get a MMU.

- What do you think about an emulation of the "old" Nintendo ? Need Info !

- What do you think about an emulation of the "old" SEGA Master ? Need Info !

==============================================================================

IMPORTANT ! IMPORTANT ! IMPORTANT ! IMPORTANT ! IMPORTANT ! IMPORTANT ! IMPORT

==============================================================================

Use GBSIM at your own risk.  There is no warranty for the program of any kind.

I don't  take  responsibility  for  any  damage  caused by the program, either

direct or indirect, physical or psychical, to your hardware, software or soul.

==============================================================================

Hey, if you didn't notice it:

GB is short for Nintendo GAMEBOY

SF/SNES is short for Super Family Computer/Super Nintendo Entertainment Systen

TP is short for Turbo-Pascal 6.0

Genesis = "Mega Drive"

Nintendo, FamiCom and Gameboy are registered to Nintendo corp.

Please excuse my wired ideas about english grammar and oatogrfi.

Fuck for Windows [NT/95].

You may give this stuff to anybody you like. (If anybody likes to get it.)

-------------------------------------------------------------------------------