---- The SPECTRUM connection ----

Written by H. de Groot.

The programs presented here are used to transfer data from a ZX Spectrum

computer to a PC and from a PC to a ZX Specturm. The ZX Spectrum must have

a ZX Interface I connected. The ZX Interface I has a RS232 interface which

will be used. This interface also contains a ROM with programs to enable

saving and loading via RS232.

These programs are Public Domain, and because of that I will not be liable

for any damages resulting from use of this software. I suggest to leave the

source and this file with the programs, you got it this way so please if

you distribute it, leave it this way!

All programs are compiled with TURBO-C 2.0.

The files IBMCOM.C and IBMCOMT.C are not mine. The authors notes are at the

top of these files. These files are retrieved from SIMTEL20, an American army

system at White Sands Missile Range, which contains public accesible public

domain and shareware programs. I changed IBMCOM.C to work with the ZX Specturm

interface I (which makes very much use of hardware flow-control, which wasn't

fully implemented). I did not mark the changes because I made them before I

decided to give away copies of this program.

The programs included are:

IBMCOMT - Interactive program to talk to the Spectrum via RS232

READRS  - Read from RS232 port and write data in a file

WRITERS - Write to RS232 port with data comming from a file

SLOAD   - Load a program from a Spectrum via RS232

SPCONV  - Convert snapshot files (.SNA, .SP, .Z80 and RAW)

GETA    - Read a file from the EDITAS Assembler and convert to ASCII

PUTA    - Read an ASCII file and send it to the EDITAS Assembler (converted)

The READRS and SLOAD programs seem to have the same functionality, but this is

not the case. READRS just reads everything until the user stops the program.

SLOAD houever will recognize the header of the loaded data and will load the

data that follows.

All programs, except SPCONV, can be set up by means of a -s flag. The setup

will be used to select the apropriate COM port. Note that compressing the

program with LZEXE will cause the setup routine to malfunction. If you

recompiled the program than the first time you run the setup the program will

take longer to store the setup value in the program. This is because it has

to search for the patch area the first time. The location of the patch area is

stored in the file, so the next time the setup is used the program doesn't

have to search. The files distrubuted are setup once. The COM port is set to

COM1.

IBMCOMT

This program is started without parameters. Characters written will be send

to the selected COM port. Characters received are put on the screen. To stop

the program type ALT-X.

READRS <file>

This program reads from RS323 and puts the data in the named file. To stop

reading hit ALT-X.

WRITERS <file>

This program writes to RS323 and gets the data from the named file. To stop

reading hit ALT-X. The program will stop automaticly if all the data from

the file is send.

SLOAD [-h] <file>

This program reads from RS323 and puts the data in the named file. To stop

reading hit ALT-X. The program will stop if all the data is read from the

RS232 link. The data must be send by the spectrum via the command:

SAVE *"b"

Extention to this command are as usual, but there is no "name" part in the

command. Examples:

SAVE *"b"LINE 10  - save basic program automaticly

start running at line 10 after

load.

SAVE *"B"CODE 40000,256 - save peice of memory

The header will be put in the destination file toghether with the data. One

can send the data to the Spectrum again with the WRITERS command. At the

Spectrum side type:

LOAD *"b"

to load the data. Note that LOAD also has no "name" part. Extentions to

LOAD are the same as SAVE, so to load a CODE file type:

LOAD *"b"CODE

SPCONV <infile> <outfile>

Spconv is a utility to convert between various snapshot formats. Snapshot

files are created by various spectrum emulators. SPCONV converts between .SNA,

.SP and .Z80 files. The conversion to/from .SP files may not work always

however, it is not clear to me how IFF, IFF2 and the interrupt mode are stored

in the .SP file. Cashing of a converted image may also be caused by bugs in the

SPECTRUM emulator. Converion to .SP will work if the image is converted from a

RAW image (see later)

Normaly the <infile> and <outfile> are not of the same type. If they are you

will get a error message, unless both files are of the .SP type. In that case

the converter will convert an old style .SP file to a new style .SP file or form

a new style .SP file to an old style .SP file, depending on the input. The old

style .SP file is used by VGASPEC, with seems to be an unauthorised version

of the spanish "SPECTRUM". The "SPECTRUM" will use the new style .SP files.

Conversion between .SNA and .Z80 and from .Z80 to .SNA should be no problem,

both formats are sufficiently described in the documentation of JPP (the

norwegian emulator) and Z80 (the dutch emulator).

Spconv can also be used to convert a raw spectrum image to a snapshot file.

The image has to be captured with SLOAD. At the Spectrum side the the command

SAVE *"b"CODE 16384,49152

HAS to be used. Convert will convert the file is such way that it will continue

in the basic interpreter. so

SAVE *"b"CODE 16384,49152:PRINT "Hello, World!"

will after conversion to a snapshot file and loading it in the emulator, begin

with printing "Hello, World!".

Example: Converting a .SNA JPP file to the .Z80 Dutch emulator:

spconv game.sna game.z80

Thanks to: Arnt Gulbrandsen (author of JPP)

Ramco v.d. Woestijne (supplied new .SP format)

G.A. Lunter (author of Z80)

Pedro Gimeno (author of SPECTRUM)

for suppling information and for creating these spectrum emulators.

GETA <outfile>

This procedure will read an assembly file form the RS232 port. The assembly

file is produced with EDITAS. In editas type

SAVE *"b"T

The assembler file will be read from the RS232 port, and converted to

ASCII. The ASCII text will be stored in the named <outfile>. The layout

of the file will be:

<label><tab><opcode><tab><operand>

PUTA <infile>

This procedure will write an assembly file to the RS232 port. The assembly

file will be suitable for EDITAS. In editas type

LOAD *"b"

The assembler file will be send to the RS232 port, and converted from

ASCII to the layout used by EDITAS. The ASCII text will be read from

the named <infile>. The layout of the file will be:

--------------------------------------------------------------------------

AAAAA	BBBB	CCCCCCCCCC

--------------------------------------------------------------------------

where AAAAA are labels, BBBB is an opcode and CCCCCCCCCC is an operand.

A valid example of a program:

--------------------------------------------------------------------------

;

;		"Hello, world program"

;		"This program will print"

;		"Hello world to your"

;		"screen."

;

;		"start address"

ORG	40000

;

LD	HL,TEXT

LOOP	LD	A,(HL)

CP	0

RET	Z

PUSH	HL

RST	16

POP	HL

INC	HL

JR	LOOP

;

;		"Here constants are defined"

;

TEXT	DEFM	"Hello, world!"

DEFB	13

DEFB	0

;

;		"The END directive must"

;		"be the last in every"

;		"listing!"

;

END

--------------------------------------------------------------------------

The indentation may be constructed with TAB's, spaces or both.

Labels are at most 5 characters (this restriction is imposed by

EDITAS). The opcode is a Z80 instruction or an assembler directive.

An opcode is at most 4 characters long. Note that empty lines must

contain a ";" in the first column.

For more information read your EDITAS manual.

The RS232 connection.

Drawing for a COM port with a 9 Pin D connector. You need one male and one

female connector.

Both connectors shown at the wire side!

Female (COM port)                     Male (ZX Interface I)

ZDDDDDDDDDDDD?

3      ____  3                              ____

3     /   1| 3                             |1   \

3   /6   ODDD4                             | O   6\

@DDDDDO   2| 3                             |2   O  |

| 7   ODDDDDDDDDDDDDDDDDDDD? ZDDDDDDDDDDDDO   7 |

|  O   3| 3              ZD3DY          |3   ODDDDDDD?

| 8   ODDDDDDDDDDDDDDDDDDY @DDDDDDDDDDDDD-O   8 |    3

ZDDDDDO   4| 3                             |4   O  |    3

3  | 9   ODDDADDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDO   9 |    3

3  |  O   5|                               |5   O  |    3

3   \    ODDDDDDDDDDDDDDDDDDDDDDDDD?     ZDDDO    /     3

3     \____|                       3     3 |____/       3

3                                  3     3              3

@DDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDY              3

3                    3

@DDDDDDDDDDDDDDDDDDDDY

Signal names:

COM port:                                   ZX Interface I port

pin 1 (in) : Carrier Detect (DCD)           pin 1 (---) No connection

pin 2 (in) : Recieved Data (RXD)            pin 2 (in)  TX data

pin 3 (out): Transmitted Data (TXD)         pin 3 (out) RX data

pin 4 (out): Data Terminal Ready (DTR)      pin 4 (in)  DTR

pin 5 (---): Signal Ground (SG)             pin 5 (out) CTS

pin 6 (in) : Data Set Ready (DSR)           pin 6 (---) n.c.

pin 7 (out): Request To Send (RTS)          pin 7 (---) Ground

pin 8 (in) : Clear To Send (CTS)            pin 8 (---) n.c.

pin 9 (in) : Ring indicator (RI)            pin 9 (---) 9 Volt

Don't comment to me if you find the names at the Interface I side funny.

I didn't invent then, they come straight out of Clive's "Microdrive and

Interface 1 manual" page 49.

Good luck!

Henk de Groot -- groot@idca.tds.philips.nl

It is August 1992 right now, don't expect the above address to work in

the future!