Timex 2068 Emulation on a PC Using Z80

08/12/95

The following information describes the Timex 2068 emulation and Series One

interface operation.  Gerton Lunter's excellent Spectrum emulator, Z80 V3.02b,

is used with modified Timex and Series One Interface code to make it all work.

To try it, you simply replace the ROMS.BIN file in Z80-302b.ZIP with one

available for free in the EMULATOR area of QBOX-USA (file area #5).  Download

file TX2068_3.ZIP and the shareware version of Z80 V3.02b.  TX2068_3.ZIP

includes the new ROMS.BIN, an updated LAYOUT.SCR, and this DOC file.  QBOX-USA

is a free BBS system dedicated to the QL and other Sinclair computers.  It is

operated on a QL computer equipped with an IDE hardrive and a 300-14400 baud

modem.  The phone number is 810-254-9878 and it is in operation 24 hours a day.

QBOS-USA is located in Sterling Heights, Michigan.

Features now available to the TIMEX 2068 emulated in Z80.

1.  Microdrive Interface - up to 8 microdrives using an emulated Series One

interface.  CAT, FORMAT, ERASE, MOVE, LOAD *, SAVE *, VERIFY *, and

MERGE * are used with the microdrives.  You must enable the microdrives

with the F8 key.

FORMAT "m";n;"name"              This must be done to create cart.

CAT n                            n = microdrive number.

LOAD *"m";n;"filename"           "m" = microdrive channel.

LOAD *"m";n;"filename"CODE x     Loads code starting at address x.

SAVE *"m";n;"filename"LINE x     Auto starting program at line x.

SAVE *"m";n;"filename"CODE x,y   Saves y bytes at address x.

SAVE *"m";n;"filename"SCREEN     Saves SCREEN file.

ERASE "m";n;"filename"           Do not add CODE for binary files.

VERIFY *"m";n;"filename"

MERGE *"m";n;"filename"

Read Gerton Lunter's Documentation for more detailed information.

2.  Serial Port - Send/Receive (SAVE/LOAD) data/programs over the PC serial

ports; LLIST/LPRINT to the PC parallel ports; and SAVE/LOAD to disk.

You must set up the comm port, LPT port or disk file name with the F4

key.  Some of the following must have the streams OPENed first.

LOAD *"b"                Load basic files from comm port or disk.

LOAD *"t"                Load text files from comm port or disk.

LOAD *"b"CODE x          Load binary file to address x.

LOAD *"b"SCREEN          Load SCREEN file from comm port or disk.

SAVE *"b"                Send basic files to comm port or disk.

SAVE *"t"                Send text files to comm port or disk.

SAVE *"b"LINE x          Save basic prog w/auto starting at line x.

SAVE *"b"CODE x,y        Send binary file to comm port or disk.

SAVE *"b"SCREEN          Send SCREEN file to comm port or disk.

LIST #3 or LLIST         OPEN #3;"t" first.  Use comm, LPT or disk.

PRINT #3 or LPRINT       OPEN #3;"t" first.  Use comm, LPT or disk.

LIST #n                  Lists program to stream n.

INKEY$ #n                Returns single character string, if avail.

INPUT #n;x               Inputs variable from stream n.

PRINT #n                 Outputs print sequence (...) to stream n.

CAT #n;x                 Catalog of Microdrive x sent to stream n.

Read Gerton Lunter's Documentation for serial port emulation/operation.

3.  Design your own command - feature of the SERIES ONE interface.  Machine

code knowledge is required.  If possible, refer to the SPECTRUM MICRO

DRIVE BOOK by Dr. Ian Logan for details.

4.  CLEAR # - Reset all channels to original and reclaim all memory used by

the channels.

5.  CLS # - Restores screen attributes to the original values.

6.  OPEN #n;"x" - Links specified channel "x" with stream n.  Start of basic

memory moves up to make room for channels.  Channels are:  "m" for

microdrive, "b" or "t" for serial port, and "n" for network.  The

network feature is not supported.

7.  CLOSE #n - Unlinks any channel from stream n and reclaims memory used by

it.

8.  48k snapshot save - Saves the memory state of the Timex 2068 to disk in

a compressed file with a filename you select and an extension of .Z80.

Note:  you cannot normally load Spectrum snapshot (.Z80) files to this

Timex 2068 emulation and the reverse is also true.  Memory organization

is similar but not the same.

Features not in Spectrum but working in the TIMEX 2068 emulation.

1.  ON ERR - Traps errors and redirects program execution.  Example:

10 ON ERR GO TO 1000

2.  DELETE - Works like a normal Timex 2068.  Used as follows:

DELETE ,           Deletes all basic lines from memory.

DELETE X,Y         Deletes basic lines from line X to line Y.

3.  FREE - Prints out the available basic memory.  Used as follows:

PRINT FREE

4.  RESET, RESET *, RESET #n - Works just like the normal Timex but is not

very useful.  Requires firmware written to take advantage of it.  It

could have worked much like CLOSE #n does in the Series One interface.

These functions are displayed on the new LAYOUT.SCR file in TX2068_3.ZIP

All features of the Timex 2068 emulation work except:

1.  STICK - Hardware support in the Z80 emulator is not available for the

joy stick as provided for in the Timex.

2.  SOUND - Hardware support in the Z80 emulator is not available for the

Timex sound chip.

3.  Access to the XROM memory and its TAPE routines cannot be done because

the rom is not provided for.  The same is true for the DOCK bank memory.

This makes AROS and LROS programs unavailable as well as any of the

extra VIDEO modes (normal display only).  The second display file cannot

be opened because the Z80 Spectrum emulator does not support it at

address 6000 hex.  Just imagine if it were supported, the 64 and 85

column potental of the Timex would really look good with a VGA display.

4.  SAVE/LOAD/MERGE/VERIFY - Emulation of TAPE commands to disk is not

supported with the TAP feature.  However, the tape commands can be used

to load files through the parallel port only when a file named SA_LD.BIN

(included in TX2068_3.ZIP), is loaded to memory and you are using a

registered Z80 V3 emulator (see note below) and a fast computer.   To

obtain a registered copy, you must write to Gerton Lunter.  Please read

the instructions in the shareware Z80-302b.ZIP file.  The TAPE commands

will only work in the REAL MODE and only with the parallel port

connected to a tape cassette interface.  Use key F7 to set up loading

from the LPT port and not from DISK.  The file, SA_LD.BIN, must be

loaded to ram from an emulated Microdrive or Serial Port to address

25282 decimal (62C2H).  The normal location of the TAPE routines is in

the XROM (see #3).  This file is designed to be loaded from disk as

though it were from an emulated comm port (key F4, RS232 Input).  Load

it as CODE, from Timex basic, to address 25282 as in:

LOAD *"b" CODE 25282.       (you must include 25282 with this file)

Save it to a microdrive file, if desired, as in:

SAVE *"m";1;"SA_LD" CODE 25282,1330

Or, save it with the name "run" if you want it to be loaded when the

keyword RUN is entered after initializing the emulator.  Refer to

Gerton's documentation about the save/load syntax.  Again, you must use

REAL mode (F6) and your emulated Timex speed must be between 100% and

110% in order to load tape programs through the parallel port (requires

a fast 386 or higher).  Around 105% was best on my computer.

If the SA_LD.BIN code is not loaded, any tape commmand written in basic

will pass syntax.  But during execution, an "Invalid I/O device" error

will be generated.  After loading the tape routines, the LOAD "" or LOAD

"filename" will work properly as long as no code address nor length is

used.  If there are numbers, such as an auto starting line number, a

"Nonsense in Basic" error will be displayed because the five byte

floating point form of these numbers has not been added.  To get around

this problem, use VAL "number".  To avoid this condition entirely, make

sure the file, SA_LD.BIN has been loaded before using the tape commands.

One final note about "MERGE": the SA_LD.BIN file must be loaded in order

to use the "MERGE" command for microdrive files or an "Invalid I/O

device" error will be displayed.  The microdrive "MERGE" command uses

some code in the tape routines for MERGEing operations.  Note: the

Series One code will not allow MERGEing an auto starting basic program.

In addition, I have not found the MERGE operation to be very reliable

with large programs in either the Timex or Spectrum emulation.

NOTE:  At the time that this is written, I do not have a registered

version of V3.02b.  I believe that the Tape routines will work

because of the way Z80 responds with the tape commands after

SA_LD.BIN is loaded and because it worked just fine in the

version 2 emulator.  However, I can't be sure until I have the

registered copy to test this.

5.  The RAM RESIDENT CODE is not in memory.  There is no XROM to copy it

from so the space is free for the use of other code.  However, this

space is also used by the special tape routine, SA_LD.BIN.  See #4.

6.  The Timex variable, Physical RamTop (P_RAM) has been relocated to 5CCCH

and the first Series One interface variable (FLAGS3) is at 5CB4H.  The

other Timex Series One interface variables are located starting at

6856H.  In contrast, the Spectrum Series One interface variables start

at 5CB6H.  The Series One interface variables cause the start of Basic

to move up by 58 bytes.

7.  To OPEN a channel, you must use OPEN #n;"x".  The letter n is the number

of the channel and "x" can be  "t", "b", "n", or "m".  See Gerton's

documentation.  The important item here is that you must use a semicolon

and not a comma in the command like the Spectrum allows.  The Timex

syntax check passes the comma, thus not paging in the interface one rom.

When a comma is used, an "Invalid I/O device" error is generated by the

Timex error routines and the command is never acted upon by the Series

One interface.

8.  BASIC programing starts at address 26709 decimal, before enabling the

Series One interface.  It normally starts at 26710 in the Timex.  I had

to do this because of the way the Series One interface code expects

memory to be organized.  Between the Spectrum and Timex computers, there

is a one byte difference in length from the start of Channels to the

start of Basic.  If I can figure out how to work around this difference,

the start of basic may be changed back to 26710.  The way it is now may

cause problems with some programs.

One such program may be TIMACHINE.  TIMACHINE loads OK but basic

programs will not load from the Series One interface.  Basic programs

can only be loaded from the Tape interface.

You may run into another problem that keeps some programs from loading

properly.  Some programs will not load correctly if the Series One

interface variables are initialized and the start of Basic is moved up

to accommodate them.  One such program is WINDOW PRINT 32.  The solution

to this is complicated but I will try to explain.  To load the Tape

routines, you must enable the Series One variables and extended

channels.  But, this is why WINDOW PRINT 32 fails to load properly.  The

Series One variables must be removed and the channels restored back to

normal.  We can do this with a NEW command but the Tape routines must be

protected by loading them above RAM TOP first.  Load the tape routines

to address 60000 and do CLEAR 59999.  Next, do a NEW to remove the

Series One variables.

10 CLEAR 59999

20 LOAD *"m";1;"SA_LD" CODE 60000 : REM  from mircodrive, F8 menu

30 REM  LOAD *"b" CODE 60000 : REM   RS232 redirection from F4 menu

30 NEW

Then, using a FOR/NEXT loop, PEEK the Tape routines and POKE them back

to address 25282.

10 LET y=25282

20 FOR x=60000 TO 61329 : REM 1330 total bytes

30 POKE y,PEEK x

40 LET y=y+1

50 NEXT x

RAM TOP should be moved back up to the normal location with CLEAR 65367

or higher if desired.  Once completed, the TAPE routines are back in

place and the Series One variables have been eliminated.  Be careful not

to enable the Series One interface with a syntax error or the interface

variables will be recreated.  Hopefully, you can now load the Tape

program without problems.

I have included NORMTAPE.Z80 in the ZIP file.  This file has been saved

after the above operations have been completed.  In this file, the TAPE

code is already loaded to address 25282 decimal.  Also, the hardware has

been changed to the first selection in the F9 menu, SPECTRUM emulation

without the Series One interface.  This file can be used to load other

programs, through the parallel port, without having to go to the effort

explained above.  Further more, the Series One interface cannot be paged

in to add the extra interface variables when a syntax error occurs.  You

will lose the microdrive features, but, if you are using a registered

version of the emulator, you can save and load with cassette tape

through the parallel port.

9.  OUT 244,n will cause the border to change color.  Many Timex programs

have this instruction written in machine code to ensure that HOME bank

memory chunks are enabled during program execution.  Short of changing

the program code or Gerton Lunter rewriting his emulator, there is no

solution for this.  Note:  when low, each of the 8 bits sent out port

244 enables 8k of HOME bank memory.  Or, when high, each bit enables

either DOCK or XROM bank memory.  The DOCK bank is enabled when the

value of 8 bits sent out port 255 is <128.  Otherwise, the XROM bank is

enabled when the value of the 8 bits sent out port 255 is >127.  This

memory bank switching is not available in this Timex emulation.  The Z80

v3 emulator does not recognize port 244 and handles any OUT 244,n

instruction like a border color change through port 254.

10.  Most programs that I have tried have worked with this Timex emulation.

That includes HOTZ and HOTZ AROS (my favorite), as long as the XROM

bank, DOCK bank and ram resident code are not accessed.  Of course,

programs that make use of the STICK and SOUND command are not going to

work.

ARTWORX is one program that will not work properly in this emulation.

There may be many others that fail to work properly but I haven't tried

them yet.

If you are not a Spectrum Z80 emulator user, please read Gerton Lunter's

documentation carefully.  Some people find the control and setup of the

microdrives, tape interface, parallel port, serial port and floppy/hard drive

complicated from the menus in the emulator.  Be patient, it is worth it.

Another item that could be a problem for some computers is the amount of free

memory.  If you are not using expanded memory, you will need to have at least

572k of free memory to run the version 3 emulator.  Expanded memory is the

better way to run Z80 v3 because it requires less free memory and the Spectrum

128 emulation will run much faster.  If you are unable to get the emulator to

run because of insufficient memory, try launching the emulator with the -xt

switch as in: Z80 -xt.  This will reduce certain buffers in the emulator and

may make the difference if the emulator will run or not.  By the way, use only

the hardware selections of the 48k computer alone or the 48k computer with the

Series One Interface.  Other selections are unpredictable with the Timex

emulation.

If you are having problems getting this emulation to work, you can leave me a

message on the QBOX BBS, (810) 254-9878, and I will try to help.

If you can help make this Timex 2068 project better, let me know how it can be

done.  I personally would like to use OS-64 with this emulation but I don't

have the knowledge to change the Z80 emulator to make it compatible with the

XROM bank, the DOCK bank and the two displays files.  At best, this emulation

is only a compromise, but, if enough USA users will become registered users of

Z80, maybe Gerton Lunter will consider making his emulator FULLY Timex

compatible.  So, when you register, write that you would really like Z80 to be

Timex compatible.

Keith Watson

(810) 286-9049

41634 Amberly Dr

Clinton Twp, MI  48038