D110 PATCH EDITOR	   by David Comeskey

This programme was written in gfa basic v2 to allow editing of the Roland D110

multi timbral sound module patch and part parameters.

The editor was intended to be first and foremost simple and quick to use so all

necessary information is displayed on screen at the same time.So a high

resolution mono monitor is required to use the editor.

To use this programme connect your d110 to your st midi in and out ports.If you

don't the programme will indicate a midi comunication error and if you choose to

continue the default parameters stored on disc will be loaded into your st.

It is possibble to use the editor with only the st midi out connected but your

own internal patch data and tone names will not be shown on screen,only the defa

ult parametrers.Any editting is done to the temporary patch area of the d110 unt

il another patch is selected in the patch directory.

When editting a parameter use the left mouse button to decrease and right to

increase the said parameter value.

SCREEN AREAS

On the top left of the screen is the function area each funtion operates as

follows:

MIDI THRU'-Key on/off,pitch bend and modultion recieved at the midi input of the

st will be transmitted to the st's midi out port on the midi channel of the part

the mouse pointer is indicating on the part parameter area at the bottom of the

screen.If the mouse is moved to another part the notes being played on the

previous part will be muted and new notes will be transmitted on the midi

channel of the new part.If no part is indicated then no messages recieved will

be transmitted.

OPTIONS-Currently there are five options the first,the CONTROL CHANNEL which is

the midi channel that patch changes are transmitted on,this should be set on the

d110 to match the one selected in the editor.This allows the editor to select

the corresponding patches on both the editor and the d110.although this isn't

really necessary for editting.

The second option is TONE LIST which gives a list of the tone names either the

ones recieved from the d110 or the default names loaded from disc.

The third option is READ DOC. FILE this allows the user to read the doc file

from within the programme.

The fourth and fifth options are RHYTHM EDITOR and TONE BANK MANAGER neither

function has been implemented yet(23-5-91).

SEND PATCH-Sends either selected patch or all 64 patches in the st to the d110

thus the corresponding patch data in the d110 will be overwritten.

UNDO EDIT-This allows the recovery of one parameter or all parameters in the

patch currently being editted.

GET PATCH-Allows one or all patches or the current tone names to be read from

the d110.

COPY/SWAP PATCH-Allows manipulation of single patches within the patch directory

LOAD-Allows loading of patch data and/or tone names from disc.

SAVE-Allows saving of patch data and/or tone names to disc.

QUIT-Exit to desktop.

PATCH DIRECTORY-Lists patch names.The parameters of the one highlighted are

displayed in the patch and part parameter areas.When a new patch is selected

the patch data of the old patch is sent to the d110 and the new patch's

parameters are displayed.

PATCH PARAMETERS-Are the parameters which affect all parts with th exception of

master tune,which applies to all patches and is not saved with the patch data.

PART PARAMETERS-Are the parameters which affect only one part individually.

n.b for detailed information of these parameters please consult your d110 manual

FUTURE UPDATES-I am currently working on upgrades to this first version of the

d110 patch editor programme.These include a rhythm editor and tone bank manager

If you have any ideas or criticisms or have found any bugs please contact me at

the address given below.

OTHER PROGRAMMES-I am also planning to write a patch editor for the Roland u110

module.If you are interested or have any ideas again please write to me at the

address below.

DAVID COMESKEY   23-5-91

DAVID COMESKEY

1563 DUMBARTON ROAD,

SCOTSTOUN,

GLASGOW G14 9XF.

UNITED KINGDOM.

END OF FILE