D110 PATCH EIDTOR INSTRUCTIONS

SETTING UP

1.CONNECT THE MIDI OUT PORT OF YOUR ST TO THE MIDI IN OF THE D110 AND THE MIDI

IN OF THE ST TO THE MIDI OUT OF THE D110,USING STANDARD MIDI CABLES.

2.SWITCH ON THE D110 AND CHECK THAT THE EXCLUSIVE UNIT NUMBER OF YOUR D110 IS

SET TO 17(THE DEFAULT).DO THIS BY PRESSING THE SYSTEM BUTTON ONCE AND THE

PARAMETER/GROUP " ^ " BUTTON THREE TIMES.THE D110 DISPLAY SHOULD THEN SHOW:

"Exclu Unit#=  17".IF THE NUMBER IS DIFFERENT CHANGE IT TO 17 USING THE VALUE

/NUMBER BUTTONS.ONCE THIS HAS BEEN SET THERE IS NO FURTHER NEED TO TOUCH THE

D110.

3.RUN THE PROGRAM BY DOUBLE CLICKING ON THE D110ED.PRG ICON.THE PROGRAM WILL

START BY LOADING TONE NAMES AND PATCH DATA FROM THE D110 VIA MIDI.IF YOU GET A

"D110 IS NOT RESPONDING MESSAGE" THEN CHECK THE CONNECTIONS MADE IN STEP 1.IF

YOU CHOOSE TO CONTINUE THE PROGRAM WILL LOAD DEFAULT.DTN AND DEFAULT.D11 FILES

WHICH SHOULD BE PRESENT ON THE DISC.IF THEY ARE NOT THE EDITOR WILL INDICATE

THIS AND GIVE YOU THE CHOICE TO RETRY OR TO CONTINUE.

EDITTING

ONCE THE MAIN SCREEN COMES UP PATCH NO.1 (I-11 IN ROLAND SPEAK) IS SELECTED

THE "EDITTABLE" PARAMETERS ARE HIGHLIGHTED BY MOVING THE MOUSE POINTER OVER

THEM.BY PRESSING THE RIGHT MOUSE BUTTON THE PARAMETER VALUE IS INCREASED

PRESSING THE LEFT MOUSE BUTTON DECREASES THE PARAMETER VALUE.

SELECT A PATCH TO EDIT IN THE PATCH DIRECTORY BY CLICKING ON IT.THE PARAMETERS

OF THIS PATCH ARE DISPLAYED IN THE PATCH PARAMETER AND PART PARAMETER SCREEN

AREAS AND EDITTING CAN TAKE PLACE.

IN EDITTING ONLY THE TEMPORARY PATCH MEMORY AREA OF THE D110 IS CHANGED ALTHOUGH

ALL EDITTED PATCHES ARE RETAINED IN THE EDITOR.TO UPDATE THE D110 PATCH MEMORY

CLICK ON THE SEND PATCH BOX IN THE FUNCTION AREA AND CHOOSE WHETHER TO SEND

EITHER THE SELECTED PATCH OR ALL PATCHES(64) TO THE D110 PATCH MEMORY.

THE OTHER FUNCTIONS ARE EXPLAINED IN THE DOCS FILE.

TO USE THE MIDI THRU' FUNCTION DISCONNECT THE MIDI LEAD FROM THE D110 MIDI OUT

PORT AND PLUG IT INTO A MIDI KEYBOARD MIDI OUT.CLICK THE MIDI THRU' FUNCTION BOX

WHEN IT TURNS BLACK IT IS ACTIVE AND NOTES PLAYED ON THE KEYBOARD WILL BE

TRANSMITTED TO THE D110 SO THAT YOU CAN HEAR THE EDITS AS YOU MAKE THEM.THE MIDI

THRU ONLY WORKS WHEN A PART IS INDICATED BY THE MOUSE POINTER.WHEN USING THE

MIDI THRU' NOTE THAT IT WILL NOT BE POSSIBLE TO GET DATA FROM THE D110 SINCE THE

ST MIDI IN IS NOW CONNECTED TO YOUR MIDI KEYBOARD.