DEF 3.0 File Specifications

Overview of changes since DEF 2.0 :

-The addition of the View Table.

-The addition of the GCP Table.

-The addition of the Image Table.

-The addition of the Color Table.

-Layer tables have been enhanced to include color, weight and

style information.

-New Header variables :

Version

Color_By_Layer

Style_By_Layer

Weight_By_Layer

-The addition of the new entity type : 3D_Face

-Bsplines and Beziers no longer add an extraneous point

DEF Import will automatically detect whether the file being

imported is a DEF 2.0 or a DEF 3.0 file. Full DEF 3.0

importation is supported.

DEF 3.0 file format

-------------------

A complete file exists of the following sections:

Header

Layer Table

Style Table

View Table

GCP Table

Image Table

Color Table

Floating Global Variables

Entity Definitions

Note:The following descriptions will be much clearer if you

print a small DEF file in DynaCADD to follow as an

example.

Header

------

Version

The Major version number multiplied by 100 plus the minor

version number. For example the current DEF version number

written by DynaCADD 2.00.02 is 300.

Note : This is the one header variable that is not optional. ALL

DEF 3.0 files must include the Version number 300.

Scale

A floating point value representing the drawing scale.

For example, 0.5 represents one half scale.

Units

An integer index to the selected unit base:

1 = inches

2 = feet

3 = yards

4 = miles

5 = millimeters

6 = centimeters

7 = meters

8 = kilometers

Width

The width of the drawing specified in the currently

selected units.

Height

The height of the drawing specified in the currently

selected units.

Line_Scale

The global line style scale.

Origin_X

The 2D drawing origin along the X axis.

Origin_Y

The 2D drawing origin along the Y axis.

Color_By_Layer

A flag determining the state of the Color_By_Layer system

variable

Off = 0

On  = 1

Style_By_Layer

A flag determining the state of the Style_By_Layer system

variable

Off = 0

On  = 1

Weight_By_Layer

A flag determining the state of the Weight_By_Layer system

variable

Off = 0

On  = 1

Comments

Any line preceded by *** is considered to be a

comment and is ignored.

*** Sample DEF 3.0 Header

Version

300

Scale

1.0

Units

1

Width

17.0

Height

11.0

Line_Scale

1.0

Origin_X

0.000000

Origin_Y

0.000000

Color_By_Layer

0

Style_By_Layer

0

Weight_By_Layer

0

*** End of Sample Header

Layer Table

-----------

The layer table is optional and is used to define all layer

parameters. A layer definition consists of the following structure:

Layer_Definition

layer index ( integer )

layer name ( string )

layer lock status ( integer )

1 = locked

0 = unlocked

layer visibility ( integer )

1 = visible

0 = invisible

layer color ( integer [ 1..15 ] )

layer style ( integer [ 0..63 ] )

layer weight ( integer [ 0..2 ] )

End

*** Sample Layer Definition

Layer_Definition

0

SAMPLE_LAYER

0

1

1

0

1

End

*** End of Sample Layer Definition

The preceding sample definition names layer 0 as SAMPLE_LAYER,

and declares it to be unlocked and visible. The layer color is 1,

using style 0 from the style table. The layer weight is set to 1.

Style Table

-----------

The style table is optional and is used to define all line styles.

A style definition consists of the following structure:

Style_Definition

style index ( integer )

style name ( string )

style local scale ( floating point )

style string ( string )

End

*** Sample Style Definition

Style_Definition

0

SAMPLE_STYLE

1.50

++++ ++++ ++++ ++++

End

*** End of Sample Style Definition

The preceding sample definition names style 0 as SAMPLE_STYLE, with a

local scale of 1.50. The style string is the line pattern,

constructed according to the manner explained in Chapter 5.

GCP Table

---------

The GCP table is optional and is used to define all

Geometric Coordinate Planes.

A GCP definition consists of the following structure:

GCP_Definition

GCP index ( integer )

GCP name ( string )

plane rotation about the Z axis ( angle : degrees )

plane rotation about the X axis ( angle : degrees )

plane rotation about the Y axis ( angle : degrees )

GCP X Translation ( floating point )

GCP Y Translation ( floating point )

GCP Z Translation ( floating point )

End

*** Sample GCP Definition

GCP_Definition

0

Top

0.000000

0.000000

0.000000

1.000000

2.000000

0.000000

End

*** End of Sample GCP Definition

The preceding sample definition names GCP 0 as Top with no rotation

from the absolute 3D plane, and an X-Y-Z translation of 1,2,0

from the absolute 3D origin.

View Table

----------

The View table is optional and is used to define all active 3D views.

A View definition consists of the following structure:

View_Definition

View index ( integer )

View GCP index ( integer )

X location of lower left corner of View bounds

Y location of lower left corner of View bounds

X location of upper right corner of View bounds

Y location of upper right corner of View bounds

X location of View origin

Y location of View origin

View Zoom factor ( floating point )

plane rotation about the Z axis ( angle : degrees )

plane rotation about the X axis ( angle : degrees )

plane rotation about the Y axis ( angle : degrees )

View Scale factor ( floating point )

Scroll distance of view origin along the 2D X axis

Scroll distance of view origin along the 2D Y axis

End

*** Sample View Definition

View_Definition

0

1

0.000000

0.000000

34.000000

22.000000

17.000000

11.000000

1.000000

0.000000

0.000000

0.000000

0.250000

0.000000

0.000000

End

*** End of Sample View Definition

The preceding sample defines view 0 as having GCP 1 as it's prime

rotation viewing plane. The bounding area of the view is encompassed

by the coordinates 0,0 to 34,22 with the origin in the center at

coordinate 17,11.

The view zoom factor is 1.0. The current viewing plane

has not been rotated from the absolute 3D plane. The scale factor is

set to 0.25 and the view has not been scrolled along the 2D X or Y

axes.

Color Table

-----------

The Color table is optional and is used to define the full drawing

color pallete.

A Color definition consists of the following structure:

Color_Definition

Color index ( integer )

Red Component of RGB color ( integer [ 0..1000 ] )

Green Component of RGB color ( integer [ 0..1000 ] )

Blue Component of RGB color ( integer [ 0..1000 ] )

End

*** Sample Color Definition

Color_Definition

0

800

800

800

End

*** End of Sample Color Definition

The preceding sample defines color 0 as have RGB components of

R:800 G:800 B:800 ( Light Gray ).

Note : Color indecies 0 and 1 are used by the DynaCADD Interface and

should normally be left undefined in a DEF file. If they

are defined they should be set to the RGB values :

Color 0 - R:1000 G:1000 B:1000

Color 1 - R:0    G:0    B:0

Image Table

-----------

The Image table is optional and is used to define all Images.

A Image definition consists of the following structure:

Image_Definition

Image index ( integer )

Image name ( string )

X location of lower left corner of Image

Y location of lower left corner of Image

X location of upper right corner of Image

Y location of upper right corner of Image

End

*** Sample Image Definition

Image_Definition

0

TestImg

2.500000

3.000000

5.000000

8.000000

End

*** End of Sample Image Definition

The preceding sample definition names Image 0 as TestImg with a

bounding area encompassed by the coordinates 2.5,3.0 and 5.0,8.0.

Floating Global Variables

-------------------------

There are nine floating global variables used when defining the

attributes of any entity definitions that follow in the DEF file.

The variables are :

Layer

Color

Weight

Style

Group

Groupend

Mask

Unmask

Viewmask

Note : Floating variables may be placed anywhere in a DEF file

with 2 restrictions.

1] They must never be placed in a Table.

2] They must never be placed where a ascii name is expected.

Layer

Layer defines the current layer index assigned to all following

entities. The range of valid layer indexes is 0 to 255.

*** Example

Layer

12

*** End of example

The preceding example sets the current layer index as 12.

Color

Color defines the current color index assigned to all following

entities. The range of valid color indexes is 1 to 15.

*** Example

Color

3

*** End of example

The preceding example sets the current color to 3.

Weight

Weight defines the current weight index assigned to all following

entities. The range of valid weight indexes is 0 to 2.

*** Example

Weight

1

*** End of example

The preceding example sets the current pen weight to 1.

Style

Style sets the current style index assigned to all following

entities. The range of valid style indexes is 0 to 63.

*** Example

Style

2

*** End of example

The preceding example set the current style index to 2.

Group and Groupend

All entities that are grouped together as a DynaCADD figure should be

preceded by the global name Group and end with the global name

Groupend. These globals must not be nested. That is no Group label

should appear between another set of Group and Groupend labels.

Note: Hatch patterns and section lines are treated as groups.

*** Example of grouping entities

Group

*** Entity definitions go here

Groupend

*** End of example

Mask and Unmask

All entities defined after a Mask command will not be displayed

by DynaCADD. Unmask is used to reset the state of the mask flag.

*** Example of masking entities

Mask

*** Entity definitions go here

Unmask

***End of example

Viewmask

Viewmask is a bitwise flag that controls the generation of 3D

entities each active 3D View.

1 = View #1

2 = View #2

4 = View #3

8 = View #4

To mask a 3D entity in view #1 and view #3 Viewmask would be

set to 5 (1+4).

*** Example

Viewmask

7

*** End of example

The preceding example set all 3D entities that follow to display

is only in view #4. To reset the display to all views, set

Viewmask to 0.

Entity Definitions

------------------

The following list contains the specification for currently

supported 2D and 3D entities. If entities types are added to

DynaCADD, this list will be updated. You should therefore read

entities in a table driven format and ignore all unsupported

entities.

All angles are given in degrees with the zero degree mark towards the

east and the positive direction increasing in a counter-clockwise

direction. Unless otherwise noted, all values are defined as

floating point variables. All entity definitions must start with

the entity name, as shown here, and finish with End.

2D Entities

-----------

2D_Point

X location

Y location

End

2D_Line

X location of start

Y location of start

X location of end

Y location of end

End

2D_Circle

X location of center

Y location of center radius

End

2D_Arc

X location of center

Y location of center

radius

start angle

end angle

End

2D_Ellipse

X location of center

Y location of center

major axis

minor axis

rotation

End

2D_Ell_Arc

X location of center

Y location of center

major axis

minor axis

rotation

start angle

end angle

End

2D_Text

X location, lower left corner of text string

Y location, lower left corner of text string

character width

character height

character slant, in degrees

character rotation

text spacing type(integer)

0 = constant(mono spacing)

1 = proportional

2 = kerned

additional spacing

name of font (i.e., LEROY_R)(string)

text string(string)

End

2D_Bspline

X location of point #1

Y location of point #1

X location of point #2

Y location of point #2 (up to 49 control points...)

X location of last point

Y location of last point

End

2D_Bezier

X location of point #1

Y location of point #1

X location of point #2

Y location of point #2 (up to 49 control points...)

X location of last point

Y location of last point

End

2D_Solid

X location of point #1

Y location of point #1

X location of point #2

Y location of point #2

X location of point #3

Y location of point #3

X location of point #4

Y location of point #4

End

3D Entities

-----------

All 3D coordinates are given in absolute 3D space and are NOT

considered relative to the current GCP.

3D_Point

X location

Y location

Z location

End

3D_Line

X location of start

Y location of start

Z location of start

X location of end

Y location of end

Z location of end

End

3D_Circle

X location of center

Y location of center

Z location of center radius

plane rotation about the Z axis

plane rotation about the X axis

plane rotation about the Y axis

End

3D_Arc

X location of center

Y location of center

Z location of center radius

start angle

end angle

plane rotation about the Z axis

plane rotation about the X axis

plane rotation about the Y axis

End

3D_Ellipse

X location of center

Y location of center

Z location of center major axis

minor axis

rotation

plane rotation about the Z axis

plane rotation about the X axis

plane rotation about the Y axis

End

3D_Ell_Arc

X location of center

Y location of center

Z location of center major axis

minor axis

rotation

start angle

end angle

plane rotation about the Z axis

plane rotation about the X axis

plane rotation about the Y axis

End

3D_Text

X location, lower left corner of text string

Y location, lower left corner of text string

Z location, lower left corner of text string

character width

character height

character slant

text rotation

plane rotation about the Z axis

plane rotation about the X axis

plane rotation about the Y axis

text spacing type(integer)

0 = constant(mono spacing)

1 = proportional

2 = kerned

additional spacing

name of font (ie; LEROY_R)(string)

text string(string)

End

3D_Bspline

X location of point #1

Y location of point #1

Z location of point #1

X location of point #2

Y location of point #2

Z location of point #2 (up to 49 control points...)

X location of last point

Y location of last point

Z location of last point

End

3D_Bezier

X location of point #1

Y location of point #1

Z location of point #1

X location of point #2

Y location of point #2

Z location of point #2 (up to 49 points...)

X location of last point

Y location of last point

Z location of last point

End

3D_Face

X location of point #1

Y location of point #1

Z location of point #1

X location of point #2

Y location of point #2

Z location of point #2

X location of point #3

Y location of point #3

Z location of point #3

X location of point #4

Y location of point #4

Z location of point #4

End