EEMRAM 2.00

Purpose

-------

EEMRAM is a small utility that can expand your DOS memory from

640K to a maximum of 736K using Enhanced or LIM 4.0 expanded

memory.  To use EEMRAM, you must have a memory board that

provides EEM or LIM4 memory.  A companion program (DERAM) can be

used to restore the "normal" 640K if necessary.  EEMRAM will NOT

work with Intel EMS products if you have an EGA or VGA video

system.

Before you run EEMRAM

---------------------

In order for EEMRAM to work, several conditions must be met.

There's more about this elsewhere in this document, but for now

run these two checks.

1. Run MS (just type "MS") and verify that the total free

memory is 655,360 bytes (640K).  If it is anything else,

EEMRAM cannot be used.

2. Run EMSINFO (just type "EMSINFO") and note the results.

If they scroll beyond one screenful, you can print the

results by typing:

EMSINFO > PRN

EMSINFO must indicate that the EMM type is either "EEMS" or

"LIM 4.x".  Also, the list of memory segments to which EMS

memory can be mapped must include the segment "A000".

If EMSINFO does not indicate that segment A000 is available, you

probably have an EGA or VGA installed in your system.  See the

section titled "If you have an EGA or VGA".

If EMSINFO indicates that EEMRAM cannot be used, and you aren't

clear on exactly why, just run EEMRAM.  No harm will be done,

and EEMRAM will display an error message indicating why it won't

work.  Then look in "EEMRAM error messages" below for possible

causes and solutions.

Running EEMRAM

--------------

Here is EEMRAM's syntax:

eemram [/#] [/S]

When run, EEMRAM will increase DOS-usable memory DOS from 640K

to a maximum of 736K.  The exact amount depends on your hardware

configuration, particularly on what type of display adapter you

have installed.  You may want to run EEMRAM via your AUTOEXEC

startup batch, ideally as the first external program run by the

batch:

echo off

cd c:\util

eemram

...

The optional /S parameter is required if you have EEMS memory

and want to be able to use DERAM (about which, more later).  It

is not necessary if you have LIM 4.0-compatible memory or don't

need DERAM.

The optional /# parameter limits EEMRAM's use of EEMS memory to

some amount less than the maximum possible.  See the next

section.

To see the effects of EEMRAM, just run MS (Memory Size) before

and after running EEMRAM.

Limiting Memory Size: /# parameter

----------------------------------

The optional EEMRAM /# parameter can be used to explicitly

reduce the number of 16K pages of EEMS memory it assigns to DOS

(see "Memory Allocation Information" for more information about

memory use).  "#" is a decimal digit from 1 to 6 that specifies

how many pages you want assigned.

EEMRAM will normally assign all contiguous pages available from

A000 to B400 (6 pages maximum, or 96K).  However, there may be

cases in which you don't want all of this used, and that's what

/# does.  An example of /# use:

C> eemram /2

The /2 means "use only two 16K pages of EEMS memory."

If you specify more pages than are available, EEMRAM will only

use the number available, in effect ignoring your parameter.

Here is a map showing the effects of all possible /#:

Page       /1  /2  /3  /4  /5  /6

----       --  --  --  --  --  --

A000       XX  XX  XX  XX  XX  XX

A400           XX  XX  XX  XX  XX

A800               XX  XX  XX  XX

AC00                   XX  XX  XX

B000                       XX  XX <---MDA memory begins

B400                           XX

B800                              <---CGA memory begins

--  --  --  --  --  --

KB added   16  32  48  64  80  96

Total KB  656 672 688 704 720 736

DERAM

-----

EEMRAM works by mapping EEMS memory to DOS addresses beginning

at memory segment A000; EGA/VGA systems also use this area of

memory, so you will have a conflict using EGA/VGA hi-res

graphics modes in an EEMRAM environment.  You may also find that

you have only a few programs that can really use the extra

EEMRAM memory and that you prefer to leave the EMS memory free

most of the time.

The DERAM program reverses the effects of EEMRAM:  it restores

the original 640K of DOS memory and releases the EMS memory

EEMRAM was using back to the free memory pool.  This may allow

you to use EGA/VGA hi-res modes.

To restore the original 640K system, just run DERAM:

C> deram

[NOTE:  to use DERAM with EEMS memory, you must

have run EEMRAM with the /S option:

C> eemram /s

This saves certain information the DERAM needs.

The /S switch is not necessary if you have LIM

4.0-compatible memory.  When you use /S, EEMRAM

places in memory a small amount (four bytes) of

information that DERAM needs.  The information

is stored in the little used Interapplications

Communication Area (IACA).  If EEMRAM cannot

find four consecutive unused bytes in this area,

it cannot store the information and DERAM won't

work.]

As an example of using EEMRAM and DERAM, suppose that you only

want to have the extra memory available for one program, your

word processor BESTWP.  The following command sequence will do

the trick:

C> eemram /s            [/S needed only for EEMS]

C> bestwp

C> deram

You can enter these commands manually or make a small batch file

or PCED synonym.

The opposite situation holds where you want to run with extra

memory most of the time, but release it before running the EGA

graphics program GREATGFX.  The following commands may help:

C> deram

C> greatgfx

C> eemram /s            [/S needed only for EEMS]

IMPORTANT NOTE: it is not possible to guarantee that the system

will operate correctly after DERAM.  Some programs, especially

system software, TSRs, and device drivers, assume that the

amount of memory in your system is fixed--that it will not

change during a session.  However, that is exactly the effect of

DERAM.  If you find that something doesn't run after DERAM,

you'll just have to reboot without EEMRAM.

Memory Allocation Information

-----------------------------

When a typical EEMS or LIM 4.0 memory board is installed, it

"backfills" DOS memory to 640K; that is, some of the memory on

the EEMS board is permanently assigned to DOS in such a way that

the system will appear to DOS to have 640K of main memory.  This

640K occupies RAM addresses 00000 to 9FFFFF, or, in the more

traditional segment notation, memory segments 0000 to 9FFF.

Segments beginning at A000 (the next segment above 9FFF) are not

assigned to DOS by the EMS manager.

However, 64K to 96K of addresses beginning at A000 are often

unused on PC's.  On a machine equipped with a monochrome display

adapter (MDA), display memory begins at segment B000, leaving

64K open from A000-AFFF; on a machine equipped with a color

graphics adapter (CGA), display memory begins at segment B800,

leaving 96K open from A000-B7FF.

It is possible to force the EMS manager to assign memory for

these addresses and to convince DOS that the EMS memory so

assigned is usable by the system, resulting in as much as 736K

of DOS-usable memory.  That is what EEMRAM does.

[Contrary to popular opinion, DOS is not limited to 640K; it can

use whatever memory is available, up to a limit of 1 megabyte.

The limiting factor is your hardware.  In particular, some

hardware such as video display adapters and fixed disk

controllers may use some areas of memory above the 640K mark.

Also, the DIP switches on many machines are limited to a maximum

of 640K switch-settable; to go beyond this, it's necessary for

software such as EEMRAM to override the switch settings.]

If you have an EGA or VGA

-------------------------

As described above, EEMRAM works by assigning pages of EMS

memory to DOS addresses beginning at A000h (just above 640K).

EGA and VGA systems use the same area of memory, so there is a

potential for conflicts.  However, most EGA/VGA systems use the

64K of memory between A000 and B000 only in high resolution

graphics modes, and briefly during mode changes; if you do not

use high resolution graphics modes frequently, you may still be

able to use EEMRAM.

You will have to do some work with your system configuration

file, CONFIG.SYS.  We STRONGLY recommend that you check the

following steps on a bootable diskette, not with your hard disk

CONFIG.SYS.  If you are successful, you can then make the

apprpriate changes in your hard disk CONFIG.SYS.

First, check the statement in CONFIG.SYS file that loads your

EMS driver software.  If you have a parameter that excludes the

range of memory beginning at A000, you must remove the

exclusion.  For example, AST's REMM.SYS might be loaded like

this:

DEVICE=C:\SYS\REMM.SYS /X=A000-AFFF

The /X parameter tells REMM.SYS not to permit mapping of EMS

memory to segment A000, and it must be removed.

Next, add the statement:

DEVICE=[d:path]EEMEGA.SYS

to the file IMMEDIATELY BEFORE the DEVICE statement that loads

your EMS software.  For example:

DEVICE=C:\SYS\EEMEGA.SYS

DEVICE=C:\SYS\REMM.SYS

Now reboot your system.  It should appear to run normally.  Run

EMSINFO again and check to be sure that the A000 segment now

appears in the list of mappable segments.  If it does not, you

will have to check with your dealer or hardware manufacture to

see if it's possible to correct the situation.

If you have EEMS memory, EEMEGA uses 16K of EMS.  It does not

use any EMS memory if you have a LIM 4.0 EMS driver.

When EEMRAM is active, EEMEGA.SYS will not allow any EGA/VGA

BIOS calls to be used.  Thus, your system will not be permitted

to go into high-resolution graphics modes.  If you have software

that must be "installed" for a specific video system, you might

want to install it for CGA or MDA rather than EGA or VGA.

EEMRAM error messages and solutions

-----------------------------------

EEMRAM will not be able to do its job under a number of possible

conditions.  If EEMRAM reports that it has failed, it will

display one of the following messages:

EMS MANAGER IS NEITHER LIM 4.0 NOR EEMS

In order to use EEMRAM, your Expanded Memory Manager (EMM)

must conform to either the EEMS 3.2 or LIM 4.0

specifications.  Check with your dealer to see if a newer

EMM is available for your EMS board.

EXPECTED 640K OF DOS MEMORY

Run MS; in order to use EEMRAM, MS must report exactly

655,360 bytes of total memory (640K).  If MS reports less

and you know that you do have a 640K system, it's likely

that some other TSR or device has taken some memory from

just below 640K.  Try to locate which TSR or device is doing

this and check with your dealer to see if there's any way to

eliminate it.  If MS reports more that 640K, perhaps you've

already run EEMRAM.

UNABLE TO RESIZE MEMORY

EEMRAM must be the owner of the last block of DOS memory

when it's run.  Always run EEMRAM directly from the DOS

prompt or from AUTOEXEC.BAT.

UNABLE TO LOCATE EMS MANAGER

EEMRAM wasn't able to find the Expanded Memory Manager

(EMM).  Either you don't have expanded memory, you haven't

loaded the EMM, or another program is interfering.

NO PAGES AVAILABLE AT A000

In order for EEMRAM to work, it must be able to assigne

chunks of EMS memory to DOS addresses beginning at

segment A000.  If EEMRAM reports that it can't do this, it's

usually for one of these reasons:

1. Your EMS hardware is not capabable of mapping EMS memory

to arbitrary memory addresses.  This is true of many older

EMS boards such as the Intel AboveBoard.  There's no

solution for this other than to purchase new hardware.

2. Your Expanded Memory Manager (EMM) doesn't support this

function.  Your EMM must conform to either the EEMS 3.2

or the LIM 4.0 specification.  Run EMSINFO; it will tell you

which type of EMM you have.  Check with your dealer to see

if you can obtain an EEMS or LIM 4.0 EMM for your hardware.

3. Your EMM has detected that you have an EGA or VGA and has

in effect "prohibited" mapping memory to A000, even though

both your EMS board and the EMM are capable of such mapping.

See the EGA/VGA section for a possible solution.

NO EMS MEMORY AVAILABLE

Your EMM has reported that there is no free EMS memory.

You'll have to reduce the amount of memory used by other

software.  EEMRAM must have at least 16K free EMS memory and

can use up to 96K on some systems.

PROGRAM HAS ALREADY BEEN RUN

This message will appear only under LIM 4.0 and indicates

that EEMRAM has already been run since the last reboot.

ERROR xx ACCESSING EMS MANAGER (AX=yyyy)

Generic message indicating that there's a problem with the

EMM or the hardware:  the EMM is corrupted, there's a

hardware failure, your EMM doesn't support a needed

function, etc.  The number (xx) indicates the specific error

code returned that the EMM returned to EEMRAM.  The message

also indicates the EMS function that failed (register AX

contents were yyyy at time of EMM function call).

WARNING: UNABLE TO SAVE /S INFORMATION FOR DERAM

There is insufficient memory unused in the area that

EEMRAM/DERAM use to pass needed information back and forth.

This is a warning message: EEMRAM itself has run successfully,

but DERAM won't work.

DERAM error messages

--------------------

SYSTEM ALREADY SET TO 640K

DERAM has found that the system already shows 640K of DOS

memory, so it hasn't done anything.

EEMRAM NOT RUN WITH /S OPTION, UNABLE TO RESIZE MEMORY

In order to use DERAM with EEMS memory, you must run EEMRAM

with the /S switch.  If you did run EEMRAM with /S and you

see this message, some other program is using the same area

of memory that EEMRAM/DERAM use to pass information back and

forth.  You will not be able to use DERAM in this case

(fortunately, it's unlikely).

ERROR xx ACCESSING EMS MANAGER (AX=yyyy)

Generic message indicating that there's a problem with the

EMM or the hardware:  the EMM is corrupted, there's a

hardware failure, your EMM doesn't support a needed

function, etc.  The number (xx) indicates the specific error

code returned that the EMM returned to EEMRAM.  The message

also indicates the EMS function that failed (register AX

contents were yyyy at time of EMM function call).

UNABLE TO RESIZE MEMORY

DERAM can't modify DOS's memory allocation blocks, so

nothing has been done.  Always run DERAM directly at the

DOS prompt or via AUTOEXEC; don't run it as a "child" of

some other program.

Notes

-----

1. We can't promise that EEMRAM will work with any specific

hardware configuration.  If your hardware conflicts with what

EEMRAM does (EEMRAM won't run properly, system locks up, etc.),

there's nothing we can do about it.

2. EEMRAM is not a TSR (i.e., it is not resident).  It runs and

terminates, restoring the full amount of memory to the system.

Because EEMRAM is not a TSR, don't try to use MARK/RELEASE,

REFEREE, POPDROP, or similar TSR managers to "remove" EEMRAM.

EEMRAM cannot be removed by these managers.  EEMEGA.SYS is a

device driver and also cannot be removed; however, it uses very

little memory.

3. EEMRAM and DERAM both return an errorlevel of 255 if they

encounter any errors.

4. If EEMRAM needs more EMS memory than is available, it will

use all that is available.  If no EMS memory at all is

available, it is an error.

Versions

--------

2.00 03/30/89

Added support for LIM 4.0 memory.

Added EEMEGA.SYS

1.01 08/20/87

Changed test for enhanced memory, may be a little more

tolerant of older versions of REMM.SYS et al.

Copyright/License/Warranty

--------------------------

This document and the program files EEMRAM.COM, DERAM.COM

EEMEGA.SYS, MS.COM, and EMSINFO.EXE ("the software") are

copyrighted by the author.  The copyright owner hereby licenses

you to: use the software; make as many copies of the program

and documentation as you wish; give such copies to anyone; and

distribute the software and documentation via electronic means.

There is no charge for any of the above.

You are specifically prohibited from charging, or requesting

donations, for any such copies, however made; and from

distributing the software and/or documentation with commercial

products without prior permission.  An exception is granted to

recognized user's groups, which are authorized to charge a

small fee (not to exceed $7) for materials, handling, postage,

and general overhead.  NO FOR-PROFIT ORGANIZATION IS AUTHORIZED

TO CHARGE ANY AMOUNT FOR DISTRIBUTION OF COPIES OF THE SOFTWARE

OR DOCUMENTATION, OR TO INCLUDE COPIES OF THE SOFTWARE OR

DOCUMENTATION WITH SALES OF THEIR OWN PRODUCTS.

THIS INCLUDES A SPECIFIC PROHIBITION AGAINST FOR-PROFIT

ORGANIZATIONS DISTRIBUTING THE SOFTWARE, EITHER ALONE OR WITH

OTHER SOFTWARE, AND CHARGING A "HANDLING" OR "MATERIALS" FEE OR

ANY OTHER SUCH FEE FOR THE DISTRIBUTION.  NO FOR-PROFIT

ORGANIZATION IS AUTHORIZED TO INCLUDE THE SOFTWARE ON ANY MEDIA

FOR WHICH MONEY IS CHARGED WITHOUT PRIOR PERMISSION FROM THE

COPTRIGHT OWNER.

No copy of the software may be distributed or given away without

this document; and this notice must not be removed.

There is no warranty of any kind, and the copyright owner is not

liable for damages of any kind.  By using this free software,

you agree to this.

The software and documentation are:

Copyright (C) 1987,1989 by

Christopher J. Dunford

The Cove Software Group

Post Office Box 1072

Columbia, Maryland 21044

(301) 992-9371

CompuServe 76703,2002 [IBMNET]