132 COLUMN SUPER VGA EXTENDED TEXT MODES, IN HEXADECIMAL NOTATION

132  132  132  132  132  132  132  132  132  Cols

Manufacturer              25   28   29   30   32   43   44   50   60  Rows

--------------           ---  ---  ---  ---  ---  ---  ---  ---  ---

Acumos AGVA2/3            55   -    -    -    -    54   -    -    -

Ahead V5000 A/B           23   24   -    -    -    -    22   -    -

Allstar Logic VGA         -    -    -    -    -    22   -    -    -

Allstar Peacock VGA       24   23   -    -    -    -    -    -    -

AST VGA Plus              55   -    -    -    -    54   -    -    -

ATI VIP, ATI x8800        23   -    -    -    -    -    33   -    -

ATI Prism Elite          (see Trident)

AT&T VDC600               55   -    -    -    -    54   -    -    -

Avance Logic 2101         20   -    -    21   -    22   -    -    23

Bocca Super X VGA        (see Tseng)

Bocca (older boards)     (see Chips & Tech.)

Cardinal                 (see Chips & Tech.)

Chips & Tech. chipset     60   -    -    -    -    -    -    61   -

Cirrus Logic 5xx/6xx      1F   -    -    22   -    -    20   -    -

Cirrus Logic 5320 chips   1C   24   -    -    -    1D   1E   61   -

Cirrus Logic 542x         14   -    -    -    -    54   -    -    -

Diamond Speedstar VGA    (see Tseng)

Diamond Speedstar 24X     55   47   -    -    -    54   -    69   -

Diamond Stealth 24X      (see S3)

Everex 236/620/673/678   (see Trident)

Everex 629 Viewpoint-TC   -    -    -    40   50   -    -    -    -   *

Genoa 6xxx                60   -    61   -    62   -    63   -    64

Genoa 5xxx/7xxx          (see Tseng)

Groundhog Graph. Shdw.   (see Tseng)

Headland Technologies    (see Video 7)

HP D1180A                 55   -    -    -    -    54   -    -    -

Morse VGA                 -    -    -    50   -    -    -    51   52

MX 68010                  50   -    -    51   -    -    -    -    -

Number 9 GXiTC           (see Acumos)

NCR 77C2x                 55   -    -    -    -    -    -    54   -

Oak Tech. VGA-16          50   -    -    -    -    51   -    -    -

Oak 067/077/037C          50   -    -    -    -    51   -    -    4F

Oak O37C + NEL Bios       57   -    -    -    -    56   -    -    -

Orchid Prodesigner VGA    24   23   -    -    -    -    22   -    -

Orchid Prodesigner II    (see Tseng)

Paradise PVGA1x/WD90Cxx  (see Western Digital)

Quadram Ultra VGA         60   -    61   -    62   -    63   -    -

Realtek RT 3106           1B   -    -    1C   -    1D   -    -    1E

S3 chipset (some)         55   -    -    -    -    54   -    -    -   ***

STB VGA 1024             (see Tseng)

Tatung VGA                41   45   -    -    -    42   -    -    -

Tecmar VGA/AD             17   -    -    -    -    -    -    -    -

Trident 8xxx/9xxx         57   -    -    58   -    59   -    -    5A

Tseng EVA & ET3000        23   24   -    -    -    -    22   -    -

Tseng ET4000              23   24   -    -    -    -    22   -    21

UM587 chipset             50   -    -    -    -    51   -    -    -

Vega VGA                  41   -    -    -    -    42   -    -    -   *

VESA SuperVGA BIOS       109   -    -    -    -   10A   -   10B  10C  **

Video 7 VRAM & 1024i      41   45   -    -    -    42   -    -    -   *

Voltane                  (see Acumos)

Western Digital WD90Cxx   55   47   -    -    -    -    54   69   -

Willow VGA               (see Tseng)

SUPER VGA EXTENDED TEXT MODE NOTES:

The "x" in card and chip models indicates a variable digit, applicable to

two or more model numbers which fit the pattern.

Only 132 column color modes are listed.  Where more than one font size is

offered for a given resolution, the modes for larger fonts are shown.

If your video card is not listed, its chipset manufacturer or BIOS may be.

Microsoft's MSD.EXE program (included with MS-DOS 6.x and Windows 3.1) is

capable of identifying many common video adapters.  Beware of the fact

that manufacturers implementing third-party chipsets may or may not choose

to implement the chipset's standard BIOS.  If neither card nor chipset is

listed, you still may be able to use one or more of the VESA SuperVGA

modes (see table above, and note below).

Except as noted below, the modes listed above are for use with the

standard BIOS "set video mode" call: Interrupt 10h, AH = 00h, AL = mode.

*  A few video cards use non-standard BIOS calls to implement extended modes,

including Everex, Vega and Video 7.  ALLY automatically detects Everex,

Vega, and Video 7 and handles them appropriately.

**  The VESA SuperVGA BIOS standard is implemented in most modern SVGA cards,

either directly in firmware or by running a VESA TSR program supplied by

the video card manufacturer.  If you are unable to get ALLY working with

your video card's extended modes, try using a VESA mode.  ALLY recognizes

any mode greater than or equal to 100 hexadecimal as a VESA mode.  Not

every VESA mode listed above is available in every case, depending on the

video hardware and VESA implementation being used.

***  A number of OEMs manufacture boards with the S3 chipset, but don't count

on their BIOS implementations being the same.  Check your specific video

card manual for extended mode numbers or use VESA modes.  Note that

current S3 drivers for OS/2 do not support 132 column modes.

Some AutoCAD display list processor and video card combinations do not

tolerate video mode changes when "shelled" out from AutoCAD - video mode

changes should be avoided in such situations.  If you load a VESA TSR

program to implement extended text modes, this must be done before AutoCAD

is loaded.

Although we attempt to provide accurate data, Wasco Technical Software

provides this document "as is" for information only, and shall not be held

responsible for errors or omissions.  Users should consult their specific

video card manual for manufacturer's information.  If you have data you

would like to contribute to this listing, please write or fax.

Wasco Technical Software

1211 Grand Junction Drive

Katy, Texas  USA  77450

Tel: 713-392-1359

Fax: 713-392-1073

Internet: 72020.2031@compuserve.com

Compuserve: 72020,2031                                         Rev.3  22-Mar-94