--------------------------------------------------------------------------

---                      The QuickBBS ST Ansi Emulator                 ---

---                            (and related topics)                    ---

--------------------------------------------------------------------------

[1]  What is Ansi?

ANSI  is short for American National Standards  Institute.  The  ANSI

organisation develops and documents standards, especially in the area

data communications.  One of those standars is Ansi X3.64, which is a

terminal emulation protocol.  It helps a host computer,  or a program

running locally,  use the intelligent console,  with features such as

cursor movement, advanced text attributes etc.

ANSI  is also the name of the screen and keyboard driver used on  MS-

Dos  machines (ANSI.SYS).  The choice of name is not rather  idiotic,

because ANSI.SYS is neither a complete implementation of  X3.64,  nor

a subset of X3.64.  Anyway,  when I refer to ANSI in this document, I

mean  the  MicroSoft version.  Some communication programs  (such  as

ProComm and Telix) call it Ansi-BBS.

[2]  What has Ansi got to do with QuickBBS ST?

A hell of a lot of PC communications packages have an Ansi mode. It's

fairly easy for 'them',  because they can just re-route the  incoming

characters straight to the console driver. To make it possible for PC

users  with  such  packages  to have 'fancy'  screens  and  menus  in

QuickBBS ST,  a graphics mode called 'Ansi-BBS' is available.  Due to

the fact that I have to do my own screen driving, it is *much* slower

than the VT-52 modes when used locally, but on-line it shouldn't make

too much difference. When operating in Ansi mode QuickBBS will use an

MS-Dos style font, so you can create box graphic text files.

[3]  Ansi compared to VT-52.

Both  Ansi and VT-52 use things called Escape  Sequences.  An  Escape

Sequence is a string of characters which starts with an ESC character

(27),  followed  by  one or more parameters.  An example of  an  Ansi

escape sequence is

<ESC> [ 10; 20 H

Where <ESC> is the an escape, and the spaces are only for clarity (as

in the rest of this document).  This particular escape sequence makes

the cursor move to line 10,  column 20,  and is defined by the 'H' at

the end.

The VT-52 version of this escape sequence is

<ESC> Y * 4

The  '<ESC>  Y' indicates that the cursor has to  be  moved,  and  is

followed  by  two characters whose ASCII codes are Y and  X  plus  32

(10+32 = 42 = "*", 20+32 = 52 = "4").

Note that the VT-52 sequence is much more efficient for two reasons:

1)   The code is 4 bytes instead of 8.

2)   The command is followed by the parameters, instead of parameters

followed by command.  For practical reasons this is much  easier

to implement.

So, if possible, use one of the VT-52 modes of QuickBBS ST instead of

Ansi.

[4]  Limitations and differences.

Due to the structural differences between the screen of a PC and that

of an ST, the Ansi driver in QuickBBS ST has some 'differences'...

1)   A PC with CGA monitor can display 8 colours for foreground  and

8  colours  for background,  whereas the ST can only  display  4

each  on a colour monitor.  See the 'change  attributes'  escape

sequence to see how Quick handles colours.

2)   A PC can underline,  highlight,  blink and reverse text.  An  ST

using  BIOS/GEMDOS  for output can only  use  reverse.  See  the

'change attributes' escape sequence.

3)   I  have not found any way of changing the scroll  region  X,  Y,

and Width for TOS output.

4)   All  escape  sequences  that QuickBBS ST doesn't  know  will  be

ignored. They WILL however be sent to the user on-line.

The implementation in QuickBBS ST also has some escape sequences  not

found in ANSI.SYS, but which are part of X3.64, or frequently used by

Unix machines.

[5]  Recognised Escape Sequences.

QuickBBS   ST   recognises  and  reacts  to  the   following   escape

sequences.  All  characters  in <brackets> should  be  replaced,  and

spaces should be removed...

Name      : CUU - Cursor Up

Escape    : <ESC> [ <n> A

Example   : <ESC> [  5  A

This  sequence  moves the cursor up <n> lines.  If parameter  <n>  is

ommited,  the cursor moves up one line.  The cursor will not move off

the screen,  nor will the screen scroll down.  The cursor remains  on

the same column.

Name      : CUD - Cursor Down

Escape    : <ESC> [ <n> B

Example   : <ESC> [  3  B

CUD  is  the same as CUU (cursor up),  but the cursor is  moved  down

instead.

Name      : CUF - Cursor Forward (right)

Escape    : <ESC> [ <n> C

Example   : <ESC> [  8  B

This moves the cursor <n> columns to the right. If the cursor reaches

column 80,  it will stay there.  The cursor remains on the same line.

If <n> is ommited, the cursor moves one column.

Name      : CUB - Cursor Backward (left)

Escape    : <ESC> [ <n> D

Example   : <ESC> [  7  D

Is identical to CUF (cursor forward),  exept that the cursor is moved

to the left, as far as the left hand side of the screen.

Name      : CUP - Cursor Position

Escape    : <ESC> [ <y> ; <x> H

Example   : <ESC> [ 10  ; 20  H

This escape moves the cursor to location (x,y),  where X is between 1

and  80,  and Y between 1 and 24.  If <y> and <x>  are  ommited,  the

cursor is moved to the top left-hand corner (1,1). The sequences

<ESC> [ <y> ; <x> H

<ESC> [ <y> ; <x> f

<ESC> [ <y> ; <x> j

are identical, but the first is recommended.

Name      : SCP - Save Cursor Position

Escape    : <ESC> [ s

SCP  tells the Ansi driver to note down the current  cursor  position

for future use.  If an RCP is received the cursor will be returned to

this position. The SCP and RCP escapes are not nestable.

Name      : RCP - Restore Cursor Position

Escape    : <ESC> [ u

RCP places the cursor at the location noted down by SCP.

Name      : ED - Erase Display

Escape    : <ESC> [ <n> J

Example   : <ESC> [  2  J

ED  erases all or part of the screen depending on the parameter  <n>.

If this parameter is ommited, it is assumed to be '0'...

<n> = 0 : The  screen  is  erased  between  the  current  cursor

location  and  the lower right corner of  the  screen,

inclusive. The cursor doesn't move.

<n> = 1 : The screen is erased between the upper left corner  of

the screen and the current cursor location, inclusive.

The cursor doesn't move.

<n> = 2 : The whole screen is erased and the cursor is moved  to

the upper left corner.

The screen is cleared using the current background colour.

Name      : EL - Erase Line

Escape    : <ESC> [ <n> K

Example   : <ESC> [  0  K

All  or part of the current line is erased,  depending  on  parameter

<n>.  The cursor is never moved by this escape. If <n> is ommited, it

is assumed to be '0'...

<n> = 0 : All characters to the right of the cursor are erased.

<n> = 1 : All characters to the left  of the cursor are erased.

<n> = 2 : All characters on the current line are erased.

The line is cleared using the current background colour.

Name      : IL - Insert Lines

Escape    : <ESC> [ <n> L

Example   : <ESC> [  3  L

IL inserts <n> lines at the current cursor location. The current line

and  all lines below it are moved down.  Lines at the bottom  of  the

screen will be lost.  If <n> is omitted,  one line is  inserted.  The

cursor  doesn't move.  The new line will be displayed in the  current

background colour.

Name      : DL - Delete Lines

Escape    : <ESC> [ <n> M

Example   : <ESC> [  9  M

DL  deletes <n> lines,  starting with the current  cursor  line.  All

lines  underneath  the line(s) to be deleted will be  moved  up.  New

lines, in the current background colour, will appear at the bottom of

the screen. The cursor doesn't move.

Name      : DECSC - DEC Private, Save Cursor Parameters

Escape    : <ESC> 7

This  is supposed to be a DEC Private escape,  but is used  on  other

systems as well. It saves the current cursor position (like SCP), but

also stores the current text attributes and colours. With DECRC these

parameters can be restored. THIS IS NOT IMPLEMENTED IN ANSI.SYS!!!

Name      : DECRC - DEC Private, Restore Cursor Parameters

Escape    : <ESC> 8

DECRC  is  the  opposite of DECSC,  and  restores  the  saved  cursor

parameters. THIS IS NOT IMPLEMENTED IN ANSI.SYS!!!

Name      : RIS - Reset to Initial State

Escape    : <ESC> c

This 'resets' the Ansi driver: the colours are set to default values,

all text attributes are switched off and the screen is cleared. It is

the same as doing an SGR 0 followed by ED 2.  THIS IS NOT IMPLEMENTED

IN ANSI.SYS!!!

Name      : NEL - Next Line

Escape    : <ESC> E

This  moves  the  cursor  to the first position  on  the  next  line,

scrolling the screen up if neccesary.  It is effectively the same  as

Carriage  Return  followed by Line Feed,  and is NOT  IMPLEMENTED  IN

ANSI.SYS.

Name      : SGR - Set Graphics Rendition

Escape    : <ESC> [ <n> ; <n> ; ... ; <n> m

Example   : <ESC> [  1  ; 37  m

Now this is the good stuff...  With this escape you can control three

things:

- The text foreground colour,

- The text background colour,

- The text attributes.

There can be any number of parameters,  separated by ';'  characters.

The  effect  is  cumulative.  If no parameters  are  passed,  '0'  is

assumed. The following parameters are available:

0         - Reset colours and attributes

1         - Bold (*)

4         - Underlined (*)

5         - Blink (*)

7         - Reverse Video

30        - Foreground Black

31        - Foreground Red

32        - Foreground Green

33        - Foreground Yellow

34        - Foreground Blue

35        - Foreground Magenta

36        - Foreground Cyan

37        - Foreground White

40 to 47  - Background Colours

Attributes  marked  by (*) are ignored by Quick.  When running  on  a

monochrome monitor the colours are also ignored.  On a colour monitor

colour 30 is black (register 0), colour 37 is white (register 3), and

all  colours  inbetween  are alternated Blue  (register  1)  and  Red

(register 2). Of course this depends on your screen settings, but the

above  are the QuickBBS defaults.  If foreground and background  turn

out to be the same, Quick will adjust the background colour.

Some examples...

Bold, White on Blue     : <ESC>[0;1;37;44m

Blink, Red on Yellow    : <ESC>[0;5;31;43m

Normal, White on Black  : <ESC>[m or <ESC>[0m

Bold, Underlined, Green : <ESC>[0;1;4;32m

Note  that  setting  - say - Bold does not effect any  of  the  other

attributes,  nor the colours. To make sure you've got the right mode,

you  could  add  a '0' at the beginning,  but  that  depends  on  how

intelligent your Ansi editor is, which brings us to...

[6]  How to create your Ansi screens.

Well, there are basically three options. You can do it all by hand in

a  text  editor  like Tempus,  you can buy a PC and  use  TheDraw  or

AnsiPaint,  or  you  can use a little VT-52 to  Ansi  converter  I've

written called...  QANSI. The convertor can only handle a screen at a

time,  but  then most VT-52 editors can only create one screen  at  a

time.  Operation is simple,  you just specify input and output files,

and QANSI does the rest.

[7]  Box Graphics

The MS-Dos character set has a number of BOX GRAPHICS which are  very

nice for making logo screens,  boxes around texts and such. Here is a

list of some often-used characters.

� = 174     � = 175     � = 176     � = 177     � = 178     � = 179

� = 180     � = 181     � = 182     � = 183     � = 184     � = 185

� = 186     � = 187     � = 188     � = 189     � = 190     � = 191

� = 192     � = 193     � = 194     � = 195     � = 196     � = 197

� = 198     � = 199     � = 200     � = 201     � = 202     � = 203

� = 204     � = 205     � = 206     � = 207     � = 208     � = 209

� = 210     � = 211     � = 212     � = 213     � = 214     � = 215

� = 216     � = 217     � = 218     � = 219

� = 220     � = 221     � = 222     � = 223

To print this out on a printer without MS-Dos character set:  display

the file in QuickBBS with Ansi Graphics,  and just do a screen  dump.

Perhaps someone has time to write a proper VT-52 and ANSI editor???